3GPP TSG-RAN WG2 Meeting #107                                                 	R2-1908760

Prague, Czech Republic, 26th - 30th Aug 2019

Title:	[Draft] LS on MDT Configuration of Event triggered Out-of-coverage

Response to:

Release:	Rel-16

Work Item:	          NR_SON_MDT-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

Name:	Fan Jiangsheng

E-mail Address:	fanjiangsheng (at) catt (dot) cn

- Overall Description:

RAN2 has agreed that UE can be configured to only perform the logging for out-of-coverage detection, i.e., UE is not required to log the DL pilot strength measurements if the configuration is only out-of-coverage detection. Therefore an event triggered periodic measurement for Out-of-coverage should be configured independently from the periodic logged MDT.

For signaling based MDT, it is the responsibility of AMF to transfer the MDT configuration to the gNB via NG-AP, therefore the indicator of logged MDT type (periodic measurement for DL pilot strength or event triggered periodic measurement for Out-of-coverage) should be included in the MDT configuration. All other parameters configured in NR logged MDT are similar as in LTE.

- 2. Actions:

To RAN3:

ACTION: RAN2 respectfully asks RAN3 to take the above RAN2 recommend into account.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #107bis	14th – 18th October 2019	Chongqing, China

TSG-RAN2 Meeting #108	    18th – 22th November 2019	      Reno, USA