3GPP TSG-RAN WG2 Meeting #107                                                 	R2-1908763

Prague, Czech Republic, 26th - 30th Aug 2019

Source:	CATT

Title:	Clarification of UL Over-the-air Delay Measurement

Agenda Item:	11.12.3

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, we discussed whether to define uplink and downlink user plane latency measurements or to define uplink and roundtrip user plane latency measurements [1] asked by SA5 [2], achieved the agreements and sent a reply LS to SA5 [3].

Agreements

1	Introduce separate measurement associated to over-the-air delay in UL.

2	Average delay UL air-interface is defined as the average time between the time of sending the successful HARQ feedback to the time of scheduling grant in UL for the UE.

The agreements above are captured by TR37.816 and the SI is already finished. But the definition of UL HARQ feedback is not correct. In this contribution, we intend to clarify the solution of how to record the UL over-the-air delay measurement.

Discussion

In early version of LTE, only synchronous HARQ is supported in UL, and the re-transmission could be adaptive or non-adaptive. And the asynchronous UL HARQ is introduced from Rel-13.

HARQ feedback for UL data transmission is carried on the PHICH channel in LTE. After receiving the UL data send from UE, the network will feedback whether the data is successfully received. It is more useful for synchronous and non-adaptive HARQ: If UE receives NACK, it will re-transmit the UL data. After the asynchronous HARQ is introduced for UL data transmission in LTE, the HARQ feedback in PHICH is useless, since the network could carry the New Data Indicator(NDI) in DCI, to identity whether a new UL data should UE transmit.

In NR, there are only three DL physical channels [4]:

7.1.1 Overview of physical channels A downlink physical channel corresponds to a set of resource elements carrying information originating from higher layers. The following downlink physical channels are defined: - Physical Downlink Shared Channel, PDSCH - Physical Broadcast Channel, PBCH - Physical Downlink Control Channel, PDCCH

PHICH channel is not used in NR, and there is no HARQ feedback for UL data. Only the NDI inherit from LTE is used to indicate the UE whether to send the re-transmission data or a new data.

Observation: There is no HARQ feedback for UL data in NR.

The UL delay in air-interface could be calculated as the figure below:

Figure1 Calculation of UL delay in air-interface

T1: The time of the first scheduled uplink transmission of this HARQ process, it is also the scheduled grant time decided by the network.

- The network is aware of the T1 as the network is the one that provides the scheduling grant [1]. After receiving the scheduling grant, UE will send the UL data according to the grant. So T1 could be described as the first scheduled uplink transmission of this HARQ process, but not the time of scheduling grant which seems indicating the time of T0 in Figure1.

T2: The time of  the data of the corresponding HARQ process is successfully decoded.

- The network checks whether the UL data is successfully decoded. If so, the timer of T2 could be determined, and network could send DCI with toggled NDI if there is still data waiting for transmission in UL; if not, the network should send DCI with non-toggled NDI to expect a re-transmission of the data.

So the UL delay in air-interface equals to T2-T1. But since there is no HARQ feedback for UL data transmission, and the description of the start time of the UL delay measurement is unclear, the description in TR37.816 should be modified.

Proposal: Redefine the over-the-air delay in UL to the average time between the data of the corresponding HARQ process is successfully decoded to the time of the first scheduled uplink transmission of this HARQ process for the UE.

Conclusion

According to the analyses in section 2, we have:

Observation: There is no HARQ feedback for UL data in NR.

Proposal: Redefine the over-the-air delay in UL to the average time between the data of the corresponding HARQ process is successfully decoded to the time of the first scheduled uplink transmission of this HARQ process for the UE.

Reference

- R2-1907184, URLLC delay measurements in UL and DL Ericsson

- R2-1801755/S5-181576 LS on measurement of user plane latency SA5

- R2-1908144 Reply LS on the user plane latency measurement RAN2

- TS38.212 v15.4.0

TP for TR37.816

This change is based on TR37.816 v1.0.0.

6.2.2.2.1 			UL packet delay measurement

Figure 6.2.2.2.1-1: RAN part of UL delay

As shown in figure 6.2.2.2.1-1, RAN part (T2-T1) of the UL delay is defined as the delay from packet entering the UE’s PDCP upper SAP to leaving gNB’s PDCP upper SAP. It can be separated into D1 and D2:

- D1 is the PDCP queuing delay in the UE, including the delay from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to getting first grant). D1 is invisible to the network and should be measured by the UE.

- D2 is the rest of the delay, including HARQ (re)transmission delay, RLC delay, F1 delay* and PDCP re-ordering delay in gNB.  Separate measurement associated to over-the-air delay in UL will be introduced, and it is defined as the average time between the data of the corresponding HARQ process is successfully decoded to the time of the first scheduled uplink transmission of this HARQ process for the UE.

Note: the measurements “average delay DL in CU-UP” and “average delay on F1-U” in the Table 6.2.2-1 could be used.

The RAN part of UL delay is measured by the following mechanism:

- UE measures D1 and reports the average of D1 to gNB in RRC;

- gNB measures the D2 and derives UL delay as D1+D2.