3GPP TSG-RAN WG2 Meeting #107		R2-1908891

Prague, Czech Republic, 26th – 30th August 2019

Source:	CATT

Title:	Per BWP configuration of maxMIMOLayer

Agenda Item:	11.11.4

Document for:	Discussion and Decision

# Introduction

In RAN#83 meeting, the WI on power saving was updated [1]. One of the objectives is to

- Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation

According to the WID, it shows that different maximum number of MIMO layers for initial/default BWP has been agreed. In this document, we further discuss whether to support per-BWP maximum MIMO layer for BWP other than initial/default BWP and the ASN.1 design for per-BWP maximum MIMO layer.

# Discussion

In Rel-15, maximum MIMO layer for downlink can be configured at cell level [2]. In NR, different BWPs are used to adapt, for instance, to different traffic requirements, various power saving purposes, etc. But since the maximum MIMO layer is configured based on cell level [2], the UE may have to support a maximum MIMO layer for a BWP even if it does not have the necessity. This is unfair, especially when the UE is configured for the purpose of low power consumption.

For example, there are two BWPs, BWP1 and BWP2, configured for one serving cell except initial/default BWP. BWP1 is configured to be available for large data amount transmission, e.g. video, while, BWP2 is used for small and intersperse packet traffic. In order to satisfy the high throughput on BWP1, maxMIMO-Layers of the serving cell is set to the maximum value of the layer number that can be configured considering the UE capability and etc.

However, due to the fluctuant traffic pattern, the UE may be switched between BWP1 and BWP2. Consequently, there is possibility that the UE is scheduled with large MIMO layer number on BWP2, which eventually increases power consumption according to the simulation results from RAN1 which show that reducing the number of RX antennas helps to decrease the power saving consumption for PDCCH monitoring [3]-[5]. Therefore, enhancements are needed to improve the configuration flexibility of maximum MIMO layer for DL.

When parameter maxMIMO-Layers is defined per-BWP, the UE adaptation to the maximum MIMO layers could also be incorporated with the dynamic adaptation of BWP switch. This will help to achieve power saving.

Then, it can be seen that:

Observation 1: It facilitates power saving when maximum MIMO layers is configured per BWP for DL.

For uplink, currently, the maxRank which has the same meaning of maxMIMO-Layers helps to reduce the complexity of UE understanding the DCI. However, unlike for downlink, introducing per BWP based maximum MIMO layer for UL is not justified and may introduce a lot of issues from specification and implementation perspective. Indeed, the UL rank and precoder are dynamically controlled by gNB, based on UL measurement with SRS. And the number of SRS ports is RRC configured, which is not equivalent to the number of physical antennas. UE can virtualize SRS to physical antennas – this is up to UE implementation. In other words, power saving optimization by adapting the number of antennas in UL MIMO is a UE implementation task. Therefore, it is proposed that:

Proposal 1: No further improvement is justified for the UL maximum MIMO layer.

It may be argued that per-BWP configuration may have great impacts on the IE structure and brings complexity to the signaling design. In the following, we further analyze the signaling impacts to check whether high complexity is introduced.

In current specification, IE maxMIMO-Layers is configured under the IE PDSCH-ServingCellConfig, which is cell level based. In order to achieve backward compatibility, cell level IE maxMIMO-Layers is not changed. For simplicity, one new parameter, i.e. maximum MIMO layer for BWP level, which also can be named maxMIMO-Layers can be configured under the BWP level, i.e. under the IE PDSCH-Config. Meanwhile, it should be noted that maxMIMO-Layers of cell level will be ignored, if it is also configured under BWP. Possible signaling structure can be:

Figure 1 Possible signaling structure of maxMIMO-Layers

One example of ASN.1 format is shown in Annex A. It can be seen minor change is introduced here.

Hence, it is proposed that:

Proposal 2: Define per-BWP based maxMIMO-Layer for DL.

Proposal 3: RAN2 to agree the signaling structure in Figure 1.

# Conclusion

This contribution discusses the issues related maximum MIMO layer. And we propose that:

Proposal 1: No further improvement is justified for the UL.

Proposal 2: Define per-BWP based maxMIMO-Layer for DL.

Proposal 3: RAN2 to agree the signaling structure in Figure 1.

# Reference

- RAN#84 meeting, RP-191607, New WID: UE Power Saving in NR, Newport Beach, USA, 3rd  – 6th   June 2019;

- 38.331, Radio Resource Control (RRC) protocol specification;

- R1-1900040, Power saving schemes, Huawei, HiSilicon, 3GPP TSG RAN WG1 Ad-Hoc Meeting 1901, Taipei, 21st – 25th January, 2019

- R1-1901803, Evaluation Results of NR Power Saving Designs, MediaTek Inc. 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, 25th Feb. - 1st Mar., 2019;

R1-1902507, System level evaluation of UE power saving schemes, Intel Corporation, 3GPP TSG RAN WG1 Meeting #96, Athens, Greece, 25th Feb. - 1st Mar., 2019

# Annex

#### PDSCH-Config

The PDSCH-Config IE is used to configure the UE specific PDSCH parameters.

PDSCH-Config information element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                          ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...

[[

maxMIMO-Layers                          INTEGER (1..8)                                                      OPTIONAL,   -- Need M

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Config field descriptions

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2)

dataScramblingIdentityPDSCH Identifier used to initialize data scrambling (c_init) for PDSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 7.3.1.1).

dmrs-DownlinkForPDSCH-MappingTypeA DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

dmrs-DownlinkForPDSCH-MappingTypeB DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PDSCH of the BWP (see TS 38.212 [17], clause 5.4.2.1). The IE maxMIMO-Layers configured per serving cell should be ignored when IE maxMIMO-Layers is configured for the BWP.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH. (see TS 38.214 [19], clause 5.1.3.1). If the field is absent the UE applies the value 64QAM.

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent the UE applies the value 1.

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data (see table 5.1.2.1.1-1 in TS 38.214 [19]).

prb-BundlingType Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2.

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set.

rateMatchPatternGroup1 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the nested bitmaps (see TS 38.214 [19], clause 5.1.4.1). FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary.

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5).

vrb-ToPRB-Interleaver Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).