3GPP TSG-RAN WG2 Meeting #107                                                            	R2-1908903

Prague, Czech Republic, 26th - 30th Aug 2019                      Resubmission of	 R2-1905883

Source:	CATT

Title:	Common Cell Configuration for Signaling Reduction in NR

Agenda Item:	11.10.4.4

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, some agreements were made regarding a number of aspects of euCA based enhancements.  But the common cell configuration which is one of the enhancements for euCA in LTE has not been discussed for CA/DC enhancements of NR.

In this contribution, we discuss the options of whether and how to perform the common cell configuration in NR.

Discussion

  - Whether to perform common cell configuration

The common SCell configuration has been introduced in euCA for purpose of reducing signalling overhead [1]:

Common Scell configuration:

- UE may be provided with up to four Scell configuration groups that share common parameters. SCells can be added or removed from the groups according to normal Scell parameter modification rules

- The group-specific common SCell parameters can only be provided when the Scell group is created or in handover (i.e. similar rules as with SCell addition).

From the singalling perspective, it can be seen that in each SCell group, a group identity and a common configuration are needed. The different parameters of each SCell can be added or modified as required. If network wants to remove a SCell or change it to another group, a field of release the SCell could be used.

SCellGroupToAddMod-r15 ::=			SEQUENCE {

sCellGroupIndex-r15					SCellGroupIndex-r15,

sCellConfigCommon-r15				SCellConfigCommon-r15			OPTIONAL,	-- Need ON

sCellToReleaseList-r15				SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellToAddModList-r15				SCellToAddModListExt-r13		OPTIONAL	-- Need ON

}

The method used in LTE euCA is flexible to perform add/modify/release of the SCell from a SCell group, and can be reused as NR baseline.

Proposal 1: LTE euCA common SCell configuration could be reused in NR as baseline.

  - How to perform common cell configuration

All the solution of euCA is introduced in LTE release15 and before that the SCells can be configured individually. This may result in redundant and repetition parameter signaling if multiple cells share the same parameters. Due to the different structure of configurations of PCell and SCell, only the common Scell configuration is focused in euCA.

But in NR, RRCReconfiguration message[2] may include one field of masterCellGroup, and allow at most one secondaryCellGroup field in Rel-15, and both fields have the same IE of CellGroupConfig. In CellGroupConfig IE, although the SPCell and the SCell(s) have different structures due to different usage, they still use the same IE for the common configuration and dedicated configuration in lower level. Therefore, all cells (SPCells and SCells) are configured with the same structure (ServingCellConfigCommon IE and ServingCellConfig IE) as shown in the figure below:

Figure1 Structure of CellGroupConfig IE

Observation: SPCell and SCell  have almost same configuration structure.

Due to the unified structure, the common SCell configuration solution in LTE could be extended to common cell configuration of all kinds of cells in NR, and the SPCell/SCell(s) could use the common cell configuration as a template to perform a delta configuration.

Proposal 2: In NR, SPCell and SCell(s) could all be configured base on “common cell configuration”.

Proposal 3: A template of common cell configuration is used, and all the SPCell and SCell(s) perform delta-configuration based on the template.

In MR-DC except NR-DC, the common (S)Cell enhancement could be performed separately for MCG and SCG respective of RAT. And in NR-DC cases, although all kinds of cells in NR have the same common and dedicated configuration structure, the MCG and the SCG are configured separately. Therefore, the PCell and the SCell(s) in MCG could use a list of common configuration, and the SPCell and the SCell(s) in SCG could use another list of common configuration.

Proposal 4: Separate common cell configurations should be configured for different CGs in MR-DC.

Conclusion

According to the analysis in section 2, we have:

Observation: SPCell and SCell have almost same configuration structure.

And we propose:

Proposal 1: LTE euCA common SCell configuration could be reused in NR as baseline.

Proposal 2: In NR, SPCell and SCell(s) could all be configured base on “common cell configuration”.

Proposal 3: A template of common cell configuration is used, and all the SPCell and SCell(s) perform delta-configuration based on the template.

Proposal 4: Separate common cell configurations should be configured for different CGs in MR-DC.

Reference

- R2-1809270 Signalling for euCA Nokia, Nokia Shanghai Bell

- TS38.331-f40