3GPP TSG RAN WG2 Meeting #107                                                        	R2-1908928

Prague, Czech, 26 August - 30 August 2019             Resubmission of R2-1906646

Source:	CATT

Title:	Consideration on the UE capability of supporting CHO

Agenda Item:	11.9.3.4

Document for:	Discussion and Decision

Introduction

In RAN2#105bis meeting, the agreements for NR conditional handover were agreed as below:

Agreements

0: CHO is introduced in NR to solve robustness/reliability issue.

1:The LTE agreements below are applicable for NR:

a:CHO is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

b: Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

c:Support configuration of one or more candidate cells for conditional handover;

=>FFS how many candidate cells (UE and network impacts should be clarified).

=>FFS how to include the CHO conditions in UE configuration

d:The baseline operation for Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

e: The baseline operation for Conditional HO assumes the source RAN remains responsible for RRC until UE successfully sends RRC Reconfiguration Complete message to target RAN.

According to the agreements, the conditional handover is introduced in NR to solve robustness/reliability issue. In this contribution, we focus on how many candidate cells should be configured from the point of view of the UE.

Discussion

Considering the conditional handover is enhancement of handover for NR, the capability of the UE to support conditional handover should be reported to the network. Usage of conditional handover is decided by the network according to the capability of the UE and the network implementation.

Proposal 1: The UE capability to support conditional handover should be defined and reported to the network.

In CHO, the NW decides the number of candidate cells configured to the UE according to the situation of neighbor cells. All the candidate cells have prepared resources for the UE. If the UE is configured with multiple candidate cells, which exceeds the UE capability, some cells may be deleted according to the UE implementation. Therefore, the NW needs to know the max number of cells that t can be configured to the UE taking into accounbt the UE capability..

Observation 1: In CHO, too many candidate cells may lead to a waste of NW resources.  The NW should consider both the ability of UE and  neighbor cells when configuring CHO .

Proposal 2: The max cell number that the UE can support in CHO should be added to UE capability.

Proposals

According to the analysis in section 2, we have the following observations and proposals:

Proposal 1: The UE capability to support conditional handover should be defined and reported to the network.

Observation 1: In CHO, too many candidate cells may lead to a waste of NW resources.  And more candidate cells may depress the cell select efficiency of UE. So NW should consider both the ability of UE and the situation of neighbor cells.

Proposal 2: The max cell number of UE can support in CHO should be added to UE capability.

Reference

- TS 38.306	 User Equipment (UE) radio access capabilities

- RP-190489    “ New WID:  NR mobility enhancements”