3GPP TSG-RAN WG2 Meeting #107		R2-1908996

Prague, Czech Republic, August 26th - 30th, 2019

Source:	CATT, CAICT

Title:	Summary of Introduction of B1C signal in BDS system

Agenda Item:	11.20.1

Document for:	Discussion and Decision

- Introduction

The global navigation service in BDS system is supported by BDS phase III named “BDS-3”. The B1C signal (navigation data) is the main core signal in BDS-3 system. China Satellite Navigation Office published “Beidou Navigation Satellite System Signal in Space Interface Control Document—Open Service Signal B1C (Version 1.0)” [1] in December, 2017.  http://www.beidou.gov.cn/xt/gfxz/201712/P020171226741342013031.pdf

The B1C signal is transmitted by the Medium Earth Orbit (MEO) satellites and the Inclined Geo Synchronous Orbit (IGSO) satellites of BDS-3 for providing global open services.

BDS-3 system can’t support global navigation services without B1C signal.

So RP-181776 Introduction of B1C signal in BeiDou System [2] was discussed in 3GPP TSG RAN meeting #81(Gold Coast, Australia, 10/09/2018 to 13/09/2018). The decision in this meeting is that a CR should be involved to support B1C signal in 3GPP instead of a WI.

- Impact analysis

  - Summary of impact on introduction of signal B1C

The new assistance data of B1C signal in the network-assisted BDS should be introduced as a part of A-GNSS positioning methods in both LTE and NR.

Network-assisted B1C of BDS-3 positioning method can reuse the procedures of network-assisted BDS positioning method.

The impact of B1C signal in BDS-3 on Stage-2 and Stage-3 for LTE and NR is summarized here:

- Impact of B1C signal in BDS-3 on Stage-2 36.305 for LTE

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C as the reference file [1] should be involved in this spec.

- Impact of B1C signal in BDS-3 on Stage-2 38.305 for NR

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C as the reference file [1] should be involved in this spec.

- Impact of B1C signal in BDS-3 on Stage-3 36.355 for LTE and NR

  - BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C as the reference file [1] is added into section 2 as reference.

  - All impacted IEs have been pointed out and the main parts have been changed for introduction of B1C signal in 6.5.2.

  - New klobucharModel, Clock Model, Orbit Model, Almanac Model, UTC model, related IEs for B1C signal are added in 6.5.2.2.

Table 1   Summary of impact on Specs

| TS 36.305 | TS38.305 | TS 36.355 |
| --- | --- | --- |
| The reference file should be added in section 2 References | The reference file should be added in section 2 References | The reference file should be added in section 2 References |
| The reference file [1] should be mentioned in section 8.1 GNSS positioning methods. | The reference file [1] should be mentioned in section 8.1 GNSS positioning methods. | All impacted IEs have been pointed out and the main parts have been changed for introduction of B1C signal in 6.5.2 of TS36.355. |
|  |  | New klobucharModel, Clock Model, Orbit Model, Almanac Model, UTC model, related IEs for B1C signal are added in 6.5.2.2 of TS36.355. |

  - Impact analysis on 36.305

Network-assisted B1C of BDS-3 positioning method can reuse the procedures of network-assisted BDS positioning method. So the impact on 36.305 is about the B1C BDS-3 ICD file as reference.

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (ICD) should be added in section 2 References.

- The B1C BDS-3ICD file [1] should be mentioned in section 8.1 GNSS positioning methods.

  - Impact analysis on 38.305

The impact on 38.305 is the same as on 36.305 above.

  - Impact analysis on 36.355

The current working procedures can be reused for B1C signal in BDS-3, but new BDS-ID, some necessary information (e.g. klobucharModel for BDS, Clock Model for BDS, Orbit Model for BDS, UTC Model for BDS, and Almanac for BDS) and related description shall be introduced.

    - GNSS Assistance Data (Section 6.5.2.1)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideAssistanceData | No impact |
| GNSS-CommonAssistData | No impact |
| GNSS-GenericAssistData | No impact |
| GNSS-PeriodicAssistData | No impact |

    - GNSS Assistance Data Elements (Section 6.5.2.2)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTime | No impact |
| GNSS-SystemTime | No impact |
| GPS-TOW-Assist | No impact. |
| NetworkTime | No impact |
| GNSS-ReferenceLocation | No impact |
| GNSS-IonosphericModel > bds-klobucharModel-r16 | The IonosphericModel of BDS-3 is quite different from the existed GNSS systems, so new IE bds-klobucharModel-r16 should be defined. bds-klobucharModel-r16 is introduced for BDS-3 with condition B1C. |
| KlobucharModelParameter >dataID | B1I should be added in the description because KlobucharModelParameter works only for B1I in BDS. |
| BDS-KlobucharModel | New introduction for B1C BDS-3. |
| NeQuickModelParameter | No impact |
| GNSS-EarthOrientationParameters | GNSS-EarthOrientationParameters can be reused for B1C BDS-3. The reference ICD file of B1C BDS-3 shall be added in description. |
| GNSS-RTK-ReferenceStationInfo | No impact |
| GNSS-RTK-CommonObservationInfo | No impact. |
| GNSS-RTK-AuxiliaryStationData | No impact |
| GNSS-TimeModelList > deltaT | GNSS-TimeModelList can be reused for B1C BDS-3. deltaT is not present for B1C BDS-3. |
| GNSS-DifferentialCorrections | No impact. |
| GNSS-NavigationModel > NavModelSatelliteList >>GNSS- NavModelSatelliteElement >>>GNSS-ClockModel >>>>BDS-ClockModel2-r16 >>>GNSS-OrbitModel >>>>NavModel-BDS-KeplerianSet2-r16 | - BDS-ClockModel2-r16 is introduced for B1C BDS-3. - NavModel-BDS-KeplerianSet2-r16 is introduced for B1C BDS-3. - B1C health state is added in ‘GNSS to svHealth Bit String(8) relation’ table. - IOD of B1C is updated in ‘GNSS to iod Bit String(11) relation’ table. |
| StandardClockModelList | No impact |
| NAV-ClockModel | No impact |
| CNAV-ClockModel | No impact |
| GLONASS-ClockModel | No impact |
| SBAS-ClockModel | No impact |
| BDS-ClockModel | No impact |
| BDS-ClockModel2 | BDS-ClockModel2-r16 is introduced for B1C BDS-3 |
| NavModelKeplerianSet | No impact |
| NavModelNAV-KeplerianSet | No impact |
| NavModelCNAV-KeplerianSet | No impact |
| NavModel-GLONASS-ECEF | No impact |
| NavModel-SBAS-ECEF | No impact |
| NavModel-BDS-KeplerianSet | No impact |
| NavModel-BDS-KeplerianSet2 | NavModel-BDS-KeplerianSet2-r16 is introduced for B1C BDS-3 |
| GNSS-RealTimeIntegrity | No impact |
| GNSS-DataBitAssistance | No impact |
| GNSS-AcquisitionAssistance | No impact |
| GNSS-Almanac >GNSS-AlmanacList >>GNSS-AlmanacElement >>>AlmanacBDS-ReducedKeplerianSet-r16 >>>AlmanacBDS-MidiKeplerianSet-r16 | - AlmanacBDS-ReducedKeplerianSet-r16 is introduced for B1C BDS-3 as model-8. - AlmanacBDS-MidiKeplerianSet-r16 is introduced for B1C BDS-3 as model-9. |
| AlmanacKeplerianSet | No impact |
| AlmanacNAV-KeplerianSet | No impact |
| AlmanacReducedKeplerianSet | No impact |
| AlmanacMidiAlmanacSet | No impact |
| AlmanacGLONASS-AlmanacSet | No impact |
| AlmanacECEF-SBAS-AlmanacSet | No impact |
| AlmanacBDS-AlmanacSet | No impact |
| AlmanacBDS-ReducedKeplerianSet | AlmanacBDS-ReducedKeplerianSet-r16 is introduced for B1C BDS-3 |
| AlmanacBDS-MidiKeplerianSet | AlmanacBDS-MidiKeplerianSet-r16 is introduced for B1C BDS-3 |
| GNSS-UTC-Model | New utcModel field i.e. utcModel-6-r16 is added for B1C BDS-3. |
| UTC-ModelSet1 | No impact |
| UTC-ModelSet2 | No impact |
| UTC-ModelSet3 | No impact |
| UTC-ModelSet4 | No impact |
| UTC-ModelSet5 | No impact |
| UTC-ModelSet6 | UTC-ModelSet6-r16 is added for B1C BDS-3. |
| GNSS-AuxiliaryInformation | No impact |
| BDS-DifferentialCorrections | No impact |
| BDS-GridModelParameter | No impact |
| GNSS-RTK-Observations | No impact |
| GLO-RTK-BiasInformation | No impact |
| GNSS-RTK-MAC-CorrectionDifferences | No impact |
| GNSS-RTK-Residuals | No impact |
| GNSS-RTK-FKP-Gradients | No impact |
| GNSS-SSR-OrbitCorrections | No impact |
| GNSS-SSR-ClockCorrections | No impact |
| GNSS-SSR-CodeBias | No impact |

    - GNSS Assistance Data Request (Section 6.5.2.3)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestAssistanceData | No impact |
| GNSS-CommonAssistDataReq | No impact |
| GNSS-GenericAssistDataReq | No impact |
| GNSS-PeriodicAssistDataReq | No impact |

    - GNSS Assistance Data Request Elements (Section 6.5.2.4)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTimeReq | No impact |
| GNSS-ReferenceLocationReq | No impact |
| GNSS-IonosphericModelReq | Request for bds-klobuchar is introduced for B1C BDS-3. |
| GNSS-EarthOrientationParametersReq | No impact |
| GNSS-RTK-ReferenceStationInfoReq | No impact |
| GNSS-RTK-AuxiliaryStationDataReq | No impact |
| GNSS-TimeModelListReq | No impact |
| GNSS-DifferentialCorrectionsReq | No impact |
| GNSS-NavigationModelReq | No impact |
| GNSS-RealTimeIntegrityReq | No impact |
| GNSS-DataBitAssistanceReq | No impact |
| GNSS-AcquisitionAssistanceReq | No impact |
| GNSS-AlmanacReq >modelID | Almanac Model ID shall be extended to support B1C BDS-3. |
| GNSS-UTC-ModelReq | No impact |
| GNSS-AuxiliaryInformationReq | No impact |
| BDS-DifferentialCorrectionsReq | No impact |
| BDS-GridModelReq | No impact |
| GNSS-RTK-ObservationsReq | No impact |
| GLO-RTK-BiasInformationReq | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesReq | No impact |
| GNSS-RTK-ResidualsReq | No impact |
| GNSS-RTK-FKP-GradientsReq | No impact |
| GNSS-SSR-OrbitCorrectionsReq | No impact |
| GNSS-SSR-ClockCorrectionsReq | No impact |
| GNSS-SSR-CodeBiasReq | No impact |

    - GNSS Location Information (Section 6.5.2.5)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideLocationInformation | No impact |

    - GNSS Location Information Elements (Section 6.5.2.6)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-SignalMeasurementInformation | No impact |
| MeasurementReferenceTime | No impact |
| GNSS-MeasurementList | No impact |
| GNSS-LocationInformation | No impact |

    - GNSS Location Information Request (Section 6.5.2.7)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestLocationInformation | No impact |

    - GNSS Location Information Request Elements (Section 6.5.2.8)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-PositioningInstructions | No impact |

    - GNSS Capability Information (Section 6.5.2.9)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideCapabilities | No impact |

    - GNSS Capability Information Elements (Section 6.5.2.10)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-CommonAssistanceDataSupport | No impact |
| GNSS-ReferenceTimeSupport | No impact |
| GNSS-ReferenceLocationSupport | No impact |
| GNSS-IonosphericModelSupport | IE ionoModel shall be extended to support B1C BDS-3. |
| GNSS-EarthOrientationParametersSupport | No impact |
| GNSS-RTK-ReferenceStationInfoSupport | No impact |
| GNSS-RTK-AuxiliaryStationDataSupport | No impact |
| GNSS-GenericAssistanceDataSupport | No impact |
| GNSS-TimeModelListSupport | No impact |
| GNSS-DifferentialCorrectionSupport | No impact |
| GNSS-NavigationModelSupport | model-7 should be added in both clockModel and orbitModel for B1C BDS-3. |
| GNSS-RealTimeIntegritySupport | No impact |
| GNSS-DataBitAssistanceSupport | No impact |
| GNSS-AcquisitionAssistanceSupport | No impact |
| GNSS-AlmanacSupport | model-8 and model-9 should added for B1C BDS-3. |
| GNSS-UTC-ModelSupport | model-6 should added for B1C BDS-3. |
| GNSS-AuxiliaryInformationSupport | No impact |
| BDS-DifferentialCorrectionsSupport | No impact |
| BDS-GridModelSupport | No impact |
| GNSS-RTK-ObservationsSupport | No impact |
| GLO-RTK-BiasInformationSupport | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesSupport | No impact |
| GNSS-RTK-ResidualsSupport | No impact |
| GNSS-RTK-FKP-GradientsSupport | No impact |
| GNSS-SSR-OrbitCorrectionsSupport | No impact |
| GNSS-SSR-ClockCorrectionsSupport | No impact |
| GNSS-SSR-CodeBiasSupport | No impact |

    - GNSS Capability Information Request (Section 6.5.2.11)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestCapabilities | No impact |

    - GNSS Error Elements (Section 6.5.2.12)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-Error | No impact |
| GNSS-LocationServerErrorCauses | No impact |
| GNSS-TargetDeviceErrorCauses | No impact |

    - Common GNSS Information Elements (Section 6.5.2.13)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-FrequencyID | The frequency of B2a is added into the table ‘Value & Explanation relation’ |
| GNSS-ID | No impact |
| GNSS-ID-Bitmap | No impact |
| GNSS-Link-CombinationsList | No impact |
| GNSS-NavListInfo | No impact |
| GNSS-NetworkID | No impact |
| GNSS-PeriodicControlParam | No impact |
| GNSS-ReferenceStationID | No impact |
| GNSS-SignalID | ‘B1C’, ‘B2a’ and ‘B1C+B2a’ should be added in the table ‘System to Value & Explanation relation’. |
| GNSS-SignalIDs | ‘B1 B1C’, ‘B2 B2a’ and ‘B1C+B2a’ should be added in table ‘interpretation of the bit map in gnssSignalIDs-Ext’. |
| GNSS-SubNetworkID | No impact |
| SBAS-ID | No impact |
| SBAS-IDs | No impact |
| SV-ID | The value of satellite-id is updated in BDS, and the Reference file should be added in the description as well. |

–	GNSS-FrequencyID

New frequency should be added in the BDS part in the table ‘Value & Explanation relation’ to support BDS-3 system.

| System | Value | Explanation |  |
| --- | --- | --- | --- |
|  |  | Link | Centre Frequency [MHz] |
| BDS | 0 | B1 (Phase II) | 1561.098 |
|  | 1 | B1 (Phase III) | 1575.420 |
|  | 2 | B2 | 1207.140 |
|  | 3 | B3 | 1268.520 |
|  | 4 | B2a | 1176.45 |
|  | 5-7 | reserved |  |

–	GNSS-SignalID

The BDS part in the table ‘System to Value & Explanation relation’ should be updated for supporting BDS-3 system.

| BDS | 0 | B1 I |
| --- | --- | --- |
|  | 1 | B1 Q |
|  | 2 | B1 I+Q |
|  | 3 | B3 I |
|  | 4 | B3 Q |
|  | 5 | B3 I+Q |
|  | 6 | B2 I |
|  | 7 | B2 Q |
|  | 8 | B2 I+Q |
|  | 9 | B1 B1C |
|  | 10 | B2 B2a |
|  | 11 | B1C+B2a |
|  | 12-23 | Reserved |

–	GNSS-SignalIDs

The BDS part in ‘interpretation of the bit map in gnssSignalIDs-Ext’ should be updated to support BDS-3 system.

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L2 Z | L2C(M) | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) | L1C(P) | L1C(D+P) |
| GLONASS |  |  |  |  |  |  |  |  |
| Galileo | E1 B+C | E1 A+B+C | E6C | E6A | E6B | E6 B+C | E6 A+B+C | E5B I |
| BDS | B2 I+Q | B1C | B2a | B1C+B2a |  |  |  |  |

NOTE: “No impact” means UE can reuse the IE for B1C BDS-3 or the IE is not suitable for B1C BDS-3.

–	SV-ID

the Value of satellite-id of BDS should be updated to 0-62.

interpretation of satellite-id

| System | Value of satellite-id | Interpretation of satellite-id |
| --- | --- | --- |
| GPS | '0' – '62' '63' | Satellite PRN Signal No. 1 to 63 Reserved |
| SBAS | '0' – '38' '39' – '63' | Satellite PRN Signal No. 120 to 158 Reserved |
| QZSS | '0' – '4' '5 – '63' | Satellite PRN Signal No. 193 to 197 Reserved |
| GLONASS | '0' – '23' '24 – '63' | Slot Number 1 to 24 Reserved |
| Galileo | '0' – '35' '36' – '63' | Code No. 1 to 36 Reserved |
| BDS | '0' – '62' '63' | Satellite ranging code number signal No.1 to 63 [23], [xx] Reserved |

    - Definition of B1C BDS-3 specific IE in TS 36.355

According to the WID [1], some B1C BDS-3specific IE will be introduced to allow the native support for BDS-3. In this section, we list some B1C BDS-3specific IEs, and try to identify the definition of those B1C BDS-3 specific IEs.

      - Ionospheric Model for B1C BDS-3

In the current spec, 2 models have been defined for different GNSS.

- klobucharModel

- neQuickModel

According to the Ionospheric definition for BDS given in 7.8.1 in [1], we can find the formats of parameters for B1C BDS-3 are different from the existing klobuchar model. So, we suggest introducing the new klobuchar Model for B1C BDS-3 as IE BDS-KlobucharModel-r16. The definition of parameters for B1C BDS-3 can be found as follow:

-	BDS-KlobucharModel

-- ASN1START

BDS-KlobucharModel-r16::= CHOICE {

bds-klobucharModel-r16			BDS-KModelParameter-r16,

spare1	NULL

}

# BDS-KModelParameter-r16::= SEQUENCE {

dataID-r16			BIT STRING (SIZE (2)),

alfa1-r16			INTEGER (0..1023),

alfa2-r16			INTEGER (-128..127),

alfa3-r16			INTEGER (0..255),

alfa4-r16			INTEGER (0..255),

alfa5-r16			INTEGER (0..255),

alfa6-r16			INTEGER (-128..127),

alfa7-r16			INTEGER (-128..127),

alfa8-r16			INTEGER (-128..127),

alfa9-r16			INTEGER (-128..127),

...

}

-- ASN1STOP

BDS-KModelParameter field descriptions

dataID When dataID has the value ′01′ it indicates that the parameters have been generated by BDS-3 B1C, and UE shall use these parameters according to the description given in 7.8.1 in [38]. All other values for dataID are reserved.

alpha1 This field specifies the 1 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu.

alpha2 This field specifies the 2 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu.

alpha3 This field specifies the 3 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu.

alpha4 This field specifies the 4 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu. alpha5 This field specifies the 1 parameter of the Klobuchar model, as specified in [38]. Scale factor -2-3 TECu. alpha6 This field specifies the 6 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu. alpha7 This field specifies the 7 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu. alpha8 This field specifies the 8 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu. alpha9 This field specifies the 9 parameter of the Klobuchar model, as specified in [38]. Scale factor 2-3 TECu.

      - Clock Model for B1C BDS-3

6 clock models have been defined for different GNSS currently:

- Model 1: StandardClockModelList;

- Model 2: NAV-ClockModel;

- Model 3: CNAV-ClockModel;

- Model 4: GLONASS-ClockModel;

- Model 5: SBAS-ClockModel;

- Model 6: BDS-ClockModel.

According to the clock model definition for B1C BDS-3 given in 7.5.1 in [1], we can find the clock model parameters for B1C BDS-3 are different from the existing models. So, we suggest introducing the clock model for B1C BDS-3 as model 7. The definition of the clock model for B1C BDS-3 can be found as follow:

–	BDS-ClockModel2

-- ASN1START

BDS-ClockModel2-r16 ::= SEQUENCE {

bdsToc-r16		INTEGER (0..2047),

bdsA0-r16		INTEGER (-16777216..16777215),

bdsA1-r16		INTEGER (-2097152..2097151),

bdsA2-r16		INTEGER (-1024..1023),

bdsTgdB1Cp-r16	INTEGER (-2048..2047),

bdsIscB1Cd-r16	INTEGER (-2048..2047),

...

}

-- ASN1STOP

BDS-ClockModel2 field descriptions

bdsToc Parameter Toc, Clock correction parameters reference time (seconds), see [xx], 7.5.1. Scale factor 300 seconds.

bdsA0 Parameter a0, Satellite clock time bias correction coefficient (seconds), see [xx], 7.5.1. Scale factor 2-34 seconds.

bdsA1 Parameter a1, Satellite clock time drift correction coefficient (sec/sec), see [xx], 7.5.1. Scale factor 2-50 sec/sec.

bdsA2 Parameter a2, Satellite clock time drift rate correction coefficient (sec/sec2), see [xx], 7.5.1. Scale factor 2-66 sec/sec2.

bdsTgdB1Cp Parameter TGDB1Cp Group delay differential of the B1C pilot component (seconds), see [xx], 7.6.1. Scale factor is 2-34 seconds.

bdsIscB1Cd Parameter ISCB1Cd Group delay differential between the B1C data and pilot components (seconds), see [xx], 7.6.1. Scale factor is 2-34 seconds.

      - Orbit Model for B1C BDS-3

–	NavModel-BDS-KeplerianSet2

-- ASN1START

# NavModel-BDS-KeplerianSet2-r16::= SEQUENCE {

bdsIODE-r16				INTEGER (0..255),

bdsSatType-r16			INTEGER (0..3),

bdsToe-r16					INTEGER (0..2047),

bdsdeltaA-r16			INTEGER (-33554432.. 33554431),

bdsAdot-r16					INTEGER (-16777216.. 16777216),

bdsdeltaN0-r16			INTEGER (-65536.. 65535),

bdsdeltaN0dot-r16		INTEGER (-4194304.. 4194303),

bdsM0-r16					INTEGER (-4294967296.. 4294967295),

bdsE-r16 					INTEGER (0.. 8589934591),

bdsOmega-r16				INTEGER (-4294967296.. 4294967295),

bdsOmega0-r16			INTEGER (-4294967296.. 4294967295),

bdsI0-r16					INTEGER (-4294967296.. 4294967295),

bdsOmegaDot-r16 			INTEGER (-262144.. 262143),

bdsI0Dot-r16				INTEGER (-16384..	16383),

bdsCuc-r16 					INTEGER (-1048576.. 1048575),

bdsCus-r16					INTEGER (-1048576.. 1048575),

bdsCrc-r16					INTEGER (-8388608..	8388607),

bdsCrs-r16					INTEGER (-8388608..	8388607),

bdsCic-r16					INTEGER (-32768.. 32767),

bdsCis-r16					INTEGER (-32768.. 32767),

...

}

-- ASN1STOP

NavModel-BDS-KeplerianSet2 field descriptions

bdsIODE Parameter, Issue Of Data, Ephemeris (IODE), see [38], 7.4.1.

bdsSatType Parameter Satellite orbit type, defined in [38], 7.7.1.

bdsToe Parameter toe, Ephemeris reference time (seconds), defined in [38], 7.7.1. Scale factor 300 seconds.

bdsdeltaA Parameter A, Semi-major axis difference at reference time (meter), defined in [38], 7.7.1. Scale factor 2-9 meters.

bdsAdot Parameter , Change rate in semi-major axis (meter/sec), defined in [38], 7.7.1 Scale factor 2-21 meter/sec.

bdsdeltaN0 Parameter n0, Mean motion difference from computed value at reference time (semi-circles /sec), defined in [38], 7.7.1 Scale factor 2-44 pai/sec.

bdsdeltaN0dot Parameter n0dot, Rate of mean motion difference from computed value at reference time (semi-circles /sec2) , defined in [38], 7.7.1 Scale factor 2-57 pai/sec2.

bdsM0 Parameter M0, Mean anomaly at reference time (semi-circles) [23]. Scale factor 2-32 semi-circles.

bdsE Parameter e, Eccentricity [38]. Scale factor 2-34.

bdsOmega Parameter  Argument of perigee (semi-circles) [38]. Scale factor 2-32 semi-circles.

bdsOmega0 Parameter0, Longitude of ascending node of orbital plane at weekly epoch (semi-circles) [38]. Scale factor 2-32 semi-circles.

bdsI0 Parameter i0, Inclination angle at reference time (semi-circles) [38] Scale factor 2-32 semi-circles.

bdsOmegaDot Parameter , Rate of right ascension (semi-circles/sec) [38]. Scale factor 2-44 semi-circles/sec.

bdsI0Dot Parameter i0dot, Rate of inclination angle (semi-circles/sec) [38]. Scale factor 2-44 semi-circles/sec.

bdsCuc Parameter Cuc, Amplitude of cosine harmonic correction to the argument of latitude (radians) [38]. Scale factor 2-30 radians.

bdsCus Parameter Cus, Amplitude of sine harmonic correction to the argument of latitude (radians) [38]. Scale factor 2-30 radians.

bdsCrc Parameter Crc, Amplitude of cosine harmonic correction term to the orbit radius (meters) [38]. Scale factor 2-8 meters.

bdsCrs Parameter Crs, Amplitude of sine harmonic correction term to the orbit radius (meters) [38]. Scale factor 2-8 meters.

bdsCic Parameter Cic, Amplitude of cosine harmonic correction term to the angle of inclination (radians) [38]. Scale factor 2-30 radians.

bdsCis Parameter Cis, Amplitude of sine harmonic correction term to the angle of inclination (radians) [38]. Scale factor 2-30 radians.

      - Almanac Model for B1C BDS-3

In the current spec, 7 almanac models have been defined for different GNSS.

- Model 1: AlmanacKeplerianSet

- Model 2: AlmanacNAV-KeplerianSet

- Model 3: AlmanacReducedKeplerianSet

- Model 4: AlmanacMidiAlmanacSet

- Model 5: AlmanacGLONASS-AlmanacSet

- Model 6: AlmanacECEF-SBAS-AlmanacSet

- Modele7: AlmanacBDS-AlmanacSet-r12

According to the almanac definition for B1C BDS-3 given in 7.9 & 7.10 in [1], we can find the almanac parameters for B1C BDS-3 are different from the existing models. So, we suggest introducing the reduced almanac models as model 8 and Midi almanac models as model 9 together for B1C BDS-3. The definition of almanac parameters for B1C BDS-3 can be found as follow:

–	AlmanacBDS-ReducedKeplerianSet

-- ASN1START

# AlmanacBDS-ReducedKeplerianSet-r16 ::= SEQUENCE {

bdsSatType-r16					INTEGER (0..3),

svID-r16						SV-ID,

redAlmDeltaA-r16				INTEGER (-128..127),

redAlmOmega0-r16				INTEGER (-64..63),

redAlmPhi0-r16					INTEGER (-64..63),

redAlmHealth-r16				BIT STRING (SIZE(8)),

...

}

-- ASN1STOP

AlmanacBDS-ReducedKeplerianSet field descriptions

bdsSatType This field identifies the satellite orbit type, defined in [xx], 7.10.1.

svID This field identifies the satellite for which the GNSS Almanac Model is given.

redAlmDeltaA Parameter A, meters [xx]. Scale factor 2+9 meters.

redAlmOmega0 Parameter 0, semi-circles [xx]. Scale factor 2-6 semi-circles.

redAlmPhi0 Parameter 0, semi-circles [xx]. Scale factor 2-6 semi-circles.

redAlmHealth This field indicates the satellite health state defined in table 7-14 [xx].

–	AlmanacBDS-MidiKeplerianSet

-- ASN1START

# AlmanacBDS-MidiKeplerianSet-r16 ::= SEQUENCE {

bdsSatType-r16				INTEGER (0..3),

svID-r16					SV-ID,

midiAlmE-r16				INTEGER (0..2047),

midiAlmDeltaI-r16			INTEGER (-1024..1023),

midiAlmOmegaDot-r16			INTEGER (-1024..1023),

midiAlmSqrtA-r16			INTEGER (0..131071),

midiAlmOmega0-r16			INTEGER (-32768..32767),

midiAlmOmega-r16			INTEGER (-32768..32767),

midiAlmM0-r16				INTEGER (-32768..32767),

midiAlmAf0-r16				INTEGER (-1024..1023),

midiAlmAf1-r16				INTEGER (-512..511),

redAlmHealth-r16			BIT STRING (SIZE(8)),

...

}

-- ASN1STOP

AlmanacBDS-MidiKeplerianSet field descriptions

bdsSatType This field identifies the Satellite orbit type, defined in [xx], 7.9.1.

svID This field identifies the satellite for which the GNSS Almanac Model is given.

midiAlmE Parameter e, Eccentricity, dimensionless [xx]. Scale factor 2-16.

midiAlmDeltaI Parameter i, semi-circles [xx]. Scale factor 2-14 semi-circles.

midiAlmOmegaDot Parameter , semi-circles/sec [xx]. Scale factor 2-33 semi-circles/second.

midiAlmSqrtA Parameter , meters1/2 [xx]. Scale factor 2-4 meters1/2.

midiAlmOmega0 Parameter 0, semi-circles [xx]. Scale factor 2-15 semi-circles.

midiAlmOmega Parameter , semi-circles [xx]. Scale factor 2-15 semi-circles.

midiAlmM0 Parameter M0, semi-circles [xx]. Scale factor 2-15 semi-circles.

midiAlmAf0 Parameter afo, seconds [xx]. Scale factor 2-20 seconds.

midiAlmAf1 Parameter af1, sec/sec [xx]. Scale factor 2-37 seconds/second.

midiAlmHealth This field indicates the satellite health state defined in table 7-14 [xx].

      - UTC Model for B1C BDS-3

In the current spec, 7 almanac models have been defined for different GNSS.

- Model 1: UTC-ModelSet1

- Model 2: UTC-ModelSet2

- Model 3: UTC-ModelSet3

- Model 4: UTC-ModelSet4

- Model 5: UTC-ModelSet5

According to the almanac definition for B1C BDS-3 given in 7.12 in [1], we can find the BDT-UTC Time Offset Parameters for B1C BDS-3 are different from the existing models. So, we suggest introducing new model for B1C BDS-3. The definition of UTC model parameters for B1C BDS-3 can be found as follow:

–	UTC-ModelSet6

-- ASN1START

# UTC-ModelSet6-r16 ::= SEQUENCE {

utcA0-r16			INTEGER (-32768.. 32767),

utcA1-r16			INTEGER (-4096.. 4095),

utcA2-r16			INTEGER (-64.. 63),

utcDeltaTls-r16		INTEGER (-128..127),

utcTot-r16			INTEGER (0.. 65535),

utcWNot-r16			INTEGER (0.. 8191),

utcWNlsf-r16			INTEGER (0.. 8191),

utcDN-r16			INTEGER (0..7),

utcDeltaTlsf-r16	INTEGER (-128..127),

...

}

-- ASN1STOP

UTC-ModelSet6 field descriptions

utcA0 Parameter A0UTC, Bias coefficient of BDT time scale relative to UTC time scale, seconds [38]. Scale factor 2-35 seconds.

utcA1 Parameter A1UTC, Drift coefficient of BDT time scale relative to UTC time scale, sec/sec [38]. Scale factor 2-51 sec/sec.

utcA2 Parameter A2UTC, Drift rate coefficient of BDT time scale relative to UTC time scale, sec/sec2 [38]. Scale factor 2-68 sec/sec2.

utcDeltaTls Parameter tLS, Current or past leap second count, seconds [38]. Scale factor 1 second.

utcTot Parameter tot, Reference time of week, seconds [38]. Scale factor 24 second.

utcWNot Parameter WNot, Reference week number, weeks [38]. Scale factor 1 week.

utcWNlsf Parameter WNLSF, Leap second reference week number, weeks [38]. Scale factor 1 week.

utcDN Parameter DN, Leap second reference day number, days [38]. Scale factor 1 day.

utcDeltaTlsf Parameter tLSF, Current or future leap second count, seconds [38]. Scale factor 1 second.

      - GNSS Assistance Data Request Elements for B1C BDS-3

–	GNSS-IonosphericModelReq

The klobucharModel for B1C of BDS-3 is created, so the request of this mode shall also added in GNSS-IonosphericModelReq.

-- ASN1START

GNSS-IonosphericModelReq ::=	SEQUENCE {

klobucharModelReq		BIT STRING (SIZE(2))	OPTIONAL,	-- Cond klobuchar

neQuickModelReq			NULL					OPTIONAL,	-- Cond	nequick

...,

[[	bds-klobucharModelReq-r16 	BIT STRING (SIZE(2))	OPTIONAL	-- Cond bds-klobuchar

]]

}

-- ASN1STOP

–	GNSS-AlmanacReq

The almanac modes of B1C of BDS-3 are created, so the request to GNSS-Almanac assistance shall be updated as well.

-- ASN1START

# GNSS-AlmanacReq ::= SEQUENCE {

modelID				INTEGER(1..8)	OPTIONAL,

...,

[[	modelID-ext-r16		INTEGER(9..16)	OPTIONAL

]]

}

-- ASN1STOP

      - GNSS Capability Information Elements for B1C BDS-3

–	GNSS-IonosphericModelSupport

The new IonosphericModel is created, so the capability shall be updated as well.

-- ASN1START

GNSS-IonosphericModelSupport ::=	SEQUENCE {

ionoModel		BIT STRING {	klobuchar	(0),

neQuick		(1),

bds-klobuchar-r16	(2) } (SIZE (1..8)),

...

}

-- ASN1STOP

–	GNSS-NavigationModelSupport

The model-7 is added in the capability of supporting clockModel and orbitModel for B1C of BDS-3.

-- ASN1START

# GNSS-NavigationModelSupport ::= SEQUENCE {

clockModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7		(6) } (SIZE (1..8))		OPTIONAL,

orbitModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7		(6) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

–	GNSS-AlmanacSupport

-- ASN1START

GNSS-AlmanacSupport ::=	SEQUENCE {

almanacModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7		(6),

model-8		(7) } (SIZE (1..8))		OPTIONAL,

...,

[[	almanacModel-ext-r16		BIT STRING { model-9		(0) } (SIZE (1..8))		OPTIONAL

]]

}

-- ASN1STOP

The almanacModel is extended to support the new almanac mode for B1C of BDS-3.

–	GNSS-UTC-ModelSupport

The utc-Model is extended to support new UTC model for B1C of BDS-3.

-- ASN1START

GNSS-UTC-ModelSupport ::=	SEQUENCE {

utc-Model		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

    - Summary of impact on 36.355

The impact on stage3 36.355 introducing B1C BDS-3 is mainly in:

- GNSS Assistance Data

- GNSS Assistance Data Elements

- GNSS Assistance Data Request

- GNSS Assistance Data Request Elements

- GNSS Capability Information Elements

- Common GNSS Information Elements

The new definition IEs for B1C BDS-3 is mainly about those models as below:

- Ionospheric Model

- Clock Model

- Orbit Model

- Almanac Model

- UTC Model

These new added and modified IE can meet the request that global signal B1C in BDS-3 should be supported in 3GPP as network-assist positioning method.

Proposals

In this contribution, we summarized the impacts on introduction of signal B1C, especially the update in 36.355 and we get the following proposals:

Proposal 1: The global B1C signal in BDS-3 should be supported in 3GPP both LTE and NR in Rel-16.

Proposal 2: RAN2 is kindly requested to agree CRs in [2] [3] [4].

Reference

BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0), December, 2017

RP-181776 Introduction of B1C signal in BeiDou System, CAICT,

R2-1908993 Introduction of B1C signal in BDS system in A-GNSS (36.305 CR)	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, ZTE Corporation, MediaTek Inc.

R2-1908994 Introduction of B1C signal in BDS system in A-GNSS (36.355 CR)	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, ZTE Corporation, MediaTek Inc.

R2-1908995 Introduction of B1C signal in BDS system in A-GNSS (38.305 CR)	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, ZTE Corporation, MediaTek Inc.