3GPP TSG-RAN WG2 Meeting #107	R2-1909138

# Prague, Czech Republic, 26th - 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1142 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Reconfiguration via SRB3 in NR-DC and NGEN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 08/26/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 38.331 CR 1022 (R2-1906080), it is clarified that which fields could be included in RRCReconfiguration received via SRB3 in EN-DC. Since SRB3 is also supported in NR-DC and NGNE-DC and the design concept is the same as EN-DC. So, we believe that the same clarification is necessary. This CR proposes to clarify that only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3 for NR-DC and NGEN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 5.3.5.1, specify that only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3 for NR-DC and NGEN-DC. Impact analysis Impacted 5G architecture options: NR-DC, NGEN-DC Impacted functionality: RRC Reconfiguration procedure received via SRB3 in NR-DC, NGEN-DC Inter-operability: If the network implements the CR but the UE does not, there is no Inter-operability issue. The network will not include the IEs other than measConfig, radioBearerConfig and secondaryCellGroup and this does not cause any problem to legacy UE. If the UE implements the CR but the network does not, there is Inter-operability issue. The IEs other than measConfig, radioBearerConfig and secondaryCellGroup are not designed for NR-DC or NGEN-DC. The UE behaviour is unclear if those configurations are received via SRB3 in NR-DC or NGEN-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour is unclear if the configuration other than measConfig, radioBearerConfig and secondaryCellGroup are received via SRB3 in NR-DC or NGEN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC reconfiguration, successful

Figure 5.3.5.1-2: RRC reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/modify/release RBs, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups. As part of the procedure, NAS dedicated information may be transferred from the Network to the UE.

RRC reconfiguration to perform reconfiguration with sync includes, but is not limited to, the following cases:

-	reconfiguration with sync and security key refresh, involving RA to the PCell/PSCell, MAC reset, refresh of security and re-establishment of RLC and PDCP triggered by explicit L2 indicators;

-	reconfiguration with sync but without security key refresh, involving RA to the PCell/PSCell, MAC reset and RLC re-establishment and PDCP data recovery (for AM DRB) triggered by explicit L2 indicators.

In (NG)EN-DC and NR-DC, SRB3 can be used for measurement configuration and reporting, to (re-)configure MAC, RLC, physical layer and RLF timers and constants of the SCG configuration, and to reconfigure PDCP for DRBs associated with the S-KgNB or SRB3, and to reconfigure SDAP for DRBs associated with S-KgNB in (NG)EN-DC and NR-DC, provided that the (re-)configuration does not require any MN involvement. In (NG)EN-DC and NR-DC, only measConfig, radioBearerConfig and/or secondaryCellGroup are included in RRCReconfiguration received via SRB3.