3GPP TSG-RAN WG2 Meeting #107	R2-1909250

Prague,  Czech Republic, 26 – 30 August 2019

Agenda item:	11.10.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Direct activation in resume

WID:	LTE_NR_DC_CA_enh-Core - Release 16

Document for:	Discussion and Decision

# Introduction

In RAN2#105 following agreement was reached:

- The configured SCells (MCG and SCG) can be configured in deactivated or activated state by RRC upon addition or after a handover.  Timing requirements are up to RAN4.  FFS if this applies to resume.

In this paper we discuss the FFS point whether direct activation can be specified for resume case.

# Discussion

When studying the ASN.1 for reconfiguration, handover and resume one can see that same ASN.1 structure is used i.e. cellGroupConfig in all of these cases:

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                         SEQUENCE {

cellGroupId                                 CellGroupId,

rlc-BearerToAddModList                      SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig            OPTIONAL,   -- Need N

rlc-BearerToReleaseList                     SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity      OPTIONAL,   -- Need N

mac-CellGroupConfig                         MAC-CellGroupConfig                                         OPTIONAL,   -- Need M

physicalCellGroupConfig                     PhysicalCellGroupConfig                                     OPTIONAL,   -- Need M

spCellConfig                                SpCellConfig                                                OPTIONAL,   -- Need M

sCellToAddModList                           SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig           OPTIONAL,   -- Need N

sCellToReleaseList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex            OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent-v1530           ENUMERATED {true}                                           OPTIONAL    -- Cond BWP-Reconfig

]]

}

Observation: CellGroupConfig is used in handover, reconfiguration and resume, within which scelltoAddModList is included

So if we now agreed to support direct activation for reconfiguration it is implicitly added with ASN.1 to resume. We also notice that some companies were concerned about possible RAN4 implication for doing extra requirements for this case. Thus we propose:

Proposal: From RAN2 point of view direct activation is supported for reconfiguration, handover and resume. If RAN4 identifies that it is difficult or too much extra work to do requirements for all of these cases they may drop doing resume case.

If RAN4 ends up not doing requirements for resume case then we could just in field description indicate that configuration of direct activation is not supported for resume in this release and if later introduced the limitation can be removed.

Proposal: If RAN4 ends up not doing requirements for resume case then we could just in field description indicate that configuration of direct activation is not supported for resume in this release and if later introduced the limitation can be removed.

# Conclusions

In this contribution, we discussed direct activation in resume and came to following conclusion:

Observation: CellGroupConfig is used in handover, reconfiguration and resume, within which scelltoAddModList is included

Proposal: From RAN2 point of view direct activation is supported for reconfiguration, handover and resume. If RAN4 identifies that it is difficult or too much extra work to do requirements for all of these cases they may drop doing resume case.

Proposal: If RAN4 ends up not doing requirements for resume case then we could just in field description indicate that configuration of direct activation is not supported for resume in this release and if later introduced the limitation can be removed.

# References

- RP-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson, Nokia, Nokia Shanghai Bell, Huawei