3GPP TSG-RAN WG2 Meeting#107          							R2-1909269

Prague, Czech, 26th – 30th Aug 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the Cell Selection of the Private Network

Agenda item:		11.18

Document for: 	Discussion and Decision

# Introduction

In RAN Plenary #83, a new Work Item on private network was approved [1] with the following objectives:

- Support NPN functionality in NG-RAN: - CAG/SNPN relevant parameter broadcast from SIB [RAN2] - CAG/SNPN cell selection/reselection [RAN2] - CAG/SNPN cell access control [RAN2/3] - For CAG, in the case of Intra-RAT intra-system and inter-RAT intra-system, the connected mode mobility support [RAN2/3] - The connected mode mobility support within SNPN[RAN2/3]

In this contribution, we focus on the cell selection related issues.

# Discussion

In this chapter, we first analyze the private network impact on the definition of suitable cell, and then discuss the UE’s action when there is no suitable SNPN/CAG cell, at last we give our proposals on the interaction between AS and NAS.

## 2.1 Suitable cell definition

According to [2], the NPN was divided into the following two types:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public network integrated NPN, i.e. a non-public network deployed with the support of a PLMN.

And for the Public network integrated NPN, a CAG can be optionally used in order to prevent UEs not allowed to access a public network integrated NPN from accessing the public network integrated NPN.

For the CAG, besides the “allowed CAG list”, an indication may also be provided to indicate that the UE is only allowed to access 5GS via CAG cells, which also means that if not provided, the UE can access the network though either the normal cells or the CAG cells. Thus, we can get the following observation:

Observation 1: There would be three different settings from the UE side:

Setting 1: The UE can only access the network through cells with the matched SNPN ID.

Setting 2: The UE can only access the 5Gs via CAG cells.

Setting 3: The UE can access the 5Gs though either the normal cells or the CAG cells.

Obviously the definition of suitable cell in the 38.304 shall be changed to include above NPN scenarios. For the setting 1, we shall enhance that the cell belongs to the selected SNPN NID. For the setting 2, the cell shall belong to the allowed CAG list. For the setting 3, the cell shall belong to the allowed CAG list if the cell is a CAG cell.

Proposal 1: On the suitable cell definition, if the UE is set to the SNPN mode, the cell shall belongs to the selected SNPN, else if UE is only allowed to access 5GS via CAG cells or the cell is a CAG cell, the cell shall belongs to the allowed CAG list.

Proposal 2: Based on the proposal 1, the following description on the suitable cell definition can be taken as baseline for further discussions.

A cell is considered as suitable if the following conditions are fulfilled: - if set to the SNPN mode, the cell belongs to the selected SNPN else if UE is only allowed to access 5GS via CAG cells or the cell is a CAG cell the cell belongs to the allowed CAG list else The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list; - The cell selection criteria are fulfilled, see clause 5.2.3.2. According to the latest information provided by NAS: - The cell is not barred, see clause 5.3.1; - The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" (TS 22.261 [12]), which belongs to a PLMN or SNPN that fulfils the first bullet above.

## 2.2 UE’s action when no suitable cell

In this chapter, we discuss the UE’s action when there is no suitable NPN cell. For the PLMN selection, the UE will camp on an acceptable cell and enter into the “camped on any cell” state. However, when it comes to the SNPN, as described in [2]:

“When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4. Emergency services are not supported in SNPN access mode.”

Once the UE set to the SNPN mode, it can only camp on the SNPN cells, it can’t select any PLMN cells even there is no SNPN cell. Thus we get the following proposals:

Proposal 3: If the UE set to the SNPN mode, the UE can only camp on a SNPN cell.

Proposal 3.1: If there is no suitable SNPN cell, the UE shall try to camp on any SNPN cell and enter into the “Camped on any cell” state.

Proposal 3.2: If there is no SNPN cell, the UE shall enter into the “Any cell selection” state.

When the UE camp on any SNPN cell and enter into the “Camped on any cell” state, the UE can’t initiate an emergency call for that the SNPN network doesn’t support emergency service in the current release version. Thus, the definition on “acceptable cell” shall be modified for the SNPN network.

Observation 2: Emergency services are not supported in SNPN access mode.

Proposal 4: Delete the emergency call related limitation in the “acceptable cell” definition for the SNPN network. The following modification can be taken as a baseline for further discussion.

acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network: - The cell is not barred, see clause 5.3.1; - The cell selection criteria are fulfilled, see clause 5.2.3.2. Note: For the SNPN cell, the emergency call may not be required.

For the CAG, there are two settings according to the observation 1 in chapter 2.1:

1: The UE can only access the 5Gs via CAG cells.

2: The UE can access the 5Gs though either the normal cells or the CAG cells.

For the second setting, obviously, the UE can select CAG cells or the normal cells when there is no suitable cell. For the first setting, to keep consistence to the second setting, we think besides the CAG cells, the normal cell can also be camped to get limited service. By this method, we don’t need to change the current “States and state transitions in RRC_IDLE state and RRC_INACTIVE state” in 5.2.2 of 38.304 for the CAG.

Proposal 5: For all of CAG scenarios, the UE can select CAG cells or the normal cells when there is no suitable cell.

## 2.3 Interaction between AS and NAS

In this chapter, we give some short description to the interaction between AS and NAS. For the SNPN selection, the NAS shall indicate the selected SNPN NID to the AS, and the AS shall search for a suitable cell belonging to the provided SNPN NID to camp on. When the UE is no longer camped on a cell with the provided SNPN NID, AS shall inform NAS. For the CAG, the similar interaction shall be executed.

Proposal 6: The NAS shall indicate the selected SNPN NID/Allowed CAG list to the AS, the AS shall search for a suitable cell belonging to the provided SNPN NID/Allowed CAG list to camp on. When the UE is no longer camped on a cell belong to the selected SNPN NID/Allowed CAG list, AS shall inform NAS.

# Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: There would be three different settings from the UE side:

Setting 1: The UE can only access the network through cells with the matched SNPN ID.

Setting 2: The UE can only access the 5Gs via CAG cells.

Setting 3: The UE can access the 5Gs though either the normal cells or the CAG cells.

Proposal 1: On the suitable cell definition, if the UE is set to the SNPN mode, the cell shall belongs to the selected SNPN, else if UE is only allowed to access 5GS via CAG cells or the cell is a CAG cell, the cell shall belongs to the allowed CAG list.

Proposal 2: Based on the proposal 1, the following description on the suitable cell definition can be taken as baseline for further discussions.

A cell is considered as suitable if the following conditions are fulfilled: - if set to the SNPN mode, the cell belongs to the selected SNPN else if UE is only allowed to access 5GS via CAG cells or the cell is a CAG cell the cell belongs to the allowed CAG list else The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list; - The cell selection criteria are fulfilled, see clause 5.2.3.2. According to the latest information provided by NAS: - The cell is not barred, see clause 5.3.1; - The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" (TS 22.261 [12]), which belongs to a PLMN or SNPN that fulfils the first bullet above.

Proposal 3: If the UE set to the SNPN mode, the UE can only camp on a SNPN cell.

Proposal 3.1: If there is no suitable SNPN cell, the UE shall try to camp on any SNPN cell and enter into the “Camped on any cell” state.

Proposal 3.2: If there is no SNPN cell, the UE shall enter into the “Any cell selection” state.

Observation 2: Emergency services are not supported in SNPN access mode.

Proposal 4: Delete the emergency call related limitation in the “acceptable cell” definition for the SNPN network. The following modification can be taken as a baseline for further discussion.

acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network: - The cell is not barred, see clause 5.3.1; - The cell selection criteria are fulfilled, see clause 5.2.3.2. Note: For the SNPN cell, the emergency call may not be required.

Proposal 5: For all of CAG scenarios, the UE can select CAG cells or the normal cells when there is no suitable cell.

Proposal 6: The NAS shall indicate the selected SNPN NID/Allowed CAG list to the AS, the AS shall search for a suitable cell belonging to the provided SNPN NID/Allowed CAG list to camp on. When the UE is no longer camped on a cell belong to the selected SNPN NID/Allowed CAG list, AS shall inform NAS.

# Reference

RP-190729, New Work Item Proposal on Private Network Support for NG-RAN, China Telecom, RAN Plenary #83, Shenzhen, China, Mar 18-21, 2019

3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".