3GPP TSG-RAN2 Meeting #107	R2-1909364

# Prague, Czech Republic, 26 – 30 Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | DraftCR | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC running CR on TSC reference time distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR introduces the TSC reference time distribution enhancements specified in the Work Item on support of Industrial Internet of Things |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the new information element ReferenceTimeInfo RAN2 agreements: Confirm that we use LTE rel-15 SIB and RRC unicast based methods for reference time delivery R2 assumes that either SIB9 or a new SIB is used for reference time information broadcast delivery, depending on R3 discussion outcome. The field used for reference time information delivery is excluded from estimation of changes in system information. The reference time information shall correspond to a reference SFN, explicitly indicated in unicast signalling, FFS if inferred from the transmission of the SIB for SIB signalling. R2 assumes the UE shall use the end of the reference SFN value as the precise point in time to which the reference time corresponds. FFS whether the reference SFN refers to time in the future, past or whether this need to mandated one way or another. The UE considers the frame indicated by the referenceSFN nearest to the frame where the time information is received, which can be either in the past or in future, in case the time reference information is provided by unicast signalling SFN boundary at or immediately after the ending boundary of the SI-window in which SIB is transmitted is always used as a reference in case the time reference information is provided by broadcast signalling (as in LTE) “00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time)” as the origin of the time reference information, at least for the baseline case where time info type is not present or used (as in LTE). Specify uncertainty parameter in the reference time information in NR, encoding FFS - We will have the clock type field, similar to LTE. R2 considers that this have no relation to ongoing discussions in SA2 on TSC R2 assumes that some propagation delay compensation may be needed for distance > 200m. FFS what would be the method, e.g. based on current TA, and whether this can be left for UE implementation or something need to be specified. Signalling to support 10ns granularity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

NEXT CHANGE

### 6.3.2	Radio resource control information elements

#### –	ReferenceTimeInfo

The IE ReferenceTimeInfo contains timing information for 5G internal system clock used for, e.g., time stamping ingress and egress signalling events in the 5G system to determine 5G residence time for the corresponding signalling, see TS 23.501 [32], clause 5.27.1.2.

ReferenceTimeInfo information element

-- ASN1START

-- TAG-REFERENCETIMEINFO-START

ReferenceTimeInfo-r16 ::=		SEQUENCE {

time-r16							ReferenceTime-r16,

uncertainty-r16						FFS,						OPTIONAL,	-- Need R

timeInfoType-r16					ENUMERATED {localClock}		OPTIONAL,	-- Need R

referenceSFN-r16					INTEGER (0..1023)			OPTIONAL	-- Cond TimeRef

}

ReferenceTime-r16 ::=			SEQUENCE {

refDays-r16							INTEGER (0..72999),

refSeconds-r16						INTEGER (0..86399),

refMilliSeconds-r16					INTEGER (0..999),

refTenNanoSeconds-r16				INTEGER (0..99999)

}

-- TAG-REFERENCETIMEINFO-STOP

-- ASN1STOP

ReferenceTimeInfo field descriptions

referenceSFN This field indicates the reference SFN corresponding to the reference time information.

time This field indicates time reference with 10ns granularity. The indicated time in 10ns unit from the origin is refDays*86400*1000*100000 + refSeconds*1000*100000 + refMilliSeconds*100000 + refTenNanoSeconds. The refDays field specifies the sequential number of days (with day count starting at 0) from the origin of the time field. If the referenceTimeInfo field is received in RRCUnicast message, the time field indicates the time at the ending boundary of the system frame indicated by referenceSFN. The UE considers this frame (indicated by referenceSFN) to be the frame which is nearest to the frame where the message is received (which can be either in the past or in the future). If the referenceTimeInfo field is received in SystemInformationBlockTypeX, the time field indicates the time at the SFN boundary at or immediately after the ending boundary of the SI-window in which SystemInformationBlockTypeX is transmitted. If referenceTimeInfo field is received in SystemInformationBlockTypeX, this field is excluded when determining changes in system information, i.e. changes of time should neither result in system information change notifications nor in a modification of valueTag in SIB1.

timeInfoType If timeInfoType is not included, the time indicates the GPS time and the origin of the time field is 00:00:00 on Gregorian calendar date 6 January, 1980 (start of GPS time). If timeInfoType is set to localClock, the origin of the time is unspecified.

uncertainty This field indicates the uncertainty of the reference time information provided by the time field. FFS: encoding

| Conditional Presence | Explanation |
| --- | --- |
| TimeRef | The field is mandatory present if TimeReferenceInfo is included in RRCunicast message; otherwise the field is absent. |

Editor’s note: FFS: RRCunicast message and SystemInformationBlockTypeX message

Editor’s note: FFS: The procedure text on UE actions upon reciving RRCunicast message (e.g., 5.7.x.y) and SystemInformationBlockTypeX (e.g., clause 5.2.2.4.X).

END OF CHANGES