3GPP TSG-RAN WG2#107	Tdoc R2-1909511

Prague, Czech Republic, 26th August – 30th August 2019

Agenda Item:	11.12.2

Source:	Ericsson

Title:	On barometric pressure information in MDT

Document for:	Discussion

# Introduction

In RAN2 #106 in Reno, there was the following agreement on barometric pressure reporting for MDT [1];

Agreement

1	The uncompensated barometric pressure measurement if available can included in the MDT report.

In this contribution, we focus on the definition and representation of barometric pressure for MDT.

# Discussion

Barometric pressure sensor provides a pressure measurement p that is approximately related to a height h over a reference position, corresponding to a reference  pr:

p≈pre-Ch

where the constant C is equal to Mg/T0R, M is the molar mass of dry air, g is the gravitation acceleration at the earth surface, T0 is the standard temperature at sea level and R is the universal gas constant. This equation can be rewritten as

h≈1Clnprp

Uncompensated pressure measurements are already supported in LPP [2], and the representation of barometric pressure for MDT shall leverage from the LPP representation of barometric pressure

- Barometric pressure measurements are already represented in LPP [2]

LPP has the following representation of uncompensated barometric pressure in the Sensor-MeasurementInformation IE, which can be provided to the network as part of the Sensor-ProvideLocationInformation IE:

#### – Sensor-ProvideLocationInformation The IE Sensor-ProvideLocationInformation is used by the target device to provide location information for sensor-based methods to the location server. It may also be used to provide sensor specific error reason. -- ASN1START Sensor-ProvideLocationInformation-r13 ::= SEQUENCE { sensor-MeasurementInformation-r13 Sensor-MeasurementInformation-r13 OPTIONAL, sensor-Error-r13 Sensor-Error-r13 OPTIONAL, ..., [[ sensor-MotionInformation-r15 Sensor-MotionInformation-r15 OPTIONAL ]] } -- ASN1STOP

#### – Sensor-MeasurementInformation The IE Sensor-MeasurementInformation is used by the target device to provide UE sensor measurements to the location server. -- ASN1START Sensor-MeasurementInformation-r13 ::= SEQUENCE { measurementReferenceTime-r13 UTCTime OPTIONAL, uncompensatedBarometricPressure-r13 INTEGER (30000..115000) OPTIONAL, -- Cond Barometer ..., [[ uncertainty-r14 SEQUENCE { range-r14 INTEGER (0..1000), confidence-r14 INTEGER (1..100) } OPTIONAL ]] } -- ASN1STOP Conditional presence; Explanation. Barometer; The field is mandatory present if the Sensor-MeasurementInformation is provided for barometric pressure; otherwise it is not present.. Sensor-MeasurementInformation field descriptions. measurementReferenceTime This field provides the UTC time when the sensor measurements are performed and should take the form of YYMMDDhhmmssZ.. uncompensatedBarometricPressure This field provides the uncompensated barometric pressure as measured by the UE sensor, in units of Pa.. uncertainty This field provides the expected range for the pressure measurement in units of Pa and the confidence as a percentage that the true pressure lies in a range of (measurement – range) to (measurement + range)..

It is therefore relevant to reuse the value ranges and definitions also for MDT

Reuse definitions and representation of uncompensated barometric pressure from LPP for uncompensated barometric pressure reporting in MDT

As seen, it is natural to add the uncompensated barometric pressure measurements by using an octet string encoded as a Sensor-MeasurementInformation IE, following the structure proposed in [3].

Encode the uncompensated barometric pressure measurements by an octet string encoded as an LPP Sensor-MeasurementInformation IE

RAN 2 to agree on the text proposal in R2-1909504 [3].

# Conclusion

In section 2 we made the following observation:

- Barometric pressure measurements are already represented in LPP [2]

Based on the discussion in section 2 we propose the following:

- Reuse definitions and representation of uncompensated barometric pressure from LPP for uncompensated barometric pressure reporting in MDT

- Encode the uncompensated barometric pressure measurements by an octet string encoded as an LPP Sensor-MeasurementInformation IE

- RAN 2 to agree on the text proposal in R2-1909504.

# References

Chairman’s notes, RAN2 #106

- [2]	3GPP TS 36.355 V.15.4.0 LPP

- [3]	R2-1909504, UE Location Report Capability Indication for MDT, Ericsson