3GPP TSG RAN WG2 #107		                                                                         R2-1909566

Prague, Czech, 26th – 30th August, 2019

Source: 	Intel Corporation

Title:	Further criteria on whether the UE uses 2-step RACH or 4-step RACH

Agenda item:	11.13.4

Document for:	Discussion and Decision

# Introduction

- In RAN2#105bis, the following agreement was made on whether to use 2-step and 4-step RACH:

- Criteria on whether the UE uses 2-step RACH or 4-step RACH shall be clearly specified

- In RAN2#106, it is also agreed that

- 2-step RACH selections can be based on indicating to all UEs via SIB, or dedicated configuration in RRC_CONNECTED/INACTIVE/IDLE states.

- FFS if radio quality is used for 2-step RACH selection.

In the contribution, the criteria on whether the UE uses 2-Step RACH or 4-Step RACH is further discussed.

# Discussion

Network may want to control the amount of contention and/or UL interference using 2-Step RACH procedure, depending on how the radio resources for MsgA PUSCH are allocated. Hence it is beneficial for the network to configure which use cases/RA triggers to apply the 2-step CBRA procedure and which will use only 4-step CBRA procedure. Such network control can use SIB or dedicated DL signalling to indicate whether 2-step RACH should be applied to certain random access events. The SIB signalling can be used to control the random access events related to transition from idle/inactive to connected as well as for RRC re-establishment, while dedicated signaling (such as RRC Reconfiguration message or PDCCH order) could be used for other random access events that occur in connected mode.

2-step RACH can reduce the number of handshaking between the gNB and the UE (i.e. reduce the latency for e.g. SR transmission, time alignment for UL data availability) and/or allow the UE to move back to high power saving state (e.g. idle mode, inactive state) quicker in the case of small user data transmission (R17 small data transmission) and RAN based notification area updates.

Observation#1: 2-step RACH is useful in terms of reducing user plane latency and/or reducing UE power consumption for small data transmissions (e.g. SR transmission and UL time alignment for available UL data, Rel-17 small data transmission, RAN based notification area update etc.).

It can also be used to reduce control plane latency, i.e. faster RRC Connection establishment and resume procedure due to the reduced steps. Another scenario where 2-step RACH is useful is during handover where dedicated preamble is not available due to high RACH load, it continues to allow 2-step contention based RACH procedure to be used for handover and hence reducing handover interruption time even when contention based RACH is used.

Observation#2: 2-step RACH can also help in reducing control plane latency for initial access and for RRC resume, and in reducing handover interruption time even when contention based RACH is used.

Even though RAN2 have agreed to apply 2-Step CBRA procedure for all RA triggers for 4-step CBRA procedure, in time of high 2-step RACH load, the network may want to limit the 2-step CBRA procedure usage to some of the key RA triggers, as explained in observation#1 and #2.  Hence it is proposed:

Proposal#1: Network should be able to control which RA triggers/procedures can use 2-Step CBRA procedure at any given time.

Proposal#1.1: The RACH type to be used for idle mode and RRC inactive RA triggers (e.g. RNAU, RRC setup, RRC resume) are signaled via SIB while for RRC_Connected mode RA triggers (e.g. SR transmission, UL TA and handover etc.) are signaled via dedicated signaling.

There are proposals to introduce load factor to distribute the UEs using 2-step and 4-step CBRA. As mentioned in the observations, the main motivation of introducing 2-step RACH is to reduce user plane as well as control plane latency for some of the RA triggers, and not about distributing the load for 2-step and 4-step CBRA.  Furthermore, if RAN1 agreed to introduce RSRP as criteria, it can be used as load control to control the amount of cell edge UEs triggering 2 step RACH.

Proposal#2: No load factor is introduced for distributing the UEs using 2-step and 4-step CBRA.

There are also proposals to introduce logical channel type restriction on triggering 2-step RACH. The reason for this is that some low latency services (e.g. URLLC) require less latency than other services (e.g. eMBB, voice). Note that random access is only triggered when dedicated resources are not configured for SR or when UL time alignment timer has expired. If such low latency URLLC services are configured, it is quite unlikely that dedicated resources are not configured for SR. Similar, the network will not allow the UL time alignment timer to expire (i.e. the network will maintain UL sync by sending of TAC) when URLLC services are established.

Proposal#3: Logical channel type restriction is not introduced as a condition to trigger 2-step RACH.

# Conclusions

In this contribution, we discussed the criteria on whether the UE uses 2-Step RACH or 4-Step RACH and summarize the observations and proposals as follows:

Observation#1: 2-step RACH is useful in terms of reducing user plane latency and/or reducing UE power consumption for small data transmissions (e.g. SR transmission and UL time alignment for available UL data, Rel-17 small data transmission, RAN based notification area update etc.).

Observation#2: 2-step RACH can also help in reducing control plane latency for initial access and for RRC resume, and in reducing handover interruption time even when contention based RACH is used.

Proposal#1: Network should be able to control which RA triggers/procedures can use 2-Step CBRA procedure at any given time.

Proposal#1.1: The RACH type to be used for idle mode and RRC inactive RA triggers (e.g. RNAU, RRC setup, RRC resume) are signaled via SIB while for RRC_Connected mode RA triggers (e.g. SR transmission, UL TA and handover etc.) are signaled via dedicated signaling.

Proposal#2: No load factor is introduced for distributing the UEs using 2-step and 4-step CBRA.

Proposal#3: Logical channel type restriction is not introduced as a condition to trigger 2-step RACH.