3GPP TSG-RAN WG2 #107	R2-1909818

# Prague, Czech Republic, August 26 – August 30, 2019

Agenda item:		11.11.3 (NR_UE_pow_sav-Core)

Source:	LG Electronics Inc., LG Uplus

Title: 	Considerations on state transition schemes to RRC_INACTIVE

Document for:	Discussion and Decision

1.	Introduction

A new WI on Power Saving was approved in RAN#84, and efficient transition out of RRC_CONNECTED is one of the main objective in this WI, as shown below:

Specify a mechanism for a UE to indicate its preference of transitioning out of RRC_CONNECTED state. [RAN2]

Regarding this topic, RAN2 made agreements in RAN2#105bis, as shown below:

Agreements

1.	Enable a mechanism for a UE to indicate to the network its preference of transitioning out of RRC_CONNECTED.  FFS what UE assistance and for what scenario.  The UE assistance email rapporteur will cover this in the email discussion.

2.	No consensus on UE autonomous transitions

In this document, we discuss different schemes of UE state transitions.

2.	Discussion

Assuming that RAN2 will introduce a Release Assistance Information (RAI), the potential time points that the UE can transit to RRC_INACTIVE are shown in Figure 1.

Figure 1: Potential time points that the UE can transit to RRC_INACTIVE

Each time point is explained in detail below:

- T1: Full autonomous transition, i.e. the UE transits immediately after transmitting the RAI

- T2: Semi-autonomous transition, i.e. the UE transits upon reception of L2 feedback for the RAI

- T3: Network controlled transition, i.e. the UE transits upon reception of the transition command

The network controlled transition is a basic scheme that works well for all cases. This scheme relies on the existing procedure such as RRCRelease, and there would be no state mismatch problem as the network commands the UE to transit to RRC_INACTIVE. However, as can be seen in Figure 1, this scheme takes long time for the UE to transit to RRC_INACTIVE after the UE indicates its preference to the network. Moreover, whether and when to move the UE to RRC_INACTIVE is up to network decision, and thus this scheme may not be suitable for the UE who wants to move to power saving mode quickly.

Observation1: Network controlled transition takes long time for the UE to transit to RRC_INACTIVE after the UE indicates its preference to the network.

The full autonomous transition is an opposite scheme to the network controlled transition scheme. In this scheme, the UE transmits RAI to network and transits immediately to RRC_INACTIVE without waiting for the feedback from the network. As the UE doesn’t know the RAI reception status of the network, this scheme has a potential risk of state mismatch between UE and network. If the transmission of RAI is not successful, the network considers the UE in RRC_CONNECTED but the UE is actually in RRC_INACTIVE, which may cause resource waste due to unnecessary resource reservation for the UE.

Observation2: Full autonomous transition, i.e. transition without feedback of RAI transmission has potential risk of state mismatch.

The semi-autonomous transition is a middle ground solution that meets the UE desire to move to power saving mode quickly while avoiding potential state mismatch problem. In this scheme, the UE transits to RRC_INACTIVE when it receives L2 feedback of the transmitted RAI. As the reception of RAI is acknowledged by the network, there would be no state mismatch problem. Moreover, as the UE does not have to wait for transition command from the network, the UE’s desire to move to power saving mode is met to a certain extent.

Observation3: Semi-autonomous transition, i.e. transition after reception of feedback of RAI transmission meets the UE desire to move to power saving mode quickly while avoiding potential state mismatch problem.

With the observations above, we think the semi-autonomous transition scheme provides good balance between UE and network, and thus propose to go this way.

Proposal1: The UE transits to RRC_INACTIVE upon reception of L2 feedback of MAC PDU including Release Assistance Information.

One issue with Proposal1 is how the L2 feedback is realized. In NR, HARQ feedback is not defined for UL data transmission. Therefore, other L2 feedback should be utilized.

One candidate may be RLC feedback. If the RAI is a RRC message, the AM RLC entity is used, and RLC status report can be used as a L2 feedback for the RAI.

However, if the RAI is a MAC signalling, e.g. Zero BSR or new MAC CE, the RLC status report cannot be used because the AM RLC entity is not used. In this case, UL grant for new transmission can be used as a L2 feedback. Note that the UL grant for new transmission can also be used as a L2 feedback for the case when RAI is a RRC message.

Alternatively, a new MAC CE can be introduced for L2 feedback.

Proposal2: Discuss what should be used as a L2 feedback for the MAC PDU including Release Assistance Information.

3.	Proposals

For the efficient transition to RRC_INACTIVE, we have following proposals:

Proposal1: The UE transits to RRC_INACTIVE upon reception of L2 feedback of MAC PDU including Release Assistance Information.

Proposal2: Discuss what should be used as a L2 feedback for the MAC PDU including Release Assistance Information.