3GPP TSG-RAN2 Meeting #107 	R2-1910270

Prague, Czech Republic, 26th – 30th August 2019	(Revision of R2-1907543)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | DraftCR | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UL path switch in case of SCG failure in MR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-08-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | If split SRB1 is configured and SCG is the primary path for that SRB and uplink PDCP duplication is not configured, upon the detection of an SCG failure, it is not clear how the UE can send the SCG Failure Information to the MN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 7.7, it is described that during SCG failure, if the UE is configured with SCG as the primary path for split SRB and uplink PDCP duplication is not configured, the UE switches the primary path of the split SRB to the MCG to send SCG Failure Information to the MN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence would be that UE has to do unnecessary re-establishment when an SCG failure happens, if the SCG was the primary path of the split SRB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 SCG/MCG failure handling |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.331, TS 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

If radio link failure is detected for MCG, the UE initiates the RRC connection re-establishment procedure.

The following SCG failure cases are supported:

-	SCG RLF;

-	SN change failure;

-	For EN-DC, NGEN-DC and NR-DC, SCG configuration failure (only for messages on SRB3);

-	For EN-DC, NGEN-DC and NR-DC, SCG RRC integrity check failure (on SRB3).

Upon SCG failure the UE suspends SCG transmissions for all radio bearers and reports the SCG Failure Information to the MN, instead of triggering re-establishment. During SCG failure, if the UE is configured with SCG as the primary path for split SRB, the UE switches the primary path of the split SRB to MCG in order to send SCG Failure Information to the MN and enable transmission of PDCP status reports while SCG is suspended.

In all SCG failure cases, the UE maintains the current measurement configurations from both the MN and the SN and the UE continues measurements based on configuration from the MN and the SN if possible. The SN measurements configured to be routed via the MN will continue to be reported after the SCG failure.

NOTE:	UE may not continue measurements based on configuration from the SN after SCG failure in certain cases (e.g. UE can not maintain the timing of PSCell).

The UE includes in the SCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN.	The MN handles the SCG Failure Information message and may decide to keep, change, or release the SN/SCG. In all the cases, the measurement results according to the SN configuration and the SCG failure type may be forwarded to the old SN and/or to the new SN.

END OF CHANGES