3GPP TSG-RAN WG2 #107	 TDoc R2-1910308

# Prague, Czech, 26th – 30th August 2019

Agenda Item:	11.4.6

Source:	Ericsson

Title:	On the sidelink QoS flow and radio bearer

Document for:	Discussion, Decision

# 1	Introduction

In the latest RAN2#105 meeting, it has been agreed that per-flow QoS model is used in SL unicast while per-packet QoS model is used for SL groupcast/broadcast, and it is up to SA2 to make the final decision. Besides, access stratum treatment for SL unicast/groupcast/broadcast is based on SL radio bearer (SLRB). In the latest reply from SA2, it’s agreed that per QoS flow framework will be adopted for SL unicast/groupcast/broadcast.

RAN2#105 Agreements: 4b: From RAN2 perspective, per-flow QoS model is preferred for NR SL unicast. 4f: RAN2 agrees that from RAN2 perspective, per-packet QoS model is preferred for NR SL broadcast. Also RAN2 prefers to apply per-packet QoS based model for SL groupcast. 4a: For V2X transmission in SL unicast, SLRB configurations are NW configured or pre-configured. The configuration of each SLRB may include transmission related parameters which do not need to be known by the peer UE, plus some parameters that are configured also need to be known by the peer UE. 4e: For V2X transmission in SL gouprcast or SL broadcast, SLRB configurations are NW configured or pre-configured. The configuration of each SLRB may include only transmission related parameters which do not need to be known by the peer UEs. S2-1904823 LS response on unicast, groupcast and broadcast in NR sidelink SA2 Response: It was agreed in SA2 to use the bearer based QoS Model (i.e. Per-Flow QoS model) for unicast, groupcast and broadcast operations (please see the attached S2-1904426). SA2 believes that a single PC5 QoS model for all NR sidelink cast type aligned with the Uu QoS model simplifies the overall system behavior. SA2 kindly requests RAN2 to take this information into account in their work.

In this paper, we further discuss the configuration of QoS flow and SLRB.

# 2	Discussion

## 2.1 General issues

In NR Uu, multiple QoS flows may be mapped to the same radio bearer instead of one-to-one bearer mapping in LTE Uu. The radio bearer should support the requirements of all QoS flows that it is associated with. In this way, RAN can support tremendous types of QoS flows with limited complexity and configuration options.

Similar feature should be used in NR SL. 28 use cases in [1] indicates possibly 28 types of QoS flows / PC5 QoS profiles (per packet model for groupcast and broadcast use cases) while configuring one radio bearer per use case at access stratum could be too complex or not necessary. Note that for some use cases the difference of their QoS requirements are rather minor.

Note that SL unicast, groupcast and broadcast may use different configurations in the lower layers, e.g. SL unicast supports RLC AM. Thus, we believe it is beneficial to use different SLRBs for different casting modes, i.e. unicast/groupcast/broadcast. Accordingly, the UE multiplexes the packets of different QoS flows to the same radio bearer only if they are of same casting mode. To enable this, each packet delivered from upper layer is tagged with an extra identifier reflecting casting mode. The access stratum maps the QoS flow to the corresponding SLRBs using SDAP.

A given SL QoS flow/SLRB is associated with either SL unicast, groupcast or broadcast.

Packets of the same casting mode but different SL QoS flows can be mapped to the same SLRB.

It is somewhat common understanding between companies that the SL will support also RRC signalling, at least for unicast connection establishment. Thus, there can be two types of bearers. One type of bearer to carry control signalling at least for the unicast case, and the other type of bearer is for data. Regarding the latter, we believe that data radio bearer can be applicable to all casting schemes, since all casting scheme will carry data, as it is discussed further down in this contribution. Multiple data radio bearers can be configured for different V2X services.

RAN2 considers two types of SL bearers, i.e. control signalling radio bearer (SL-SRB) to carry control signalling at least for unicast, and data radio bearer (SL-DRB) for data for all casting types.

The decision of whether to establish/release a SL QoS flow/SL-DRB may depend on channel conditions estimated by the transmitter side. In fact, for all casting schemes, the transmitter can evaluate to quality of the SL channel and determine whether to establish/release a SL bearer, e.g. on the basis of the congestion/interference level of the channel and QoS requirements associated to the specific V2X traffic.

For SL unicast/groupcast/broadcast, a SL QoS flow/SL-DRB is established/released taking into account the QoS properties and channel conditions at least at the transmitter side.

## 2.3 PC5-S and PC5-RRC transmission

Agreements on PC5-RRC signalling flow: 1: Separate RRC messages are defined capability transfer and for AS-layer configuration. FFS on whether the two messages can be transmitted together in the same MAC PDU. 2: Set the following 2a, 2b and 2c as RAN2 working assumption: 2a: Do not encapsulate PC5-S message related to link setup into PC5-RRC message for AS-layer configuration. 2b: PC5-RRC message for AS-layer configuration is not to be sent unprotected, so is not to be sent together with PC5-S messages like Direct Communication Request. 2c: Do not encapsulate PC5-S message related to link setup into PC5-RRC message for capability information.

It has been agreed in the last meeting that PC5-S message related to link setup will not be encapsulated into PC5-RRC for capability information. This subsection further discussion how they should be sent with respect to SL-SRB and priority.

First of all, as described in TR 23.786 solution#11, the initiating PC5-S message is broadcast for the purpose of interested UE discovery while later PC5-S messages are unicast between two UEs for ID exchange and security setup.

TR 23.786 Solution#11: - "UE oriented layer 2 link establishment" operates as below and Figure 6.11.3.1-1 shows the procedure: - The Direct Communication Request message can be sent by UE-1 with broadcast mechanism, i.e. to a broadcast address associated with the application instead of the L2 ID of UE-2. The upper identifier of UE-2 is included in the Direct Communication Request message to allow UE-2 to decide on if to respond to the request. The Source L2 ID of this message should be the unicast L2 ID of the UE-1. - The Direct Communication Request message should be transmitted using default AS layer setting e.g. broadcast setting, that can be understood by UE-2. Figure 6.11.3.1-1: UE oriented layer 2 link establishment procedure

Considering the fact that the PC5-S message related to link setup is transmitted before the link setup or unicast SLRB setup, we believe the PC5-S message related to like setup is transmitted using broadcast SLRB, e.g. SL-DRB, without lower layer scrambling and HARQ feedback. Note that L2 unicast can still be realized using L2 ID in this case just like how PC5-S is transmitted in LTE D2D.

PC5-S message related to link setup is transmitted via broadcast SL-DRB, while L2 unicast is supported using L2 ID.

On the other hand, PC5-RRC transmission for SL unicast occurs after the link setup and can be therefore transmitted via a unicast SLRB with scrambling and HARQ feedback. In our view, as also explained in proposal 3, a dedicated SL-SRB can be configured for PC5-RRC transmission. Also, such SL-SRB should be by default configured in order to ensure successful reception by the peer UE without AS layer handshake.

PC5-RRC message for SL unicast is transmitted via unicast SL-SRB which is of default configuration. FFS the detailed configuration w.r.t. HARQ, RLC mode.

Another issue related to PC5-RRC message transmission is whether it can be multiplexed with data into the same MAC PDU. Note that in NR Uu, RRC message is sent from SRB using control channel resource separated from shared channel resource. However, for NR SL, PSCCH resource is designed for SCI transmission but not for PC5-RRC message transmission. Therefore, RAN2 is suggested to discuss if PC5-RRC message can be multiplexed with data into the same MAC PDU or not.

RAN2 discusses if PC5-RRC message can be multiplexed with data into the same MAC PDU.

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

Proposal 1	A given SL QoS flow/SLRB is associated with either SL unicast, groupcast or broadcast.

Proposal 2	Packets of the same casting mode but different SL QoS flows can be mapped to the same SLRB.

Proposal 3	RAN2 considers two types of SL bearers, i.e. control signalling radio bearer (SL-SRB) to carry control signalling at least for unicast, and data radio bearer (SL-DRB) for data for all casting types.

Proposal 4	For SL unicast/groupcast/broadcast, a SL QoS flow/SL-DRB is established/released taking into account the QoS properties and channel conditions at least at the transmitter side.

Proposal 5	PC5-S message related to link setup is transmitted via broadcast SL-DRB, while L2 unicast is supported using L2 ID.

Proposal 6	PC5-RRC message for SL unicast is transmitted via unicast SL-SRB which is of default configuration. FFS the detailed configuration w.r.t. HARQ, RLC mode.

Proposal 7	RAN2 discusses if PC5-RRC message can be multiplexed with data into the same MAC PDU.

# References

- 3GPP, TR 22.886 Study on enhancement of 3GPP support for 5G V2X services (Release 16), 2018.