3GPP TSG-RAN WG2 #107	 TDoc R2-1910312

# Prague, Czech, 26th – 30th August 2019

Agenda Item:	11.4.7

Source:	Ericsson

Title:	On inter-RAT scheduling

Document for:	Discussion, Decision

# Introduction

RAN2#105-bis has agreed the following on inter-RAT resource allocation:

- Agreements on inter-RAT resource allocation: - 1: For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration. - 2: LTE Uu supports configuration for NR mode-2 operation through RRC signaling and through SIB signaling. However, on demand or per valid area mode-2 configuration via SIB signaling is not supported. - Agreements on inter-RAT resource allocation: - 1: New system information block type should be designed to support NR sidelink Tx and Rx resource pool configuration via LTE Uu. New system information will be defined as container (OCTET STRING) and actual information will follow what is defined in NR RRC. - 2: eNB should be able to configure the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling. - 3: NR Sidelink UE information and/or NR UE Assistance information will be transmitted as container (OCTET STRING) and actual information will be defined in NR RRC. - 4: Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC. - 5: gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

In this paper we address some additional aspects of inter-RAT resource allocation.

# Discussion

Summarizing the agreements on inter-RAT resource allocation, what is agreed is to use container (OCTET STRING) for the following signalling cases:

  - LTE Uu configures NR SL per SIB

  - NR SL UE sending NR SUI/UAI to LTE Uu

  - NR Uu configures LTE SL per SIB

  - NR Uu configures LTE SL per dedicated RRC

The agreements do not yet cover the following cases:

  - LTE Uu configures NR SL per dedicated RRC

  - LTE SL UE sending LTE SUI/UAI to NR Uu

For the remaining two cases, in our view, we do not see reasons why they should be treated differently as those cases that has been agreed. Thus, we propose to also use a container (OCTET STRING).

eNB uses a container (OCTET STRING) for configuring the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling.

LTE Sidelink UE information and/or LTE UE Assistance information will be transmitted as container (OCTET STRING) using relevant information from LTE RRC.

Another aspect worth discussion is the HARQ procedure in inter-RAT scheduling case. RAN1 has agreed that for inter-RAT scheduling, when NR Uu schedules LTE SL only SPS scheduling based on DCI activation/release is supported, when LTE Uu schedules NR SL only type 1 scheduling (configured/activated/deactivated) is supported. Since dynamic inter-RAT scheduling is not supported, we believe there is no need to introduce SR/BSR for such purpose.

RAN1-ah1901 Agreements: - LTE Uu to schedule NR sidelink mode 1 is supported: - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1 RAN1#96bis Agreements: Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least: - Spec impact - Flexibility - Performance, including latency - Implementation complexity - Timing of the activation/deactivation

RAN2 does not introduce SR/BSR for inter-RAT scheduling.

On the other hand, since HARQ procedure is still supported under inter-RAT scheduling scenario, NR SL UE may need resources for retransmission. In this case, since SR/BSR based dynamic scheduling is not supported, NR SL UE can only use provided configured grant for retransmission.

NR SL UE under inter-RAT scheduling uses configured grant for both initial transmission and re-transmission.

Besides, since SR/BSR is not supported for inter-RAT scheduling, UE can only indicate the demand of resources for transmission via UAI. One way to do it is to add an explicit indicator within the UAI such that gNB can understand the traffic indicated in the UAI is activated so that gNB can provide grant immediately. Note that in intra-RAT scheduling, traffic indicated in the UAI may be activated later and network can understand the activated traffic pattern by linking the LCH information in BSR to UAI.

For inter-RAT scheduling, explicit indicator is needed in UAI to identify activated traffic.

# Conclusions

Based on the discussion in section 2 we propose the following:

Proposal 1	eNB uses a container (OCTET STRING) for configuring the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling.

Proposal 2	LTE Sidelink UE information and/or LTE UE Assistance information will be transmitted as container (OCTET STRING) using relevant information from LTE RRC.

Proposal 3        RAN2 does not introduce SR/BSR for inter-RAT scheduling.

Proposal 4	NR SL UE under inter-RAT scheduling uses configured grant for both initial transmission and re-transmission.

Proposal 5	For inter-RAT scheduling, explicit indicator is needed in UAI to identify activated traffic.

# References