3GPP TSG-RAN2 Meeting #107	R2-1910444

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1204 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on non-critical extension for NRDC-Parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-0x-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the principle of non-critical extension, empty sequences may be used if length determinant is not required, e.g., it is at the end of a message or at the end of a structure contained in a BIT STRING or OCTET STRING. Currently, nonCriticalExtension in NRDC-Parameters makes the nonCriticalExtention appear in middle of the UE-NR-Capability-v1560 message, which brings a potential non-backward compatible issue if the nonCriticalExtention is extended in the future, as the network cannot decode the following fileds without knowing the exact extension length. Therefore it is necessary to clarify that the nonCriticalExtesion will never be used in the future. UE-NR-Capability-v1560 ::= SEQUENCE { nrdc-Parameters NRDC-Parameters OPTIONAL, receivedFilters OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs) OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- ASN1START -- TAG-NRDC-PARAMETERS-START NRDC-Parameters ::= SEQUENCE { measAndMobParametersNRDC MeasAndMobParametersMRDC OPTIONAL, generalParametersNRDC GeneralParametersMRDC-XDD-Diff OPTIONAL, fdd-Add-UE-NRDC-Capabilities UE-MRDC-CapabilityAddXDD-Mode OPTIONAL, tdd-Add-UE-NRDC-Capabilities UE-MRDC-CapabilityAddXDD-Mode OPTIONAL, fr1-Add-UE-NRDC-Capabilities UE-MRDC-CapabilityAddFRX-Mode OPTIONAL, fr2-Add-UE-NRDC-Capabilities UE-MRDC-CapabilityAddFRX-Mode OPTIONAL, lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-NRDC-PARAMETERS-STOP -- ASN1STOP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Dummify nonCriticalExtention in NRDC-Parameters. Impact Analysis Impacted 5G architecture options: NR-DC Impacted functionality: UE radio capability Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability problem, as nonCritialExtension in NRDC-Parameters has not been used. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability problem as nonCritialExtension in NRDC-Parameters has not been used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is a potential non-backward compatible problem if the nonCriticalExtention in NRDC-Parameters is extended in the future. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------START OF CHANGE-------------------------------------------

### 6.3.3	UE capability information elements

#### –	NRDC-Parameters

The IE NRDC-Parameters contains parameters specific to NR-DC, i.e., which are not applicable to NR SA.

NRDC-Parameters information element

-- ASN1START

-- TAG-NRDC-PARAMETERS-START

NRDC-Parameters ::=                 SEQUENCE {

measAndMobParametersNRDC            MeasAndMobParametersMRDC                    OPTIONAL,

generalParametersNRDC               GeneralParametersMRDC-XDD-Diff              OPTIONAL,

fdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

tdd-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddXDD-Mode               OPTIONAL,

fr1-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

fr2-Add-UE-NRDC-Capabilities        UE-MRDC-CapabilityAddFRX-Mode               OPTIONAL,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

dummy                SEQUENCE {}                                 OPTIONAL

}

-- TAG-NRDC-PARAMETERS-STOP

-- ASN1STOP

---------------------------------------------END OF CHANGE---------------------------------------------