3GPP TSG RAN WG2 Meeting #107 	R2-1910508

Prague, CZ, 26th -30th August 2019

Title:	NTN Regulatory Aspects

Source: 	Fraunhofer IIS, Fraunhofer HHI

Type:	pCR

Document for:	Agreement

Agenda Item:	11.6.4.3 Control Plane - Other

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

In TR 38.821, Annex C [1], Regulatory Aspects of inline interference between LEO and GEO satellite systems have been captured for service up- and downlink. This aspect should be also considered by RAN2.

A text proposal is provided for the Study Item Report TR 38.821, capturing the impacts on the RRC layer.

# Discussion

In the ITU Radio Regulations [2] the interference to any GEO network caused by a NGSO system is strictly constrained. One restriction to the system is that a NGSO system shall not cause unacceptable interference into any GEO network in the fixed satellite service for uplink and downlink in the same frequency (in-line interference), shown in Figure 1. For that reason, (ITU, 2016) limits the maximum equivalent power flux densities (EPFD) of the NGSO system to values which are not sufficient to enable satellite communication.

This document will focus on both the service uplink from the UE to the NGSO system (interfering the receiver on board of a GEO satellite) and the service downlink from the NGSO system to the UE (interfering a GEO receive station on earth).

Figure 1: In-line interference caused by NGSO systems (MEO or LEO)

Service uplink from UE to NGSO satellite:

Satellite specific user terminals are partly equipped with a dish antenna to reach high signal gains, due to its high directivity. Especially for frequencies in the FR2 and the use of satellite specific user terminals on ground with high directive antennas the problem of in-line interference arises for the service uplink in satellite system.

We look at a scenario in which the UE uses a directive antenna to communicate with a NGSO satellite. It may further be able to track the movement of the NGSO satellite with its antenna. Given the high directivity it is possible that the UE will glare a GEO satellite that is located in the same direction as the NGSO from the UE’s perspective. As the uplink transmission power could exceed the ITU regulatory constraints for in-line interference of the protected GEO satellites, this issue has to be addressed already in the initial and random access phase. In the following we assume that the UE is aware of its position as well as the network.

The problem with a heavy deployment of NGSO systems arises with so-called in-line interference. This type of interference occurs, whenever a NGSO satellite crosses the line of sight path of an Earth station and a GEO satellite as depicted in Figure 1.

To avoid that the cell of the NGSO satellite has to be switched off completely, only the UEs with highly directive antennas, which are pointing towards the GEO satellite should be disabled or hand over to other NGSO satellites.

Service downlink from NGSO satellite to UE:

In this case, the downlink signal from a NGSO satellite is interfering a GEO receive station on earth. The satellite beam pointing towards the GEO ground station either has to be steered to another direction or has to be switched off completely. The GEO receive stations especially for FR2 are already deployed and protected by ITU regulations.

Observation 1: In-line interference between NGSO and GSO satellite systems might occur especially when highly directive antennas are used in NTN scenarios.

Observation 2: To avoid in-line interference between NGSO and GSO satellite systems, a subset of the UEs within a beam or cell has to temporary avoid any uplink transmissions.

Uplink transmissions are performed by a UE either to establish the connectivity to the network (RACH procedure using PRACH resources) or to send control- or user plane data (PUCCH, PUSCH). To comply with the ITU regulatory constraints, all of these transmissions have to be avoided for the time interval, where the uplink transmissions would reach and interfere the GEO satellite.

RACH uplink transmissions:

To avoid RACH UL transmissions, a UE has to know, whether it is currently allowed to send the RACH preamble. The existing barring mechanism of 5G in TR 38.331 specifies to limit the access for specific static user groups as defined in the SIM cards, but not for individual UEs or for dynamic UE groups within a coverage area. So, the information to allow or block PRACH transmissions has to be provided to a UE via alternative mechanisms, e.g. MIB/SIB (which are specified in RRC layer) downlink transmissions defining these UEs, which are not allowed to transmit during a time interval, where inline interference would occur.

Uplink control/user plane transmissions:

The same information about the temporary blocked UEs as defined for the RACH uplink transmissions can be used by the UE to avoid transmissions of PUCCH and PUSCH for the time interval.

Proposal 1: Solutions to comply with the ITU regulatory constraints for in-line interference have impact on the RRC layer and are to be studied.

# Conclusion

This documents describes regulatory constraints from ITU to avoid interference between GSO and NGSO satellite systems. The observations and proposals are as follows:

Observation 1: In-line interference between NGSO and GSO satellite systems might occur especially when highly directive antennas are used in NTN scenarios.

Observation 2: To avoid in-line interference between NGSO and GSO satellite systems, only a subset of the UEs within a beam or cell have to avoid any uplink transmissions.

Proposal 1: Solutions to comply with the ITU regulatory constraints for in-line interference have impact on the RRC layer and are to be studied.

# References

[1] TR 38.821, V 0.7.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16), 2019-05

[2] ITU Radio Regulations 2016 Vol. 1

-- BEGIN TEXT PROPOSAL for TR 38.821 –

<7.3.x> RRC Layer Impacts from Regulatory Constraints

As described in Annex C, in the ITU Radio Regulations (ITU, 2016) the interference to any GEO network caused by a NGSO system is strictly constrained and NGSO systems shall not cause unacceptable interference into any GEO network in the fixed satellite service for uplink and downlink in the same frequency (in-line interference).

Uplink transmissions are performed by a UE either to establish the connectivity to the network (RACH procedure using PRACH resources) or to send control- or user plane data (PUCCH, PUSCH). To comply with the ITU regulatory constraints, all of these transmissions have to be avoided for the time interval, where the uplink transmissions would reach and interfere the GEO satellite.

RACH uplink transmissions:

To avoid RACH UL transmissions, a UE has to know, whether it is currently allowed to send the RACH preamble. The existing barring mechanism of 5G in TR 38.331 specifies to limit the access for specific static user groups as defined in the SIM cards, but not for individual UEs or for dynamic UE groups within a coverage area. So, the information to allow or block PRACH transmissions has to be provided to a UE via alternative mechanisms, e.g. MIB/SIB (which are specified in RRC layer) downlink transmissions defining these UEs, which are not allowed to transmit during a time interval, where inline interference would occur.

Uplink control/user plane transmissions:

The same information about the temporary blocked UEs as defined for the RACH uplink transmissions can be used by the UE to avoid transmissions of PUCCH and PUSCH for the time interval.

As a conclusion, solutions to comply with the ITU regulatory constraints for in-line interference have impact on the RRC layer and are to be studied and specified.

-- END TEXT PROPOSAL --