3GPP TSG-RAN WG2 Meeting #107	R2-1910633

# Prague, Czech Republic, 26th  – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Running CR] Introduction of 5G V2X with NR Sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2019-08-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5G V2X with NR sidelink is introduced in REL-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN2#106 (Endorsed in R2-1908299): - New abbreviations and definitions are added to 3.2. - Uu RRC supports for sidelink are added to 7.1, 7.3 and 7.9. - New section 16.x is added for introduction of sidelink operation. RAN2#107 (Endorsed in R2-19xxxxx): - One missing RAN2#105B agreement is captured in 7.3.1. - Minor editorial changes to 16.x.2.2 and 16.x.2.6 in yellow from R2-1908299. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G V2X with NR Sidelink will not be introduced in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 7.1, 7.3, 7.9, 16.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.300 CR ..., TS 37.340 CR… |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: "NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101: "NR; User Equipment (UE) radio transmission and reception".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[25]	Void.

[26]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[27]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[28]	3GPP TS 24.501: "NR; Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[xx]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services ".

[xy]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MU-MIMO	Multi User MIMO

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NR	NR Radio Access

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SL-SCH	Sidelink Shared Channel

SL-V-RNTI	Sidelink V2X RNTI

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle to everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

Cell-Defining SSB: an SSB with an RMSI associated.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

NR Sidelink Communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [xx], between two or more nearby UEs, using NR technology but not traversing any network node.

V2X Sidelink Communication: AS functionality enabling V2X Communication as defined in TS 23.285 [xy], between nearby UEs, using E-UTRA technology but not traversing any network node.

Editor’s Note: The above definition of V2X Sidelink Communication used for LTE V2X Sidelink is same as the existing definition specified in 36.xxx. FFS need for change to the name/definition of V2X Sidelink Communication and impact on 36.xxx.

Xn: network interface between NG-RAN nodes.

NEXT CHANGE

7	RRC

7.1	Services and Functions

The main services and functions of the RRC sublayer over the Uu interface include:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

-	Addition, modification and release of carrier aggregation;

-	Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR.

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

-	Mobility functions including:

-	Handover and context transfer;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Inter-RAT mobility.

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	Detection of and recovery from radio link failure;

-	NAS message transfer to/from NAS from/to UE.

The sidelink specific services and functions of the RRC sublayer over the Uu interface include:

-	Configuration of sidelink resource allocation via system information or dedicated signalling;

-	Reporting of UE sidelink information;

-	Measurement and reporting related to sidelink:

-	Reporting of UE assistance information for SL traffic pattern(s).

NEXT CHANGE

7.3	System Information Handling

7.3.1	Overview

System Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI:

-	Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

-	MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH.

-	SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

-	Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE or RRC_INACTIVE), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. Other SI consists of:

-	SIB2 contains cell re-selection information, mainly related to the serving cell;

-	SIB3 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB4 contains information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB5 contains information about E-UTRA frequencies and E-UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB6 contains an ETWS primary notification;

-	SIB7 contains an ETWS secondary notification;

-	SIB8 contains a CMAS warning notification;

-	SIB9 contains information related to GPS time and Coordinated Universal Time (UTC).

For sidelink, Other SI also includes:

-	SIBx contains information related to NR Sidelink Communication as specified in TS 38.331 [12];

-	SIBy contains information related to V2X Sidelink Communication as specified in TS 36.331 [29]

-	SIBz contains additional information related to V2X Sidelink Communication as specified in TS 36.331 [29].

Editor’s Note: It is FFS whether separate SIBs are defined to carry SIB21 and SIB26 for V2X Sidelink Communication.

Figure 7.3-1 below summarises System Information provisioning.

Figure 7.3-1: System Information Provisioning

For a cell/frequency that is considered for camping by the UE, the UE is not required to acquire the contents of the minimum SI of that cell/frequency from another cell/frequency layer. This does not preclude the case that the UE applies stored SI from previously visited cell(s).

If the UE cannot determine the full contents of the minimum SI of a cell by receiving from that cell, the UE shall consider that cell as barred.

In case of BA, the UE only acquires SI on the active BWP.

NEXT CHANGE

7.9	UE Assistance Information

When configured to do so, the UE can signal the network through UEAssistanceInformation if it prefers an adjustment in the connected mode DRX cycle length, or if it is experiencing internal overheating. In the latter case, the UE can express a preference for temporarily reducing the number of maximum secondary component carriers, the maximum aggregated bandwidth and the number of maximum MIMO layers. In both cases, it is up to the gNB whether to accommodate the request.

For sidelink, the UE can report SL traffic pattern(s) to NG-RAN, for periodic traffic.

Editor’s Note: FFS for aperiodic traffic in NR sidelink communication. Note that only periodic traffic is supported for V2X sidelink comm under NG-RAN.

NEXT CHANGE

16.x	Sidelink

In this subclause, an overview of sidelink transmission and reception in NR Sidelink Communication and how NG-RAN supports NR Sidelink Communication and V2X Sidelink Communication is given. Sidelink transmission and reception in V2X Sidelink Communication are specified in TS 36.300 [2].

16.x.1	General

The NG-RAN architecture supports the PC5 interface as illustrated in Figure 16.x.1-1. Sidelink transmission and reception over the PC5 interface are supported when the UE is inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when the UE is outside NG-RAN coverage.

Figure 16.x.1-1: NG-RAN Architecture supporting the PC5 interface

Support of V2X services via the PC5 interface can be provided by NR Sidelink Communication and/or V2X Sidelink Communication. NR Sidelink Communication may be used to support other services than V2X services.

NR Sidelink Communication can support three types of transmission modes in the AS:

-	unicast transmission, characterized by:

-	Support of a PC5-RRC connection between peer UEs;

-	Transmission and reception of control information and user traffic between peer UEs in sidelink;

-	Support of sidelink HARQ feedback;

-	Support of RLC AM;

-	Support of sidelink RLM for both peer UEs to detect RLF.

-	groupcast transmission, characterized by:

-	Transmission and reception of user traffic among UEs belonging to a group in sidelink;

-	Support of sidelink HARQ feedback.

-	broadcast transmission, characterized by:

-	Transmission and reception of user traffic among UEs in sidelink;

16.x.2	Radio Protocol Architecture for NR Sidelink Communication

16.x.2.1	Overview

The AS protocol stack for the control plane in the PC5 interface consists of RRC, PDCP, RLC and MAC sublayers, and the physical layer. The protocol stack of PC5-C is shown in Figure 16.x.2.1-1.

Figure 16.x.2.1-1: PC5 control plane (PC5-C) protocol stack.

Editor’s Note: FFS for AS protocol stack for carrying a PC5-S signaling and how to carry PC5-S signaling. The working assumption is that PC5-S message is not encapsulated into PC5-RRC message for direct link setup.

The AS protocol stack for user plane in the PC5 interface consists of SDAP, PDCP, RLC and MAC sublayers, and the physical layer. The protocol stack of PC5-U is shown in Figure 16.x.2.1-2.

Figure 16.x.2.1-2: PC5 user plane (PC5-U) protocol stack.

16.x.2.2	MAC

The MAC sublayer provides the following services and functions over the PC5 interface in addition to the services and functions specified in subclause 6.2.1:

-	Radio resource selection;

-	Packet filtering;

Editor’s Note: FFS for packet filtering in case of groupcast and uncast as in 38.885.

-	Priority handling between uplink and sidelink transmissions of one UE.

With LCP restrictions in MAC, only sidelink logical channels belonging to the same destination can be mutiplexed into a MAC PDU for broadcast and groupcast transmission. NG-RAN can also control whether a sidelink logical channel can utilise the resources allocated by type 1 of a configured sidelink grant (see subclause 10.x).

For packet filtering, a SL-SCH MAC header including a Source Layer-2 ID and a Destination Layer-2 ID is added to each MAC PDU.

Editor’s Note: FFS whether the SL-SCH MAC header includes full IDs or a portion of the IDs based on SCI information.

Editor’s Note: FFS whether different destinations using different Destination Layer 2 IDs can be multiplexed into the same MAC PDU for unicast transmission. FFS whether destination id can distinguish transmission mode.

When sidelink transmission collides with uplink transmission, corresponding QoS requirements of both uplink and sidelink transmissions are used to determine whether to prioritise sidelink transmission.

Editor’s Note: FFS on if and how the QoS requirement of SL and UL transmission can be taken into account in order to prioritaze the SL traffic

The following logical channels are used in sidelink:

-	Sidelink Control Channel (SCCH): a sidelink channel for transmitting control information from one UE to other UE(s).

-	Sidelink Traffic Channel (STCH): a sidelink channel for transmitting user information from one UE to other UE(s).

Editor’s Note: Need for SBCCH is FFS.

The following connections between logical channels and transport channels exist:

-	SCCH can be mapped to SL-SCH;

-	STCH can be mapped to SL-SCH.

16.x.2.3	RLC

Either UM or AM mode is used in unicast transmission while UM mode is used in groupcast or broadcast transmission.

Editor’s Note: Need for SBCCH is FFS.

Editor’s Note: FFS whether RLC over PC5 will support all services and functions specified in 6.3.2.

16.x.2.4	PDCP

PDCP duplication is not supported over the PC5 interface in this version of the specification.

Editor’s Note: FFS whether PDCP over PC5 will support all services and functions specified in 6.4.1.

16.x.2.5	SDAP

The SDAP sublayer provides the following service and function over the PC5 interface:

-	Mapping between a QoS flow and a sidelink radio bearer.

Reflective QoS is not supported over the PC5 interface in this version of the specification.

Editor’s Note: FFS for configuration of a SDAP entity in sidelink.

Editor’s Note: FFS on the need of RX UE awareness of QFI.

16.x.2.6	RRC

The RRC sublayer provides the following services and functions over the PC5 interface:

-	Transfer of a PC5-RRC message between peer UEs;

-	Maintenance of a UE context related to a PC5-RRC connection between two UEs;

Editor’s Note: Need for PC5-RRC state and explicit PC5-RRC connection establishment/release procedure is FFS.

Editor’s Note: It is FFS whether RRC performs detection of readio link failure for a PC5-RRC connection between peer UEs.

The RRC sublayer supports exchange of PC5-RRC messages between peer UEs, i.e. a UE can transfer a PC5-RRC message to the other UE over the PC5 interface. Separate PC5-RRC procedures and messages are used to transfer at least UE capability and AS-layer configuration including SLRB configuration of the other UE. For bi-directional sidelink traffic, both peer UEs exchange their own UE capability and AS-layer configuration using separate bi-directional procedures in both sidelink directions.

Editor’s Note: FFS on whether the two messages can be transmitted together in the same MAC PDU.

A PC5-RRC connection between two UEs is used to establish a sidelink UE AS context of each other. If a UE receives a PC5-RRC message from the other UE, the UE can store at least sidelink related UE capability information from the PC5-RRC message as a sidelink UE context of the other UE. Two UEs use the stored sidelink UE context for an upcoming service under the PC5-RRC connection.

Editor’s Note: FFS whether AS configuration information can be also stored in SL UE context.

Editor’s Note: FFS on whether a one-ot-many PC5-RRC message can be supported for groupcast.

16.x.3	Radio Resource Allocation

The UE can operate in two modes for resource allocation in sidelink:

-	Scheduled resource allocation, characterized by:

-	The UE needs to be RRC_CONNECTED in order to transmit data;

-	NG-RAN schedules transmission resources.

-	UE autonomous resource selection, characterized by:

-	The UE can transmit data when inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when outside NG-RAN coverage;

-	The UE autonomously selects transmission resources from a pool of resources.

For NR Sidelink Communication, the UE performs sidelink transmissions only on a single carrier in this version of the specification.

16.x.3.1	Scheduled resource allocation

NG-RAN can dynamically allocate resources to the UE via the [SL-RNTI] on PDCCH(s) for NR Sidelink Communication.

In addition, NG-RAN can allocate sidelink resources to UE with two types of configured sidelink grants:

- With type 1, RRC directly provides the configured sidelink grant only for NR Sidelink Communication

- With type 2, RRC defines the periodicity of the configured sidelink grant while PDCCH can either signal and activate the configured sidelink grant, or deactivate it. The PDCCH is addressed to [SL-CS-RNTI] for NR Sidelink Communication and SL Semi-Persistent Scheduling V-RNTI for V2X Sidelink Communication.

For the UE performing NR Sidelink Communication, there can be more than one configured sidelink grant active at a time per carrier.

Editor’s Note: FFS for same RNTI will be used for both NR sidelink communication and V2X sidelink communication. FFS how retransmission is scheduled in NR SL Mode 1.

The UE can send sidelink buffer status reports to support Scheduler Operation in NG-RAN. The sidelink buffer status reports refer to the data that is buffered in for a group of logical channels (LCG) per destination in the UE. Eight LGCs are used for reporting of the sidelink buffer status reports. Two formats, which are SL BSR and truncated SL BSR, are used.

Editor’s Note: FFS whether there is no PHR for SL needs to be confirmed by RAN1.

Upon detection of Uu RLF and during connection (re)establishment and handover, the UE temporarily uses an exceptional pool on which UE operates in UE autonomous resource selection.

16.x.3.2	UE autonomous resource selection

The UE autonomously selects sidelink grant from a pool of resources provided by broadcast system information or dedicated signalling while inside NG-RAN coverage or by preconfiguration while outside NG-RAN coverage.

For NR Sidelink Communication, the pools of resources can be provided for a given validity area where the UE does not need to acquire a new pool of resources while moving within the validity area, at least when this pool is provided by SIB (e.g. reuse valid area of NR SIB). NR SIB validity mechanism is reused to enable validity area for SL resource pool configured via broadcasted system information.

Editor’s Note: FFS whether the validity area of the V2X SIB can be different than other NR SIBs.

16.x.4	Uu Control

When a UE is inside NG-RAN coverage, NR Sidelink Communication and/or V2X Sidelink Communication can be configured and controlled by NG-RAN via dedicated signaling or system information:

-	The UE in RRC_CONNECTED can send a Sidelink UE Information to the serving cell if it is interested in sidelink transmission in order to request sidelink resources.

-	The UE in RRC_CONNECTED can send QoS information to the serving cell for a QoS flow. NG-RAN may provide SLRB configurations and QoS mapping information via dedicated signalling for the UE to configure SLRB(s);

Editor’s Note: FFS when the UE reports QoS information for a ‘new’ QoS flow as agreed in RAN2#106. FFS whether QoS information and Sidelink UE Information are sent in the same message or not.

-	The UE in RRC_CONNECTED can send UE assistance information indicating traffic pattern(s) to the serving cell (see subclause 7.9) in order to be configured with one or more configured sidelink grants;

-	The UE in RRC_CONNECTED performs measurement and reporting of CBR and reporting of location information to NG-RAN for sidelink resource allocation.

-	During handover, sidelink transmission and reception are performed based on at least configuration of the exceptional transmission resource pool and reception resource pool of the target cell as provided in the handover command;

-	While in RRC_IDLE or RRC_INACTIVE, NG-RAN may provide SLRB configuration and QoS mapping information to the UE via a SIBx.

-	For cell reselection, the UE interested in V2X service(s) considers at least whether NR Sidelink Communication and/or V2X Sidelink Communication are supported by the cell.

Editor’s Note: FFS whether and how to capture that the supported SL RAT(s) of the UE capability or the selected SL RAT(s) by the upper layer for the V2X service(s) may also need to be considered.

When a UE is outside NG-RAN coverage, SLRB configuration and QoS mapping information are preconfigured to the UE.

END OF THE CHANGE