3GPP TSG-RAN WG2 Meeting # 107	R2-1910685

Prague, Czech Republic, August 26th – 30th, 2019

Title:	Public safety usage scenario for NTN

Source:	FirstNet, AT&T, Thales

Document type: pCR

Document for:	Approval

Agenda Item: 	11.6.1

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This TDOC proposes changes to the TR 38.821.

# Discussion

Undeniably, there are several compelling reasons for first responder to embrace video streaming technology, which represents a major advance from the in-vehicle cameras to that of body worn cameras.  Body worn cameras are valuable tools in that the camera can go where in-vehicle cameras cannot. This technology lets first responders’ capture the live scenes of incidence that otherwise would be unattainable, whether it’s a fire fighter facing a raging wild wire, or a police or ambulance officer attending a domestic violence incident or chasing a suspect on foot or in a vehicle.

A minimum data speed of 3.5 Mbps for DL/UL is required for supporting a standard definition video capture.

# Text Proposal for TR 38.821 v0.7.0

Start of changes

# B.2	Performance targets for evaluation purposes

This table includes performance values that may be used for theoretical analysis or simulations.

The values relate to targeted performances, but should not be considered as strict requirements.

Table B.2-1: Non-Terrestrial network target performances per usage scenarios

| Usage scenarios | Peak data rate (note 1) |  | Experience data rate (note 2) |  | Overall UE density per km2 | Activity factor (note 3) | Max UE speed | Environment | UE categories | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |  |  |  |  |  |  |
| Pedestrian |  |  | 2 Mbps | 60 kbps | TBD | 1,50% | 3 km/h | Extreme coverage | Handheld | Data rate => see 7.10.1 "extreme coverage performance" in [13]; UE density => TBD % of rural macro in [12]; Activity factor: see table 6.1.6-1 "extreme rural" in [13] |
| Vehicular connectivity |  |  | 50 Mbps | 25 Mbps | TBD | N.A. | 250 km/h | Along roads in low population density areas | Vehicular mounted | data rate and activity factor in TS 22.261[12]; UE density => TBD % of High-speed vehicle in [12] |
| Stationary |  |  | 50 Mbps | 25 Mbps | TBD | N.A. | 0 km/h | Extreme coverage | Building mounted | Data rate =>assuming per end-user 50/25 Mbps data rate and an average of 5 end-user devices per stationary UE) rate and 20% activity factor per end-user device |
| Airplanes connectivity |  |  | 360 Mbps | 180 Mbps | TBD | N.A. | 1 000 km/h | Open area | Airplane mounted | Data rate =>assuming per end-user 15/7.5 Mbps data rate and 20% activity factor per end-user devices (See [12]); number of users per airplane: average aircraft size (assuming 120 users per plane) |
| IoT connectivity (note 4) |  |  | 2 kbps | 10 kbps | 400 | 1,00% | 0 km/h | Extreme coverage | IoT | Device density => [15] ; Data rate and activity factor => derived from [14] annex E.2 "Traffic models for Cellular IoT" |
| Public Safety |  |  | 3.5 Mbps | 3.5 Mbps | TBD | N.A | 100 km/h | Open area | Handheld | Section 7.1 of TS 22.280 (To the extent feasible, it is expected that the end user's experience is similar regardless if the MCX Service is used with a 3GPP network or based on the use of a ProSe direct communication path. Covers pedestrian speed through medium vehicular speeds.) |
| Public Safety |  |  | 3.5 Mbps | 3.5 Mbps | TBD | N.A | 250 km/h | Open area | Vehicle mounted | Section 7.1 of TS 22.280 (To the extent feasible, it is expected that the end user's experience is similar regardless if the MCX Service is used with a 3GPP network or based on the use of a ProSe direct communication path.) |

Note 1: As defined in [13]

Note 2: As defined in [12]

Note 3: As defined in [12]

Note 4: This refers to low power wide area service capability

Note 5: This does not preclude the definition of performance parameters for other usage scenarios in future stages

Signalling (control plane) and data (User plane) interruption time should be made as much as possible equivalent as in terrestrial systems except in the case of handover between satellite and terrestrial access. In the latter case, this interruption time will depend on the satellite orbit.

End of changes