3GPP RAN WG2 Meeting #107	R2-1910691

Prague, Czech Republic, 26 – 30 August, 2019

Agenda item:	11.9.3.1

Source:	Apple Inc.

Title:	Enhancements for Conditional Handover in FR2

Document for:	Approval

# Introduction

The NR Mobility Enhancement Work Item [1] was approved in the 3GPP RAN #80 meeting.  An objective of [1] is “to study solution(s) to improve HO/SCG change reliability and robustness especially considering challenges in high/med frequency”. Conditional handover (CHO) has been chosen by RAN2 as a solution to improve reliability and robustness in NR. In this contribution, we further evaluate the performance of CHO when deployed in NR networks operating in FR2 and discuss possibilities for enhancement of CHO.

# Discussion

## Background

The Rel.15 NR work item [2] adopts basic mobility procedures defined for LTE. In the existing NR mobility mechanism, the network controls UE mobility based on UE measurement reports. The serving gNB receives the measurement reports from the UE, and selects a target cell for handover (HO). The serving gNB triggers the handover by sending a HO request to the target cell, and after receiving a HO ACK from the target gNB, the serving gNB initiates the HO by sending a HO command to the UE. The UE performs PRACH to the target cell after RRC is reconfigured with the target cell configuration.

Although the details of CHO in NR are yet to be discussed, it appears from past proposals that in CHO, the serving cell will prepare some of the neighboring cells for HO based on measurement results provided by the UE. Further, the network informs the UE about conditions which can be used by the UE to initiate HO without explicit signaling from the serving cell.

NR is expected to work in the frequency range higher than 24 GHz known as FR2 [3]. UE beams are directional in FR2 and therefore when a UE moves or rotates it may experience rapid signal degradation. Such rapid degradation will result in increased signaling, a high frequency of HO and associated failure rates, and finally, more ping-pong between cells. Therefore, HO is considerably more challenging in NR at FR2 than in LTE.

In RAN2#105bis and RAN2#106, we provided simulation results showing the performance of conditional handover for NR UEs using directional antenna and operating in FR2 [6][9]. We particularly studied the impact of slow rotations of UE on the number of HO attempts, handover failure rate, and number of measurement reports. We showed some advantages and also issues of deploying CHO in NR at FR2.

In RAN2#105bis, some agreements on CHO were made including the followings as captured in meeting report:

Agreements:

0:	CHO is introduced in NR to solve robustness/reliability issue.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc.)

In RAN2#106, some more agreements on CHO were made including the followings as captured in meeting report:

Agreements

1	Deconfiguration of CHO candidates is performed by RRC signaling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates)

2	Baseline that configuration of all CHO candidates are released after successful (any) handover completion (sending complete message to the target cell).

FFS if it might be possible to keep CHO candidates after the HO.

In this paper, we present our views on the FFS items highlighted above and provide some potential solutions enhancing the performance of CHO.

## Simulation Model

In order to study the mobility performance of NR UEs operating at FR2, it is critical to incorporate the impact of antenna radiation patterns in simulations to more accurately model the impact of UE movements on the received/transmitted signal power. In addition to antenna radiation patterns, the number of antenna panels and their locations (front and/or back) also impact mobility performance.

In RAN2#105, we proposed a simulation model to be used when studying NR mobility in FR2 [5] based on studies in RAN1 and RAN4. In this paper, we use the same simulation model from [5] for the dense urban network in Table 5.2.1.2-1 of TR 38.803 [4] (see Figure 1). In this layout, a micro gNB is randomly dropped on an edge of the cluster circle. All UEs associate and communicate with a micro gNB. A hexagonal grid with ISD of 200 m, 19 macro sites, 3 sectors per site model and wrap around is assumed.

Figure 1: Network layout for dense urban from TR 38.803

Figure 2 shows a 2-D view of the UE antenna radiation pattern as introduced in [4] for azimuth in [-180, 180] range. As shown in Figure 2, the antenna radiation pattern is directional with 25 dB signal attenuation at the sides compared to the antenna boresight.

Figure 2: Antenna Radiation Pattern from TR 38.803

A summary of relevant parameters from TR 38.803 [4] is given in Table 1:

Table 1: Summary of Simulation Model for FR2

| Scenario (in TR38.803) | Indoor hotspots | Dense Urban | Urban Macro |
| --- | --- | --- | --- |
| Network Layout Model | Table 5.2.1.3-1 | Table 5.2.1.2-1 | Table 5.2.1.1-1 |
| Propagation Model | Section 5.2.2 |  |  |
| BS Antenna Model | Table 5.2.3.2.3-1 | Table 5.2.3.2.2-1 | Table 5.2.3.2.1-1 |
| UE antenna element pattern | Table 5.2.3.3-1 |  |  |
| UE Elevation distribution | Fixed at 90 |  |  |
| Indoor ratio | 100% | 0%(baseline), 20%, 100% |  |
| Blockage Modeling | No body blockage and hand grip modelling |  |  |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| BS max TX power | 23dBm | 43dBm | 33dBm |
| UE max TX power | 23dBm | 23dBm | 23dBm |
| UE min TX power | -40dBm | -40dBm | -40dBm |
| BS Noise figure | 9dB |  |  |
| UE Noise figure | 9dB |  |  |
| Minimum SINR (for outage) | -8dB |  |  |

### Simulation Results

We studied the L1 RSRP degradation between UE and serving gNB. Figure 3 shows the CDF of RSRP degradation during measurement periods of 100ms, 200ms, and 400ms while UEs are rotating with a frequency of 0.2 Hz. It is observed that for at least 20% of the users the L1 RSRP is reduced by 2 dB after 100ms and 8 dB after 400ms.

We note that RSRP of the target cells are also improved with the same rate for the UEs rotating at a fixed location. It is also worth noting that measurement period for intra frequency measurements without gaps in FR2 is at least 400 ms per TS 38.133 [7].

Figure 3: CDF of RSRP Degradation

Observation 1:  For at least 20% of the users the L1 RSRP is changed by 2 dB after 100ms and 8 dB after 400ms from the L1 measurement time.

We know that in CHO, there is some delay from the time UE identifies a neighbor cell for HO and reports it to gNB till the time gNB prepares the neighbor and provides the UE with condition to perform HO. Based on observation 1, it is understood that degradation of RSRP of serving cell and increment of the RSRP of the target cell can be large during the CHO signaling.

Observation 2: Degradation of RSRP of serving cell and increment of the RSRP of the target cell can be large during the CHO signaling.

As also described in [6], we measured the number of A4 measurement reports sent by each UE within 5 seconds. Figure 4 shows the CDF of the number of A4 reports. We see that 50% of users send at least 7 A4 reports within 5 seconds while a significant fraction of UEs also send tens of reports.

Figure 4: CDF of number of A4 reports

Observation 3: UEs may send a large number of measurement reports due to fast changes in RSRP in FR2 stemming from signal directionality.

In RAN2#106, it was discussed if it is beneficial for the UE to keep CHO configurations after CHO.

We simulated the time of stay (ToS) at each cell for all UEs in the network. Figure 5 shows the CDF of ToS. It is observed that for at least 60% of handovers the ToS is less than 2 seconds. Given the relatively short ToS, it is understood that the network may not have enough time to configure CHO on the UE assuming that 400 ms is needed for L3 filtering and some other latencies are also associated with configuring CHO such as neighbor cell preparation.

Figure 5: CDF of Time of Stay

Observation 4: For at least 60% of the handovers the time of stay is less than 2 seconds.

Based on above observations, it is understood that in order to make CHO more efficient in FR2, it is important to shorten the signaling time and reduce the amount of signaling associated with CHO based mobility.

Proposal 1: RAN2 consider enhancements for CHO to shorten the signaling time and reduce the amount of signaling associated with CHO based mobility.

In the next section, we propose some potential enhancements addressing above proposal.

### CHO Enhancements for FR2

### Beam Level Information

UE making measurements of RS of neighboring cells can provide the serving cell with beam ID of the neighboring cells. This information can be used by the target cell when being prepared for CHO. Some argued that due to latency from the time UE provides the measurement report till it executes the HO in CHO based mobility, the gNB may not know which Rx beam it should use to receive PRACH signal from the UE and thus it may need to reserve resources across many (or all) Rx beams for UE to perform CFRA. However, this concern is unnecessary as explained below.

Considering the practical amount of time between the UE measurement report to HO execution in CHO, UE rotation will be the most likely motion that only makes UE side beam out-of-date. In other words, UE can only move for a very short distance during that time. Therefore, the gNB can still use the beams according to the measurement report and there is no concern about the gNB beam ambiguity, and the need of extra PRACH resource reservation.

Figure 6, shows an example of a rotating UE which has to change its own Rx/Tx beams to communicate with gNB but the Tx/Rx beams at gNB stays the same due to beamforming at UE.

Figure 6: Beam Management for Rotating UE

Observation 5: While the UE may need to change its Rx beam due to rotation, the network can still use the same Tx and Rx beams to communicate with the UE while the UE is able to perform beamforming.

It is therefore understood that the beam ID of neighboring cell reported by UE can be used by the target cell to determine its Rx beam thus reducing the latency of CHO based mobility.

Proposal 2: Provide target cell with RS resource ID of target cell reported by UE.

### RACH-Less HO

To address proposal 1, we think one potential solution shortening signaling time of mobility is RACH-less HO which is already specified for LTE. Moreover, RAN2 has recently received LS from RAN4 implying simultaneous Tx/Rx in FR2 is not feasible [8]. We therefore think implementing CHO along with RACH-less HO can not only improve the performance of CHO but also provides an option to reduce HO interruption time in FR2.

From LTE discussions, we know that RACH-less HO is feasible only if the target and source cells have the same TA or target TA is zero. We however think RACH-less HO can be applied in FR2 cells due to smaller cell sizes. We provided more views on RACH-Less HO in [10].

Proposal 3: RAN2 specify RACH-less HO for NR and consider possibility of joint deployment of CHO and RACH-less HO when RACH-less HO is feasible.

### Support Consecutive CHO

As shown in Figure 5, the time of stay at a cell after CHO maybe very short. In this case, the network may not be able to provide the CHO command timely for the UE. It is therefore beneficial if the UE can reuse previous CHO configurations when applicable.  In this scenario, which we call consecutive HO, we can consider that UE keeps the CHO configuration after HO and thus one CHO command is used for more than one CHO. With this enhancement, we can reduce the HO latency by at least one RTT time in Uu interface for MR transmission and CHO Command transmission. As shown in Figure 7, with the consecutive CHO enhancement, UE can execute the CHO consecutively in cell-2 and cell-3 and doesn’t need to wait for the CHO from network. Another benefit of this enhancement is that the signaling overhead can be reduced. We provided more details on Consecutive CHO in [11].

Figure 7: Consecutive Conditional Handover Procedure

Proposal 4: Consecutive conditional handover should be considered for NR mobility robustness enhancement, i.e. one CHO command can be applied for more than one consecutive CHO.

# Proposals (for approval)

In this paper, we presented simulation results showing the performance of conditional handover for NR UEs using directional antenna and operating in FR2. We particularly studied the rate of RSRP variation of serving and target cells and provided some proposals enhancing the performance of CHO.

Following observations and proposals are provided:

Observation 1:  For at least 20% of the users the L1 RSRP is changed by 2 dB after 100ms and 8 dB after 400ms from the L1 measurement time.

Observation 2: Degradation of RSRP of serving cell and increment of the RSRP of the target cell can be large during the CHO signaling.

Observation 3: UEs may send a large number of measurement reports due to fast changes in RSRP in FR2 stemming from signal directionality.

Observation 4: For at least 60% of the handovers the time of stay is less than 2 seconds.

Observation 5: While the UE may need to change its Rx beam due to rotation, the network can still use the same Tx and Rx beams to communicate with the UE while the UE is able to perform beamforming.

Proposal 1: RAN2 consider enhancements for CHO to shorten the signaling time and reduce the amount of signaling associated with CHO based mobility.

Proposal 2: Provide target cell with RS resource ID of target cell reported by UE.

Proposal 3: RAN2 specify RACH-less HO for NR and consider possibility of joint deployment of CHO and RACH-less HO when RACH-less HO is feasible.

Proposal 4: Consecutive conditional handover should be considered for NR mobility robustness enhancement, i.e. one CHO command can be applied for more than one consecutive CHO.

# References

- RP-181433, “New WID: NR mobility enhancements,” Intel Corporation, 3GPP RAN #80, May 2018

- RP-172834, “New Radio Access Technology,” NTT DOCOMO, 3GPP RAN #78, December 2017

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- TR38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects,” 3GPP, September 2017

- R2-1901546,	“On NR Mobility in FR2”, Apple Inc., 3GPP RAN2 #105, Feb. 2019

- R2-1904980,	“Performance of Conditional HO in FR2”, Apple Inc., 3GPP RAN2 #105bis, April 2019

- TS38.133 v15.5.0, “Requirements for support of radio resource management,” 3GPP, March 2019

- R2-1905530, “Reply LS on NR mobility enhancement”, RAN4, 3GPP RAN2 #106, May 2019

- R2-1907216,	“Enhancements for CHO”, Apple Inc., 3GPP RAN2 #106, May 2019

- R2-1909861, “RACH-less HO”, Apple Inc., 3GPP RAN2 #107, Aug. 2019

- R2-1909862, “Consecutive Conditional HO”, 3GPP RAN2 #107, Aug. 2019