3GPP TSG-RAN WG2 Meeting #107	R2-1910718

Prague, Czech Republic, 26 – 30 August, 2019

Agenda item:	11.12.4

Source:	Nokia, Nokia Shanghai Bell

Title:	RACH report contents for Normal Uplink (NUL) and Supplementary Uplink (SUL)

WID/SID:	NR_SON_MDT-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

One of the use cases in [1] is RACH optimization. An important element in RACH optimization is the RACH report. This paper focuses on the contents of the RACH report for Normal Uplink (NUL) and Supplementary Uplink (SUL) carriers and especially on extracting additional information by reporting also failed RACH information.

# 2	RACH Report for NUL

In [1], it was agreed that RACH report should contain information on the indexes of SSBs and numbers of RACH preambles sent on each tried SSB listed in chronological order of attempts. Listing the number of preambles in chronological order of attempts gives information on the order on which the various RACH transmissions take place. Even though knowing this information is useful, additional information can be extracted if the UE stores in chronological order also preambles for which no RAR is received. This would give the network information on which preambles failed and the times during which failures took place.  For example, in beam-based operation for mobility a target cell can provide dedicated RA resources on multiple beams or SS-blocks for the incoming UE. The UE may send a preamble to several Random Access Resources before receiving a Random Access Response (RAR) from the network. Similarly, several Random Access Resources can be also configured for the UE for Beam Failure Recovery (BFR) Procedure. When the UE receives a RAR, the target cell is only aware on which Random Access Resource the UE made a successful attempt, but it does not know if the UE made other (unsuccessful) attempts. It would be beneficial for the target cell to differentiate among the following cases:

- UE tried RACH on a Random Access Resource in the first attempt and got connected

- UE first tried RACH on some other Random Access Resource(s) and failed before it attempted RACH on a different Resource and successfully completed its RACH procedure

- UE attempted RACH only on a subset of allocated resources and some resources were never used by the incoming UEs for example due to their location or serving beam in the serving cell

Proposal 1: During the Random Access procedure, a UE stores in chronological order a list of preambles for which no RAR is received. For each entry, the UE stores the Random Access Resource, the power used, RA-RNTI, if it was Contention-based Random Access or Contention-Free Random Access.

The RACH report from the UE can be communicated to the network using the UE information procedure, as for regular RACHreport fetch ,or more selectively where the network can request an RRC_Connected UE to send information on the preambles for which no RAR is received.

Proposal 2: The network can receive from the UE the list of preambles for which no RAR is received (e.g., through the UEInformationRequest procedure).

However, in some situations it is possible that the failed RACH was performed in one cell, but the UE performed another RACH and finally connected to a different cell. This means that when the RACH is finally successful the UE is connected to a different cell from the cell its RACH attempts failed. In such cases, the first cell where RACH is attempted cannot know which were the preambles that failed. The UE may store the list of failed preambles in the RACH report and report them to the next cell it is connected to. It would then be the duty of the latter cell to forward the information to the cell where RACH failed. This situation may arise during a Handover in which case the recovery cell would be responsible to forward the list of preambles to the target cell. An example of the above scenario is shown in Figure 1.

Figure 1 Handover example where the handover fails at the target cell, but the UE manages to get synchronized to a recovery cell

Figure 1 describes a scenario where a handover is triggered between the UE and the source cell, but the handover fails at the target cell. The UE finally manages to synchronize with a recovery cell. Since the handover command contains a list of dedicated and common Random Access resources that the UE can use for the target cell access, the target cell could benefit by knowing this information. For example, the Target cell can optimize its decision about the dedicated/common RA resource allocation when responding to the HO requests from serving cells. Based on the serving beam information about the incoming UE, the target cell can avoid allocating resources that are attempted but mostly fail. Target cell may obtain this information through the recovery cell.

Proposal 3: After failed RACH procedure, the UE stores the list of preambles for which no RAR is received and reports it to the next cell it is connected to (recovery cell).

# 3	RACH Report for SUL

SUL is introduced in 5G to extend NUL coverage. Depending on the location of the UE, the latter may perform RACH on NUL or SUL carriers. The parameter rsrp-ThresholdSSB-SUL has been defined to help a UE decide whether it should transmit on NUL or SUL carriers. Specifically, if the RSRP signal of the downlink pathloss reference is smaller than a threshold then the UE should use the SUL carrier otherwise the NUL for its RACH attempt. As in the case of NUL, keeping track of the unsuccessful RACH information could help the network optimize its resources and prevent access issues.   Indeed, if the parameter rsrp-ThresholdSSB-SUL is wrongly configured, this would lead to undesired effects:

- If rsrp-ThresholdSSB-SUL is too low, some UEs would attempt transmission on NUL even when outside of NUL coverage (and hence fail)

- If rsrp-ThresholdSSB-SUL is too high, a UE would attempt transmission on SUL more than needed (even in cases when NUL could be used)

The above is illustrated in Figure 2:

Figure 2 Relationship between NUL and SUL coverages under different rsrp-Threshold-SUL values

Thus, it is beneficial for the network to know when there is a mis-configuration.

If the NUL carrier is selected as downlink pathloss reference exceeds rsrp-ThresholdSSB-SUL, thus the UE had SUL-relevant config but due to settings or network conditions was instructed to select NUL carrier, it could log the details of each unsuccessful attempt, linked to rsrp-ThresholdSSB-SUL:

Proposal 4: If the UE attempts RACH on the NUL carrier based on the rsrp-ThresholdSSB-SUL, the UE stores the details of each unsuccessful attempt such as RACH resource, power, time, DL RSRP, rsrp-ThresholdSSB-SUL.

As discussed also for the case of NUL, failed RACH information on the SUL carrier is also important for the network.

Proposal 5: Once the UE is connected to a cell, the UE reports the stored information about the unsuccessful RACH attempts in NUL and associated parameters to the cell.

One drawback of the proposed solution above is that if the UE fails to access the cell using NUL, the cell would be barred – possibly the whole DL Carrier – and the UE would have to wait and find another cell to inform the network about the wrong configuration. It may be beneficial to find a way for the UE to access the network earlier for example using SUL carrier.

Proposal 6: RAN2 should discuss the possibility for the UE to accelerate RA procedure (e.g. avoiding back off) in case the UE selected NUL carriers, even though received rsrp-ThresholdSSB-SUL.

# 4	Statistical Information on RACH performance on NUL and SUL carriers

Reporting information on the failed RACH preambles can be useful to the network to determine the preamble transmission outcomes and optimize its RACH resources. The network can further use this information to obtain some statistical results on the preambles that were sent by the UEs and their corresponding success outcomes. As an example, the network can use this information to analyse which Random Access Resources are never used by UEs, which Random Access resources are selected by a UE for preamble transmission, but mostly without success, in which order the resources are attempted, etc.

The network can obtain statistical results on the preambles transmitted by the UEs and their corresponding success outcomes.

Usage of Statistical results in some example scenarios

- Handover Example HO

A target cell can optimize its decision about the dedicated/common RA resource allocation when responding to the HO requests from serving cells. Based on the serving beam information about the incoming UE, the target cell can avoid allocating dedicated resources that are never used by UEs coming from the same serving beam or the resources that are attempted but mostly fail. The quality thresholds for using the dedicated RA resources can also be optimized based on the RA attempts information from the UEs.

- Beam Failure Recovery (BFR)

The serving cell can optimize its decision about the dedicated/common RA resource allocation for the BFR operations. Based on the statistics collected, the serving cell can avoid allocating dedicated resources that are never used by UEs or the resources that are attempted but mostly fail. The quality thresholds for using the dedicated RA resources can also be optimized based on the RA attempts information from the UEs.

In addition, the network can use information from the UE or from neighbouring gNBs to reconfigure its parameters and improve the RACH transmission outcomes. For instance, if a gNB receives information from its UEs through RACH reporting or from neighbouring gNBs indicating that a high number of UEs attempted RACH access on NUL and failed and subsequently transmitted successfully on SUL, it may deduce that the parameter rsrp-ThresholdSSB-SUL must be reconfigured.

# 5	Conclusion

Proposal 1: During the Random Access procedure, a UE stores in chronological order a list of preambles for which no RAR is received. For each entry, the UE stores the Random Access Resource, the power used, RA-RNTI, if it was Contention-based Random Access or Contention-Free Random Access.

Proposal 2: The network can receive from the UE the list of preambles for which no RAR is received (e.g., through the UEInformationRequest procedure).

Proposal 3: After failed RACH procedure, the UE stores the list of preambles for which no RAR is received and reports it to the next cell it is connected to (recovery cell).

Proposal 4: If the UE attempts RACH on the NUL carrier based on the rsrp-ThresholdSSB-SUL, the UE stores the details of each unsuccessful attempt such as RACH resource, power, time, DL RSRP, rsrp-ThresholdSSB-SUL.

Proposal 5: Once the UE is connected to a cell, the UE reports the stored information about the unsuccessful RACH attempts in NUL and associated parameters to the cell.

Proposal 6: RAN2 should discuss the possibility for the UE to accelerate RA procedure (e.g. avoiding back off) in case the UE selected NUL carriers, even though received rsrp-ThresholdSSB-SUL.

# 6	References

[1] 3GPP TR 37.816v1.0.0, “Study on RAN-centric data collection and utilization for LTE and NR”