3GPP TSG-RAN WG2 #107	 TDoc R2-1910786

# Prague, Czech Republic, 26th – 30th August 2019             	      Revision of R2-1907596

Agenda Item:	11.2.1.1

Source:	Ericsson

Title:	Two step Random Access for NR-U

Document for:	Discussion, Decision

# Introduction

In NR-U, a listen before talk (LBT) operation may need be performed prior to any transmission. For the legacy four step random access this will result in four LBT occasions. As a result, RAN2 has concluded in the TR from the SI phase of NR-U [1] that the 2-step random access procedure is supported in NR-U.  In this paper, we discuss some potential issues with the 2-step random access procedure and propose corresponding solutions.

# Discussion

In the 2-step random access (RA) procedure, the UL messages (Msg1 and Msg3 in legacy 4-step procedure) are combined into MsgA and sent simultaneously (or consecutive in time without the need for a LBT between) and similarly the two DL messages (Msg2 and Msg4, combined to MsgB) are sent as a simultaneous response in the DL. The procedure is illustrated from a high level in Figure 1.

Figure 1 – Illustration of two-step procedure.

In the legacy four step procedure, one main usage of the first two messages is to obtain UL time alignment for the UE. In many situations, e.g. in small cells, for stationary UEs or UEs in connected mode, this may not be needed since either a TA equal to 0 will be sufficient (small cells) or a stored TA value could serve also for the current RA. In future radio networks and in NR-U deployments it can be expected that these situations are common, both due to dense deployments of small cells and a great number of e.g. stationary IoT devices. A possibility to skip the message exchange to obtain the TA value would lead to reduced RA latency and would be beneficial in several use cases, for example when transmitting infrequent small data packets.

In small cells or for stationary UEs, Random Access may not be needed to obtain UL time alignment.

Since every LBT faces the possibility to fail, the number of LBTs needed to perform a random access procedure is important for NR-U. A reduction of the needed number of LBTs reduces the risk of the UE or gNB not being able to transmit and thereby increasing the latency compared to the licenced case. The reduced number of needed LBTs needed for the 2-step procedure compared to the 4-step procedure is an important benefit for the 2-step procedure in NR-U.

For NR-U, the reduced number of LBTs needed for a 2-step Random access procedure compared to the legacy 4-step procedure, is an important benefit.

## The 2-step Random Access procedure

The details of the design have not yet been decided in RAN1 but may have great consequences for NR-U. One example is that for MsgA, where it has been agreed in the 2-step WID [2] that the preamble part and the Msg3 part are TDMed. How large a time difference that can be allowed between the preamble and the Msg3 part may have impacts on the number of LBTs that have to be performed. For NR-U, one of the main benefits of the 2-step RA is the reduced need of LBTs compared to the 4-step RA procedure. In case MsgA would need two LBTs much of the benefits of the 2-step RA would disappear. Therefore, it is of vital importance that MsgA can be transmitted using only one LBT, i.e. that both the preamble part and Msg3 part can be transmitted within the MCOT. We therefore propose

For NR-U, it should be possible to send the preamble part and PUSCH part of MsgA using only one LBT.

For NR-U it has also been observed that there are benefits of allowing a longer RAR window for the 4-step procedure than in the licenced case. It has been assumed by RAN2 in RAN2#105bis that the maximum RAR window is 20ms. This has implications on the RA-RNTI since the current RA-RNTI is ambiguous if applied to RAR windows exceeding 10ms. In previous meeting solutions that utilize unused RA-RNTI values for NR-U have been discussed. This depends on the fact that all SCS for PRACH in NR-U are not allowed in Rel. 16. However, relying on this will make this option sensitive for changes of allowed SCS in future releases and we do not think solutions along these lines are preferable.

Allowing longer RAR window in NR-U for 4-step should be done without changing the RA-RNTI.

Also for the 2-step Random Access procedure, the need for longer msgB window (i.e. the correspondence to RAR window) has been identified and the problem of how to enable this has been discussed. One of the main tracks is to reuse the RA-RNTI also for msgB. To simplify both specifications, implementations and future enhancements it would be advantageous if the RA-RNTI remains the same for NR-U and 2-step RA.

It may be beneficial for both NR-U and the 2-step procedure if the RA-RNTI remains the same as for NR.

Since the design of to allow longer RAR windows for the 4-step procedure may impact the 2-step procedure we believe that these issues need to be studied together, i.e. the impact of a solution for NR-U needs also consider the consequences on the 2-step procedure. In our accompanying paper [5] we describe how a new RAR header can be used to incorporate also SFN information and thereby utilizing SFN information together with the RA-RNTI to enable longer RAR windows. Hence we give the following proposal:

Enabling longer RAR windows for NR-U needs to consider consequences also for the 2-step procedure.

# Conclusion

In section 2 we made the following observations:

Observation 1	In small cells or for stationary UEs, Random Access may not be needed to obtain UL time alignment.

Observation 2	For NR-U, the reduced number of LBTs needed for a 2-step Random access procedure compared to the legacy 4-step procedure, is an important benefit.

Observation 3	It may be beneficial for both NR-U and the 2-step procedure if the RA-RNTI remains the same as for NR.

Based on the discussion in section 2 we propose the following:

Proposal 1	For NR-U, it should be possible to send the preamble part and PUSCH part of MsgA using only one LBT.

Proposal 2	Allowing longer RAR window in NR-U for 4-step should be done without changing the RA-RNTI.

Proposal 3	Enabling longer RAR windows for NR-U needs to consider consequences also for the 2-step procedure.

# References

- 3GPP TR 38.889 v16.0.0 “Study on NR-based access to unlicensed spectrum (Release 16)

- RP-182894, New work item: 2-step RACH for NR, ZTE Corporation, Sanechips

- R2-1906954, “Fallback for 2-step RA”, Ericsson, RAN2 #106, Reno, USA, May 2019.

- R2-1906953, “RNTIs and Contention Resolution for 2-step RA”, Ericsson, RAN2#106, Reno, USA, May 2019.

- R2-1910796, “New RAR formats for NR-U”, Ericsson RAN2#107 Prague, August 2019