3GPP TSG-RAN WG2 #107	Tdoc R2-1910793

Prague, Czech Republic, 26 – 30 Aug 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | XXXX | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38300 Overall description of UL LBT failure handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#105 bis, RAN2 has made below agreements regarding UL LBT failure handling. Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection Necessary changes to the Stage 2 spec are required to adopt the mechanism for handling consistent UL LBT failures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes are added in respect of LBT failure detection and recovery actions. For LBT failure detection, a counter is maintained across all LBT subbands within the same BWP. The counter is maintained per BWP. For recovery action, if the UE experiences UL LBT problem in its current active BWP, the UE may switch to Aother BWP. If the UE has detected LBT problems for all configured BWPs, the UE may declare an RLF for the cell and trigger RRC connection reestablishment. For a UE configured with CA, if the UE has detected UL LBT problem in one carrier, the UE may inform the gNB which may take appropriate actions. For a UE is configured with DC, the UE reports SCG LBT problem if the UE has experienced UL LBT failures consecutively up to a maximum number in SCG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviors on consistent UL LBT failure handling are not predictable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< First change start>

# 9	Mobility and State Transitions

<non modified parts are omitted>

## 9.2	Intra-NR

<non modified parts are omitted>

### 9.2.X	UL LBT failure detection and recovery

In RRC_CONNECTED, the UE monitors UL LBT failures in the active BWP for all UL transmissions including any transmission carried on RACH, SRS, PUCCH and PUSCH. If the active BWP comprises several LBT subbands, the UE maintains a common counter across LBT subbands within the same BWP. The UE may be configured with several BWPs, UL LBT failure handling should be operated per BWP.

The UE declares UL LBT problem when below criterion is met:

-	A maximum number of consecutive UL LBT failures has been reached

If the UE experiences UL LBT problem in its current active BWP, the UE initiate a Random Access procedure on an inactive BWP and, if successful, indicate LBT problem. If there are no PRACH occasions configured on any inactive BWP, the UE can switch to the initial BWP to start a RACH procedure. Upon reception of the Random Access procedure, the gNB can decide if the UE needs to switch to another BWP. The gNB can reply with a DCI or an RRC reconfiguration indicating the new BWP which may be a different one from which the UE has transmitted the Random Access procedure in.

If the UE has detected LBT problems for all configured BWPs, the UE may declare RLF.

For a UE configured with Carrier Aggregation, if the UE has detected UL LBT problem in one carrier, the UE may inform the gNB which may take appropriate actions, for example, to inactivate or de-configure the cell where the UL LBT problem has been detected.

For a UE configured with Dual Connectivity, the UE reports SCG LBT problem to MCG, if the UE has experienced UL LBT failures consecutively up to a maximum number in SCG.

< First change end>