3GPP TSG-RAN WG2 #107	R2-1910858

Prague, Czech republic, 26th August – 30th August 2019

Agenda Item:	11.12.3

Source:	Ericsson

Title:	Text proposal to 38.314 on number of stored Inactive UE context count in NR

Document for:	Discussion, Decision

# Text Proposal to 38.314

### X.Y.Z	 Number of RRC_CONNECTED UEs per 5QI class

### X.Y.Z.1	 Mean number of RRC_CONNECTED UEs per 5QI class

The objective of the measurement is to measure mean number of RRC_CONNECTED UEs per 5QI class for OAM performance observability.

The measurement is obtained by sampling at a pre-defined interval, the number of RRC_CONNECTED UEs per 5QI class for each NR cell and then taking the arithmetic mean over pre-defined time duration.

| Definition | Number of RRC_CONNECTED UEs per 5QI class. Detailed Definition: M(T,p,5qi)=∀iN(i,5qi)I(T,p),where explanations can be found in the table X.Y.Z.1-1 below. |
| --- | --- |

Table X.Y.Z.1-1

| M(T,p,5qi) | Mean number of RRC_CONNECTED UEs per 5QI class, averaged during time period . Unit: Integer. |
| --- | --- |
| N(i,5qi) | Number of RRC_CONNECTED UEs per 5QI class in the cell at sampling occasion. |
|  | Sampling occasion during time period. A sampling occasion shall occur once every seconds. |
|  | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
|  | Total number of sampling occasions during time period . |
|  | Time Period during which the measurement is performed, Unit: second. |

### X.Y.Z.2	 Max number of RRC_CONNECTED UEs per 5QI class

The objective of the measurement is to measure maximum number of RRC_CONNECTED UEs per 5QI class for OAM performance observability.

The measurement is obtained by sampling at a pre-defined interval, the number of RRC_CONNECTED UEs per 5QI class for each NR cell and then taking the maximum value over pre-defined time duration.

| Definition | Maximum number of RRC_CONNECTED UEs per 5QI class. Detailed Definition: MT,p,5qi= maxTNi,5qi,where explanations can be found in the table X.Y.Z.2-1 below. |
| --- | --- |

Table X.Y.Z.2-1

| M(T,p,5qi) | Maximum number of RRC_CONNECTED UEs per 5QI class sampled during time period . Unit: Integer. |
| --- | --- |
| N(i,5qi) | Number of RRC_CONNECTED UEs per 5QI class stored in the cell at sampling occasion. |
|  | Sampling occasion during time period. A sampling occasion shall occur once every seconds. |
|  | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
|  | Time Period during which the measurement is performed, Unit: second. |