3GPP TSG-RAN WG2 #107	R2-1910860

Prague, Czech Republic, 26th August – 30th August 2019

Agenda Item:	11.12.3

Source:	Ericsson

Title:	On random access preamble inclusion as L2 measurement

Document for:	Discussion, Decision

# Introduction

As part of the TR [1],  it has been agreed to include received random access preambles as part of the L2 measurements.

The following measurements are to be introduced to NR:

- Received Random Access Preambles

- Number of users for RRC_CONNECTED

- Number of users for RRC_INACTIVE

In this contribution, we discuss how this parameter can be captured in the NR L2 measurement specification.

# Discussion

In LTE, the random access preambles are already included as part of the L2 measurements [2].

### 4.1.2	Received Random Access Preambles

A use case for this measurement is RACH configuration optimization, where Received Random Access Preambles is signaled across an OAM interface.

Protocol Layer: MAC

| Definition | Received Random Access Preambles. This measurement is applicable to PRACH. The reference point is the Service Access Point between MAC and L1. The measured quantity is the number of received Random Access preambles during a time period over all PRACHs configured in a cell. The measurement is done separately for: - Dedicated preambles - Randomly selected preambles in the low range - Randomly selected preambles in the high range. The unit of the measured value is [/s]. |
| --- | --- |

As the measurements captured in LTE are still applicable in NR, we propose to have the LTE’s way of capturing the received random access preambles as the baseline.

LTE’s definition of received random access preambles is the baseline for NR L2 measurements.

As NR RACH can be performed at the beam level, it is desirable to provide the preamble information per SSB.

The preamble information included in the L2 information is per SSB.

RAN2 is requested to agree the TP [3] to 38.314.

# Conclusion

In this contribution, the following proposals were captured:

Proposal 1	LTE’s definition of received random access preambles is the baseline for NR L2 measurements.

Proposal 2	The preamble information included in the L2 information is per SSB.

Proposal 3	RAN2 is requested to agree the draft CR [3] to 38.314.

# References

37.816, TR Study on RAN centric data collection and utilization for LTE and NR, v1.0.0

36.314, 3gpp TSG RAN EUTRA; Layer 2 Measurements; Release 15, V15.2.0

R2-1910861, TP to 38.314 on random access preambles, Ericsson, RAN2#107 meeting, Prague, August 2019.