3GPP TSG-RAN WG2 #107	R2-1910861

Prague, Czech republic, 26th August – 30th August 2019

Agenda Item:	11.12.3

Source:	Ericsson

Title:	Text proposal to 38.314 on received random access preambles

Document for:	Discussion, Decision

# Text Proposal to 38.314

### X.Y.Z	 Received Random Access Preambles

A use case for this measurement is RACH configuration optimization, where Received Random Access Preambles is signaled across an OAM interface.

Protocol Layer: MAC

| Definition | Received Random Access Preambles per SSB. This measurement is applicable to PRACH. The reference point is the Service Access Point between MAC and L1. The measured quantity is the number of received Random Access preambles during a time period over all PRACHs configured in a cell. The measurement is done separately for: - Dedicated preambles - Randomly selected preambles in the low range - Randomly selected preambles in the high range. The unit of the measured value is [/s]. |
| --- | --- |