3GPP TSG-RAN2 107                                            R2-1911319

Prague, Czech Republic, 26th Aug – 30th Aug 2019

Agenda item:	10.4.1.3.1

Source:	Samsung

Title:	Discussion on maxMIMO-Layers for the normal uplink and the supplementary uplink

Document for:	Discussion & Decision

# Introduction

In the current RRC specification, "maxMIMO-Layers" indicates the maximum MIMO layer to be used for PUSCH in all BWPs of this serving cell. One problem is maxMIMO-Layers can be signalled in two different places in a RRC message; one for normal uplink and the other for SUL. This contribution discusses this aspect.

# Discussion

maxMIMO-Layers is an optional IE with Need code M. So following four cases could be possible.

|  | maxMIMO-Layers in uplinkConfig | maxMIMO-Layers in supplementaryUplink | Number of layers for a serving cell |
| --- | --- | --- | --- |
| CASE 1 | Y | Y | ? |
| CASE 2 | Y | N | According to maxMIMO-Layers |
| CASE 3 | N | Y | According to maxMIMO-Layers |
| CASE 4 | N | N | According to the reported UE capability in FeatureSetUplinkPerCC as specified in 38.212 5.4.2.1 |

The question is whether we should clarify CASE 1 in the specification. Maybe one can try to configure different value for the normal uplink and for the supplementary uplink, which can be considered valid motivation (since UL and SUL can be in the different FRs). However this operation seems not supported in Release 15. There seem two options i.e.

-	Option 1: Update the operation such that maxMIMO-Layers is applied for all BWPs of the same uplink. This is functional change and may not be possible (can be considered for Release 16)

-	Option 2: Clarify that maxMIMO-Layers should have the same value between the uplink and the supplementary uplink.

Proposal: RAN2 is asked to discuss which option to take in Release 15.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal: RAN2 is asked to discuss which option to go for in Release 15.

RAN2 is also requested to see the corresponding CRs [1], [2].

# References

[1] R2-1911333, Correction on MIMO configuration for uplink_Option 1, Samsung

[2] R2-1911342, Correction on MIMO configuration for uplink_Option 2, Samsung