3GPP TSG-RAN2 107                                            R2-1911320

Prague, Czech Republic, 26th Aug – 30th Aug 2019

Agenda item:	12.6

Source:	Samsung

Title:	Discussion on unnecessary measurement reporting procedure for H1 and H2 events

Document for:	Discussion & Decision

# Introduction

At RAN2 #105 meeting, height report triggering based on new H1 and H2 events was discussed and the following agreement was made:

R2-1902185	Correction on H1 and H2 events	Samsung Electronics	CR	Rel-15	36.331	15.4.0	3930	-	F	LTE_Aerial-Core

=>	Left this issue to network implementation in this release and can be reconsidered in next release.

=>	Not purposed

This contribution discusses the corresponding specification fix for avoiding continuous measurement reporting for H1 and H2 events in Rel-16.

# Discussion

TS 36.331 specifies entering/leaving conditions for H1 and H2 events, as specified in 5.5.4.16 and 5.5.4.17 (only relevant parts are copied below):

Event H1 (The Aerial UE height is above a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H1-2, as specified below, is fulfilled;

Event H2 (The Aerial UE height is below a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition H2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition H2-2, as specified below, is fulfilled;

As highlighted above, it is specified such that entering/leaving condition is fulfilled, then it stays as fulfilled until leaving/entering is fulfilled for H1 and H2 events as like any other events. Then it leads to initiate the measurement reporting procedure continuously when it combines with the general section as below (only relevant parts are copied as below).

5.5.4.1 General

If security has been activated succesfully, the UE shall

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the triggerType is set to event and if the eventId is set to eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined within the VarMeasConfig for this event:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

In our understanding, this kind of continuous triggering does not happen for other events due to additional checking of cellsTriggeredList. But for H1/H2 events, the triggering is not about cells but about UE height which makes applying cellsTriggeredList impossible.

At RAN2 #105 meeting, RAN2 has agreed that this issue is left to network implementation without any specification change. In our view, this is not the case i.e. the UE will anyhow trigger the measurement reporting procedure continuously.

Observation: For H1/H2 events, the UE may trigger the measurement reporting procedure continuously since the triggering is not about cells but about UE height.

One possible approach to fix without introducing a new field is as follows:

- 	Upon meeting initial entering condition for H1/H2 events, the UE adds the measID to VarMeasReportList. The UE does not trigger the measurement reporting procedure subsequently if the measID is included in VarMeasReportList.

- 	Upon meeting leaving condition for H1/H2 events, the UE remove the concerned measID in VarMeasReportList.

Proposal: Update the UE behavior on the procedure of measurement reporting procedure for H1/H2 events according to the CR [1].

# Conclusion

In section 2 we made the following observation:

Observation: For H1/H2 events, the UE may trigger the measurement reporting procedure continuously since the triggering is not about cells but about UE height.

Based on the above, RAN2 is requested to discuss and agree on the following proposal:

Proposal: Update the UE behavior on the procedure of measurement reporting procedure for H1/H2 events according to the CR [1].

# References

[1] R2-1911345, Correction on H1 and H2 events, Samsung