3GPP TSG-RAN WG2 Meeting #107	R2-1911556

# Prague, Czech Republic, 26 – 30 Aug, 2019	was R3-194566

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1250 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location Reporting: effective PSCell ID reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Vodafone, Nokia) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDCE5, LI15, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TSG SA #84 approved CRs S2-1904832, S2-1904834 and S2-1906165 on reporting of Primary cell of the Secondary node to the core network. When approving the CRs, TSG SA sent an LS (SP-190580) to SA2 which indicated that TSG SA seemed to erroneously believe that periodic TA updates are performed when the UE is in RRC connected state. Separately, the ETSI TC LI LS in S2-1906999 explores the linkage to CDRs and Retained Data. To solve the ‘timeliness’ problem identified by TSG SA, and the regional regulatory requirements from ETSI TC LI, SA2 #134 (June 2019) has agreed CR 3531r1 to TS 23.401 in S2-1908071. TSG SA also raised concerns about the signalling load coming from the LI requirements. As a consequence, SA2 #134 agreed CR 3529r2 in S2-1908138. This CR aligns TS 36.300 with these two SA2 CRs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that S1AP procedures “UE Context Suspend” and “Secondary RAT Data Usage Report” are able to provide location information without the reporting being triggered by the Location Reporting Control procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 specification is not aligned with Stage-3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.2.2.11.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.413 CR 1708 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-194566. |  |  |  |  |  |  |  |  |

************ start of  changes ********************

#### 19.2.2.11	Location Reporting procedures

##### 19.2.2.11.0	General

The Location Reporting procedures provide the means to report the current location of a specific UE.

The procedures providing this function are:

-	Location Reporting Control procedure;

-	Location Report procedure;

-	Location Report Failure Indication procedure.

If DC is configured for a specific UE, the location reported refers to the cell served by the MeNB. If EN-DC is configured for a specific UE, the location reported refers to the cell served by the MeNB and, if requested, the PSCell at the en-gNB.

NOTE:	The following S1AP procedures are able to provide location information without the reporting being triggered by the Location Reporting Control procedure: 
S1 UE Context Release, UE Context Suspend, E-RAB Release, E-RAB Release Indication, Path Switch, Handover Notification, Initial UE Message, Uplink NAS Transport, Secondary RAT Data Usage Report.

**** end of changes *********