3GPP TSG-RAN WG2 Meeting #107	 R2-1911696

# Prague, Czech Republic, 26th - 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1141 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on usage of access category 2 for UAC for RNA update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 08/26/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | ​In current 38.331 (section 5.3.13.8), ​barring alleviation of access category 8 triggers UE to initiate pending RNA update. However, according to 38.331 section 5.3.13.2, access category 2 can also be used for UAC for RNA update. 1> else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8: 2> if an emergency service is ongoing: NOTE: How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation. 3> select '2' as the Access Category; 3> set the resumeCause to emergency; 2> else: 3> select '8' as the Access Category; 2> perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23]; 3> if the access attempt is barred: 4> set the variable pendingRnaUpdate to true; 4> the procedure ends; Therefore, the barring alleviation of access category 2 should also be one of the trigger condition to check whether UE have pending RNAU or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify in 5.3.13.8 that the UE will check whether it has to initialize pending RNAU procedure also after AC2 is alleviated. Add the Annex for CRs that could be implemented in earlier release. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: RNA update procedure Inter-operability: The CR only impact UE behavior and the network does not have to implement it. So, there is no inter-operability issue. Implementation of this CR from Rel-15 will not cause interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not re-initialize the pending RNAU if the original RNAU procedure is barred due to access attempt failure of access category 2. Then, the network may consider the UE leaving the INACTIVE state. As a consequence, the connection resume procedure (e.g. for UL data transmission) will be failed and result in some performance lost. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.8, Annex X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 5.3.13.8	RNA update

In RRC_INACTIVE state, the UE shall:

1>	if T380 expires; or

1>	if RNA Update is triggered at reception of SIB1, as specified in 5.2.2.4.2:

2>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause set to rna-Update;

1>	if barring is alleviated for Access Category '8' or Access Category '2', as specified in 5.3.14.4:

2>	if upper layers do not request RRC the resumption of an RRC connection, and

2>	if the variable pendingRnaUpdate is set to true:

3>	initiate RRC connection resume procedure in 5.3.13.2 with resumeCause value set to rna-Update.

If the UE in RRC_INACTIVE state fails to find a suitable cell and camps on the acceptable cell to obtain limited service as defined in TS 38.304 [20], the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other'.

NOTE:	It is left to UE implementation how to behave when T380 expires while the UE is camped neither on a suitable nor on an acceptable cell.

2nd Change

Annex X (normative): List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table X-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-xxxxxx: Correction on usage of access category 2 for UAC for RNA update | 1141 | - | Release 15 |  |