3GPP TSG-RAN WG2 Meeting #107	R2-1911851

# Prague,  Czech Republic, 26 – 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1174 | rev | 2 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifying handling of information elements on the F1 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 38.331, the following has been stated for the handling of inter-node messages: “This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG”. However, some of the fields in these messages are described in a manner that seem to “artifically” restrict the content only between a source and target (eNB or gNB) even though in principle the same information element could be exchanged between the CU and DU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For section 11.2.2, for the following cases it is clarified that the target could also be a DU not just limited to the gNB: - as-Context IE within the HandoverPreparationInformation field. - candidateCellInfoListMN, candidateCellInfoListSN IE within the CG-ConfigInfo field. In addition, a typo in requestedP-MaxEUTRA field description is corrected. Impact analysis Impacted functionality: Inter-node messages. Impacted 5G architecture options: NR SA, NG(EN)-DC and NR-DC. Inter-operability: There is no interoperability issue as the clarification proposed by this CR will align the specification with the already common understanding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may be misinterpreted that some of the IEs in the CG-ConfigInfo and the HandoverPreparationInformation fields are only applicable to be sent on X2/Xn interfaces, which is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### –	HandoverPreparationInformation

This message is used to transfer the NR RRC information used by the target gNB during handover preparation, including UE capability information. This message is also used for transferring the information between the CU and DU.

Direction: source gNB/source RAN to target gNB or CU to DU.

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=      SEQUENCE {

criticalExtensions                      CHOICE {

c1                                      CHOICE{

handoverPreparationInformation          HandoverPreparationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

HandoverPreparationInformation-IEs ::=  SEQUENCE {

ue-CapabilityRAT-List                   UE-CapabilityRAT-ContainerList,

sourceConfig                            AS-Config                                       OPTIONAL, -- Cond HO

rrm-Config                              RRM-Config                                      OPTIONAL,

as-Context                              AS-Context                                      OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                     OPTIONAL

}

AS-Config ::=                           SEQUENCE {

rrcReconfiguration                      OCTET STRING (CONTAINING RRCReconfiguration),

...,

[[

sourceRB-SN-Config                      OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

sourceSCG-NR-Config                     OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

sourceSCG-EUTRA-Config                  OCTET STRING                                    OPTIONAL

]]

}

AS-Context ::=                          SEQUENCE {

reestablishmentInfo                     ReestablishmentInfo                             OPTIONAL,

configRestrictInfo                      ConfigRestrictInfoSCG                           OPTIONAL,

...,

[[  ran-NotificationAreaInfo            RAN-NotificationAreaInfo                        OPTIONAL

]],

[[  ueAssistanceInformation             OCTET STRING (CONTAINING UEAssistanceInformation)  OPTIONAL   -- Cond HO2

]],

[[

selectedBandCombinationSN               BandCombinationInfoSN                           OPTIONAL

]]

}

ReestablishmentInfo ::=             SEQUENCE {

sourcePhysCellId                        PhysCellId,

targetCellShortMAC-I                    ShortMAC-I,

additionalReestabInfoList               ReestabNCellInfoList                            OPTIONAL

}

ReestabNCellInfoList ::=        SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo

ReestabNCellInfo::= SEQUENCE{

cellIdentity                            CellIdentity,

key-gNodeB-Star                         BIT STRING (SIZE (256)),

shortMAC-I                              ShortMAC-I

}

RRM-Config ::=              SEQUENCE {

ue-InactiveTime             ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}                            OPTIONAL,

candidateCellInfoList       MeasResultList2NR                                           OPTIONAL,

...,

[[

candidateCellInfoListSN-EUTRA      MeasResultServFreqListEUTRA-SCG                      OPTIONAL

]]

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Context Local RAN context required by the target gNB or DU.

rrm-Config Local RAN context used mainly for RRM purposes.

sourceConfig The radio resource configuration as used in the source cell.

ue-CapabilityRAT-List The UE radio access related capabilities concerning RATs supported by the UE.

ue-InactiveTime Duration while UE has not received or transmitted any user data. Thus the timer is still running in case e.g., UE measures the neighbour cells for the HO purpose. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. Value min1 corresponds to 1 minute, value min1s20 corresponds to 1 minute and 20 seconds, value min1s40 corresponds to 1 minute and 40 seconds and so on. Value hr1 corresponds to 1 hour, hr1min30 corresponds to 1 hour and 30 minutes and so on.

AS-Config field descriptions

rrcReconfiguration Contains the RRCReconfiguration configuration as generated entirely by the MN.

sourceRB-SN-Config Contains the IE RadioBearerConfig as generated entirely by the SN. This field is only used in NE-DC and NR-DC.

sourceSCG-EUTRA-Config I Contains the current dedicated SCG configuration in RRCConnectionReconfiguration message as specified in TS 36.331 [10] and generated entirely by the SN. In this version of the specification, the E-UTRA RRCConnectionReconfiguration message can only include the field scg-Configuration . This field is only used in NE-DC.

sourceSCG-NR-Config Contains the current dedicated SCG configuration in RRCReconfiguration message as generated entirely by the SN. In this version of the specification, the RRCReconfiguration message can only include fields CellGroupConfig and measConfig. This field is only used in NR-DC.

AS-Context field descriptions

selectedBandCombinationSN Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC.

RRM-Config field descriptions

candidateCellInfoList A list of the best cells on each frequency for which measurement information was available

candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. A list of EUTRA cells including serving cells and best neighbour cells on each serving frequency, for which measurement results were available. This field is only used in NE-DC.

| Conditional Presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within NR; The field is optionally present in case of handover from E-UTRA connected to 5GC; otherwise the field is absent. |
| HO2 | The field is optionally present in case of handover within NR; otherwise the field is absent. |

NOTE 1:	The following table indicates per source RAT whether RAT capabilities are included or not.

| Source RAT | NR capabilites | E-UTRA capabilities | MR-DC capabilities |
| --- | --- | --- | --- |
| NR | Included | May be included | May be included |
| E-UTRAN | Included | May be included | May be included |

NOTE 2:	The following table indicates, in case of inter-RAT handover from E-UTRA, which additional IEs are included or not:

| Source system | sourceConfig | rrm-Config | as-Context |
| --- | --- | --- | --- |
| E-UTRA/EPC | Not included | May be included | Not included |
| E-UTRA/5GC | May be included, but only radioBearerConfig is included in the RRCReconfiguration. | May be included | Not included |

Next Modified Subclause

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB.

Direction: Secondary gNB or eNB to master gNB or eNB.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=                   SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-Config                           CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-Config-IEs ::=                   SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

CG-Config-v1540-IEs ::=             SEQUENCE {

pSCellFrequency                     ARFCN-ValueNR                                   OPTIONAL,

reportCGI-RequestNR                 SEQUENCE {

requestedCellInfo                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR,

cellForWhichToReportCGI             PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

ph-InfoSCG                          PH-TypeListSCG                                  OPTIONAL,

nonCriticalExtension                CG-Config-v1560-IEs                             OPTIONAL

}

CG-Config-v1560-IEs ::=             SEQUENCE {

pSCellFrequencyEUTRA                ARFCN-ValueEUTRA                                OPTIONAL,

scg-CellGroupConfigEUTRA            OCTET STRING                                    OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                    OPTIONAL,

candidateServingFreqListEUTRA       CandidateServingFreqListEUTRA                   OPTIONAL,

needForGaps                         ENUMERATED {true}                               OPTIONAL,

drx-ConfigSCG                       DRX-Config                                      OPTIONAL,

reportCGI-RequestEUTRA              SEQUENCE {

requestedCellInfoEUTRA          SEQUENCE {

eutraFrequency                             ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA              EUTRA-PhysCellId

}                                                                               OPTIONAL

}                                                                                   OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

PH-TypeListSCG ::=                  SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoSCG

PH-InfoSCG ::=                      SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierSCG,

ph-SupplementaryUplink              PH-UplinkCarrierSCG                             OPTIONAL,

...

}

PH-UplinkCarrierSCG ::=             SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

MeasConfigSN ::=                    SEQUENCE {

measuredFrequenciesSN               SEQUENCE (SIZE (1..maxMeasFreqsSN)) OF NR-FreqInfo  OPTIONAL,

...

}

NR-FreqInfo ::=                     SEQUENCE {

measuredFrequency                   ARFCN-ValueNR                                       OPTIONAL,

...

}

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedBC-MRDC                    BandCombinationInfoSN                               OPTIONAL,

requestedP-MaxFR1                   P-Max                                               OPTIONAL,

...,

[[

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                     OPTIONAL,

requestedP-MaxEUTRA                 P-Max                                               OPTIONAL

]]

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

BandCombinationInfoSN ::=           SEQUENCE {

bandCombinationIndex                BandCombinationIndex,

requestedFeatureSets                FeatureSetEntryIndex

}

FR-InfoList ::= SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::= SEQUENCE {

servCellIndex       ServCellIndex,

fr-Type             ENUMERATED {fr1, fr2}

}

CandidateServingFreqListNR ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueNR

CandidateServingFreqListEUTRA ::= SEQUENCE (SIZE (1.. maxFreqIDC-MRDC)) OF ARFCN-ValueEUTRA

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the source secondary node suggests the target secondary eNB to consider configuring. This field is only used in NE-DC.

candidateServingFreqListNR, candidateServingFreqListEUTRA Indicates frequencies of candidate serving cells for In-Device Co-existence Indication (see TS 36.331 [10]).

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can be used to request configuring an NR band combination whose use MN has previously forbidden.

drx-ConfigSCG This field contains the complete DRX configuration of the SCG. This field is only used in NR-DC.

drx-InfoSCG This field contains the DRX long and short cycle configuration of the SCG. This field is used in (NG)EN-DC and NE-DC.

fr-InfoListSCG Contains information of FR information of serving cells that include PScell and Scells configured in SCG.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

needForGaps In NE-DC, indicates wheter the SN requests gNB to configure measurements gaps.

ph-InfoSCG Power headroom information in SCG that is needed in the reception of PHR MAC CE of MCG

ph-SupplementaryUplink Power headroom information for supplementary uplink. In the case of (NG)EN-DC and NR-DC, this field is only present when two UL carriers are configued for a serving cell and one UL carrier reports type1 PH while the other reports type 3 PH.

ph-Type1or3 Type of power headroom for a certain serving cell in SCG (PSCell and activated SCells). Value type1 refers to type 1 power headroom, value type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

pSCellFrequency, pSCellFrequencyEUTRA Indicates the frequency of PSCell in NR (i.e., pSCellFrequency) or E-UTRA (i.e., pSCellFrequencyEUTRA). In this version of the specification, pSCellFrequency is not used in NE-DC whereas pSCellFrequencyEUTRA is only used in NE-DC.

reportCGI-RequestNR, reportCGI-RequestEUTRA Used by SN to indicate to MN about configuring reportCGI procedure. The request may optionally contain information about the cell for which SN intends to configure reportCGI procedure. In this version of the specification, the reportCGI-RequestNR is used in (NG)EN-DC and NR-DC whereas reportCGI-RequestEUTRA is used only for NE-DC.

requestedBC-MRDC Used to request configuring an NR band combination and corresponding feature sets which are forbidden to use by MN.

requestedPDCCH-BlindDetectionSCG Requested value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

requestedP-MaxEUTRA Requested value for the maximum power for the serving cells the UE can use in E-UTRA SCG. This field is only used in NE-DC.

requestedP-MaxFR1 Requested value for the maximum power for the serving cells on frequency range 1 (FR1) in this secondary cell group (see TS 38.104 [12]) the UE can use in NR SCG.

scg-CellGroupConfig Contains the RRCReconfiguration message: - to be sent to the UE, used upon SCG establishment or modification, as generated (entirely) by the (target) SgNB. In this case, the SN sets the RRCReconfiguration message in accordance with clause 6 e.g. regarding the "Need" or "Cond" statements. or - including the current SCG configuration of the UE, when provided in response to a query from MN, or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with clause 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. This field is not applicable in NE-DC.

scg-CellGroupConfigEUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. Used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SeNB. This field is absent when master gNB uses full configuration option. This field is only used in NE-DC.

scg-RB-Config Contains the IE RadioBearerConfig: - to be sent to the UE, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB or SeNB. In this case, the SN sets the RadioBearerConfig in accordance with section 6, e.g. regarding the "Need" or "Cond" statements. or - including the current SCG RB configuration of the UE, when provided in response to a query from MN or in SN triggered SN change in order to enable delta signaling by the target SN. In this case, the SN sets the RRCReconfiguration message in accordance with section 11.2.3. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG RB (re)configuration.

selectedBandCombination Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC.

BandCombinationInfoSN field descriptions

bandCombinationIndex The position of a band combination in the supportedBandCombinationList

requestedFeatureSets The position in the FeatureSetCombination which identifies one FeatureSetUplink/Downlink for each band entry in the associated band combination

Next Modified Subclause

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration. It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB or eNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=               SEQUENCE {

criticalExtensions              CHOICE {

c1                              CHOICE{

cg-ConfigInfo               CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture        SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=           SEQUENCE {

ue-CapabilityInfo               OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)          OPTIONAL,-- Cond SN-AddMod

candidateCellInfoListMN         MeasResultList2NR                                                 OPTIONAL,

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)                       OPTIONAL,

measResultCellListSFTD-NR       MeasResultCellListSFTD-NR                                         OPTIONAL,

scgFailureInfo                  SEQUENCE {

failureType                     ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, synchReconfigFailure-SCG,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG                   OCTET STRING (CONTAINING MeasResultSCG-Failure)

}                                                                                                 OPTIONAL,

configRestrictInfo              ConfigRestrictInfoSCG                                             OPTIONAL,

drx-InfoMCG                     DRX-Info                                                          OPTIONAL,

measConfigMN                    MeasConfigMN                                                      OPTIONAL,

sourceConfigSCG                 OCTET STRING (CONTAINING RRCReconfiguration)                      OPTIONAL,

scg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,

mcg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)                       OPTIONAL,

mrdc-AssistanceInfo             MRDC-AssistanceInfo                                               OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1540-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1540-IEs ::=     SEQUENCE {

ph-InfoMCG                      PH-TypeListMCG                                                    OPTIONAL,

measResultReportCGI             SEQUENCE {

ssbFrequency                    ARFCN-ValueNR,

cellForWhichToReportCGI         PhysCellId,

cgi-Info                        CGI-InfoNR

}                                                                                                 OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1560-IEs                                           OPTIONAL

}

CG-ConfigInfo-v1560-IEs ::=	 SEQUENCE {

candidateCellInfoListMN-EUTRA       OCTET STRING                                              OPTIONAL,

candidateCellInfoListSN-EUTRA       OCTET STRING                                              OPTIONAL,

sourceConfigSCG-EUTRA               OCTET STRING                                              OPTIONAL,

scgFailureInfoEUTRA                 SEQUENCE {

failureTypeEUTRA                    ENUMERATED { t313-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure},

measResultSCG-EUTRA                 OCTET STRING

}                                                                                             OPTIONAL,

drx-ConfigMCG                       DRX-Config                                                OPTIONAL,

measResultReportCGI-EUTRA               SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cellForWhichToReportCGI-EUTRA           EUTRA-PhysCellId,

cgi-InfoEUTRA                           CGI-InfoEUTRA

}                                                                                             OPTIONAL,

measResultCellListSFTD-EUTRA        MeasResultCellListSFTD-EUTRA                              OPTIONAL,

fr-InfoListMCG                      FR-InfoList                                               OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                               OPTIONAL

}

ConfigRestrictInfoSCG ::=       SEQUENCE {

allowedBC-ListMRDC              BandCombinationInfoList                                       OPTIONAL,

powerCoordination-FR1               SEQUENCE {

p-maxNR-FR1                     P-Max                                                     OPTIONAL,

p-maxEUTRA                      P-Max                                                     OPTIONAL,

p-maxUE-FR1                     P-Max                                                     OPTIONAL

}                                                                                             OPTIONAL,

servCellIndexRangeSCG           SEQUENCE {

lowBound                        ServCellIndex,

upBound                         ServCellIndex

}                                                                                             OPTIONAL,   -- Cond SN-AddMod

maxMeasFreqsSCG                     INTEGER(1..maxMeasFreqsMN)                                OPTIONAL,

-- TBD Late Drop: If maxMeasIdentitiesSCG is used needs to be decided after RAN4 replies to the LS on measurement requirements for MR-DC.

maxMeasIdentitiesSCG-NR             INTEGER(1..maxMeasIdentitiesMN)                           OPTIONAL,

...,

[[

selectedBandEntriesMN            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandEntryIndex  OPTIONAL,

pdcch-BlindDetectionSCG          INTEGER (1..15)                                              OPTIONAL,

maxNumberROHC-ContextSessionsSN  INTEGER(0.. 16384)                                           OPTIONAL

]]

}

BandEntryIndex ::=              INTEGER (0.. maxNrofServingCells)

PH-TypeListMCG ::=              SEQUENCE (SIZE (1..maxNrofServingCells)) OF PH-InfoMCG

PH-InfoMCG ::=                  SEQUENCE {

servCellIndex                       ServCellIndex,

ph-Uplink                           PH-UplinkCarrierMCG,

ph-SupplementaryUplink              PH-UplinkCarrierMCG                                       OPTIONAL,

...

}

PH-UplinkCarrierMCG ::=         SEQUENCE{

ph-Type1or3                         ENUMERATED {type1, type3},

...

}

BandCombinationInfoList ::=     SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationInfo

BandCombinationInfo ::=         SEQUENCE {

bandCombinationIndex            BandCombinationIndex,

allowedFeatureSetsList          SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSetEntryIndex

}

FeatureSetEntryIndex ::=        INTEGER (1.. maxFeatureSetsPerBand)

DRX-Info ::=                    SEQUENCE {

drx-LongCycleStartOffset        CHOICE {

ms10                            INTEGER(0..9),

ms20                            INTEGER(0..19),

ms32                            INTEGER(0..31),

ms40                            INTEGER(0..39),

ms60                            INTEGER(0..59),

ms64                            INTEGER(0..63),

ms70                            INTEGER(0..69),

ms80                            INTEGER(0..79),

ms128                           INTEGER(0..127),

ms160                           INTEGER(0..159),

ms256                           INTEGER(0..255),

ms320                           INTEGER(0..319),

ms512                           INTEGER(0..511),

ms640                           INTEGER(0..639),

ms1024                          INTEGER(0..1023),

ms1280                          INTEGER(0..1279),

ms2048                          INTEGER(0..2047),

ms2560                          INTEGER(0..2559),

ms5120                          INTEGER(0..5119),

ms10240                         INTEGER(0..10239)

},

shortDRX                            SEQUENCE {

drx-ShortCycle                      ENUMERATED  {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer                 INTEGER (1..16)

}                                                                                             OPTIONAL

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN               SEQUENCE (SIZE (1..maxMeasFreqsMN)) OF NR-FreqInfo        OPTIONAL,

measGapConfig                       SetupRelease { GapConfig }                                OPTIONAL,

gapPurpose                          ENUMERATED {perUE, perFR1}                                OPTIONAL,

...,

[[ measGapConfigFR2                 SetupRelease { GapConfig }                                OPTIONAL

]]

}

MRDC-AssistanceInfo ::= SEQUENCE {

affectedCarrierFreqCombInfoListMRDC     SEQUENCE (SIZE (1..maxNrofCombIDC)) OF AffectedCarrierFreqCombInfoMRDC,

...

}

AffectedCarrierFreqCombInfoMRDC ::= SEQUENCE {

victimSystemType                    VictimSystemType,

interferenceDirectionMRDC           ENUMERATED {eutra-nr, nr, other, utra-nr-other, nr-other, spare3, spare2, spare1},

affectedCarrierFreqCombMRDC         SEQUENCE    {

affectedCarrierFreqCombEUTRA        AffectedCarrierFreqCombEUTRA                      OPTIONAL,

affectedCarrierFreqCombNR           AffectedCarrierFreqCombNR

}       OPTIONAL

}

VictimSystemType ::= SEQUENCE {

gps                         ENUMERATED {true}               OPTIONAL,

glonass                     ENUMERATED {true}               OPTIONAL,

bds                         ENUMERATED {true}               OPTIONAL,

galileo                     ENUMERATED {true}               OPTIONAL,

wlan                        ENUMERATED {true}               OPTIONAL,

bluetooth                   ENUMERATED {true}               OPTIONAL

}

AffectedCarrierFreqCombEUTRA ::= SEQUENCE (SIZE (1..maxNrofServingCellsEUTRA)) OF ARFCN-ValueEUTRA

AffectedCarrierFreqCombNR ::= SEQUENCE (SIZE (1..maxNrofServingCells)) OF ARFCN-ValueNR

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBC-ListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select the SCG band combination. Each entry refers to a band combination numbered according to supportedBandCombinationList in the UE-MRDC-Capability (in case of (NG)EN-DC or NE-DC) or UE-NR-Capability (in case of NR-DC) and the Feature Sets allowed for each band entry. All MR-DC band combinations indicated by this field comprise the MCG band combination, which is a superset of the MCG band(s) selected by MN.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB or DU to consider configuring. For (NG)EN-DC, including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification. For NR-DC, including SSB and/or CSI-RS measurement results in candidateCellInfoListMN is supported.

candidateCellInfoListMN-EUTRA, candidateCellInfoListSN-EUTRA Includes the MeasResultList3EUTRA as specified in TS 36.331 [10]. Contains information regarding cells that the master node or the source node suggests the target secondary eNB to consider configuring. These fields are only used in NE-DC.

configRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

drx-ConfigMCG This field contains the complete DRX configuration of the MCG. This field is only used in NR-DC.

drx-InfoMCG This field contains the DRX long and short cycle configuration of the MCG. This field is used in (NG)EN-DC and NE-DC.

fr-InfoListMCG Contains information of FR information of serving cells that include PCell and SCell(s) configured in MCG.

maxMeasFreqsSCG Indicates the maximum number of NR inter-frequency carriers the SN is allowed to configure with PSCell for measurements.

maxMeasIdentitiesSCG-NR Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure.

maxNumberROHC-ContextSessionsSN Indicates the maximum number of context sessions allowed to SN terminated bearer, excluding context sessions that leave all headers uncompressed.

measuredFrequenciesMN Used by MN to indicate a list of frequencies measured by the UE.

measGapConfig Indicates the FR1 and perUE measurement gap configuration configured by MN.

measGapConfigFR2 Indicates the FR2 measurement gap configuration configured by MN.

mcg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in MCG, used by the SN to support delta configuration to UE, for bearer type change between MN terminated bearer with NR PDCP to SN terminated bearer. It is also used to indicate the PDCP duplication related information for MN terminated split bearer (whether duplication is configured and if so, whether it is initially activated) in SN Addition/Modification procedure. Otherwise, this field is absent.

measResultReportCGI, measResultReportCGI-EUTRA Used by MN to provide SN with CGI-Info for the cell as per SN′s request. In this version of the specification, the measResultReportCGI is used for (NG)EN-DC and NR-DC and the measResultReportCGI-EUTRA is used only for NE-DC.

measResultSCG-EUTRA This field includes the MeasResultSCG-FailureMRDC IE as specified in TS 36.331 [10]. This field is only used in NE-DC.

measResultSFTD-EUTRA SFTD measurement results between the PCell and the E-UTRA PScell in NE-DC. This field is only used in NE-DC.

mrdc-AssistanceInfo Contains the IDC assistance information for MR-DC reported by the UE (see TS 36.331 [10]).

p-maxEUTRA Indicates the maximum total transmit power to be used by the UE in the E-UTRA cell group (see TS 36.104 [33]). This field is used in (NG)EN-DC and NE-DC.

p-maxNR-FR1 Indicates the maximum total transmit power to be used by the UE in the NR cell group across all serving cells in frequency range 1 (FR1) (see TS 38.104 [12]) the UE can use in NR SCG.

p-maxUE-FR1 Indicates the maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1).

pdcch-BlindDetectionSCG Indicates the maximum value of the reference number of cells for PDCCH blind detection allowed to be configured for the SCG.

ph-InfoMCG Power headroom information in MCG that is needed in the reception of PHR MAC CE in SCG.

ph-SupplementaryUplink Power headroom information for supplementary uplink. For UE in (NG)EN-DC, this field is absent.

ph-Type1or3 Type of power headroom for a serving cell in MCG (PCell and activated SCells). type1 refers to type 1 power headroom, type3 refers to type 3 power headroom. (See TS 38.321 [3]).

ph-Uplink Power headroom information for uplink.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

scgFailureInfo Contains SCG failure type and measurement results. In case the sender has no measurement results available, the sender may include one empty entry (i.e. without any optional fields present) in measResultsPerMOList. This field is used in (NG)EN-DC and NR-DC.

scgFailureInfoEUTRA Contains SCG failure type and measurement results of the EUTRA secondary cell group. This field is only used in NE-DC.

scg-RB-Config Contains all of the fields in the IE RadioBearerConfig used in SCG, used to allow the target SN to use delta configuration to the UE, e.g. during SN change. The field is signalled upon change of SN. Otherwise, the field is absent. This field is also absent when master eNB uses full configuration option.

selectedBandEntiesMN Indicates the position of a band entry selected by the MN, in the first band combination entry in allowedBC-ListMRDC IE. Each band entry in the subset is identified by its position in the bandlist of this BandCombination. The SN uses this information to determine which bands out of the NR band combinations in allowedBC-ListMRDC it can configure in SCG. This field is only used in NR-DC.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

sourceConfigSCG Includes all of the current SCG configurations used by the target SN to build delta configuration to be sent to UE, e.g. during SN change. The field contains the RRCReconfiguration message, i.e. including CellGroupConfig and measConfig. The field is signalled upon change of SN, unless MN uses full configuration option. Otherwise, the field is absent.

sourceConfigSCG-EUTRA Includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration. In this version of the specification, this field is absent when master gNB uses full configuration option. This field is only used in NE-DC.

ue-CapabilityInfo Contains the IE UE-CapabilityRAT-ContainerList supported by the UE (see NOTE 3). The field is signalled upon addition, modification or change of SN.

BandCombinationInfo field descriptions

allowedFeatureSetsList Defines a subset of the entries in a FeatureSetCombination. Each index identifies a position in the FeatureSetCombination, which corresponds to one FeatureSetUplink/Downlink for each band entry in the associated band combination.

bandCombinationIndex The position of a band combination in the supportedBandCombinationList

| Conditional Presence | Explanation |
| --- | --- |
| SN-AddMod | The field is mandatory present upon SN addition and SN change. It is optionally present upon SN modification. Otherwise, the field is absent. |

NOTE 3:	The following table indicates per source RAT whether RAT capabilities are included or not in ue-CapabilityInfo.

| Source RAT | NR capabilities | E-UTRA capabilities | MR-DC capabilities |
| --- | --- | --- | --- |
| E-UTRA | Included | Not included | Included |

Next Modified Subclause