3GPP TSG-RAN2 Meeting #107	R2-1911869

# Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1220 | rev | 3 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Channel Bandwidth validation upon SIB1 acquisition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to 38.331, section 5.2.2.4.2 the UE shall verify that it supports the width of the initial BWP as indicated in SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> initialDownlinkBWP-> genericParameters-> locationAndBandwidth. In addition, it also verifies whether it supports the given band numbers and the additionalSpectrumEmission requirements. It does however not verify whether it supports the channel bandwdith given in SIB1-> servingCellConfigCommon-> downlinkConfigCommon-> frequencyInfoDL-> scs-SpecificCarrierList-> carrierBandwidth. In FR1 the maximum defined channel bandwidth of 100 MHz is mandatory without capability signalling. However, for narrower channels the UE may indicate lack of IODT in its UE capabilities. This might imply that the UE does not fulfill the out-of-band-emission requirements on carriers of such bandwidth. Example 1: Channel bandwidths supported by UE: 100, 80, 40, 20 MHz SIB1: - Carrier bandwidth: 90 MHz - Initial BWP bandwidth: 90 MHz As discussed during email discussion [106-28], a UE will likely support (initial-) bandwidth parts that are smaller than or equal to the channel bandwidths they support. Since 100 MHz is a mandatory channel bandwidth in FR1, UEs will likely support any initial BWP bandwidth up to and including 100 MHz. Hence, the current condition in 38.331 would likely be fulfilled by all UEs: 2> if the UE supports the bandwidth of the initial uplink BWP and of the initial downlink BWPs indicated in the locationAndBandwidth fields in uplinkConfigCommon: But since the UE does not support a channel bandwidth of 90 MHz, it cannot ensure appropriate out-of-band emissions and should hence not be allowed to use this cell. Based on the UE capabilities the NW could refrain from configuring a UE with an SCell or PSCell for which it does not support the necessary channel bandwidth. For initial access on the PCell this is not possible. Therefore, another condition should be added to section 5.2.2.4.2 to ensure that the UE supports the channel bandwidth used by the NW. The NW has the possibility to configure a UE specific channel bandwidth as well as UE specific BWPs. But to be able to do that, the UE must at least support a channel bandwdith which is ... a) equal to or smaller than the carrier bandwidth indicated in SIB1 and b) equal to or larger than the bandwidth of the initial BWP. In Example 1 above, the UE would hence have had to support the channel- and initial BWP bandwidth of 90 MHz. In Example 2 the UE could access the cell since it supports two channel bandwidths (80 & 40 MHz) which fulfill the requirements above: Example 2: Channel bandwidths supported by UE: 100, 80, 40, 20 MHz SIB1: - Carrier bandwidth: 90 MHz - Initial BWP bandwidth: 40 MHz Once the UE established the RRC connection the network would have to configure a UE specific channel bandwidth of 40 or 80 MHz. If it configures 80 MHz channel bandwidth, it may also configure an “other BWP” of up to 80 MHz As discussed also in [106-28], it is currently unclear which channel bandwidth the UE applies until having obtained a UE-specific channel bandwdith value. Following the reasoning above, the UE should apply a channel bandwdith that fulfills the requirements “a” and “b” above. The problems discussed above were also identified by RAN4. Their reply LS (in R4-1910522) suggests resolving them as proposed in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add a condition in section 5.2.2.4.2 to prevent a UE from accessing the cell if it does not support a suitable channel bandwidth. 2) Clarify in section 5.2.2.4.2 which channel bandwidth the UE should apply. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: Initial access Inter-operability: If the UE is implemented according to the CR but the NW does not, there is no interoperability problem. If the NW is implemented according to the CR but the UE is not, there is no interoperability problem but the original issue remains (see “Reason for change” and “Consequence if not approved”) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear which channel bandwidths UE need to support to camp on a cell. The UE may access a cell operating on a carrier bandwidth which the UE does not support. The UE may cause out of band emissions to neighbouring carriers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN;

1>	if in RRC_CONNECTED while T311 is not running:

2>	disregard the frequencyBandList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

2>	apply the configuration included in the servingCellConfigCommon;

1>	else:

2>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList for downlink, and one or more of the frequency bands indicated in the frequencyBandList for uplink for FDD, and they are not downlink only bands, and

2>	if the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList for a supported band in the downlink and a supported band in uplink for FDD, and

2>	if the UE supports the bandwidth of the initial uplink BWP and of the initial downlink BWP indicated in the locationAndBandwidth fields in uplinkConfigCommon and downlinkConfigCommon, respectively, and

2> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which
is smaller than or equal to the carrierBandwidth (indicated in uplinkConfigCommon for the SCS of the initial uplink BWP), and which 
is wider than or equal to the bandwidth of the initial uplink BWP, and

2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which
is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP), and which 
is wider than or equal to the bandwidth of the initial downlink BWP:

3> apply a supported uplink channel bandwidth with a maximum transmission bandwidth which 
is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP, and which
is wider than or equal to the bandwidth of the initial BWP for the uplink;

3> apply a supported downlink channel bandwidth with a maximum transmission bandwidth which 
is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP, and which 
is wider than or equal to the bandwidth of the initial BWP for the downlink;

3>	select the first frequency band in the frequencyBandList which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present;

3>	forward the cellIdentity to upper layers;

3>	if trackingAreaCode is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	if intraFreqReselection is set to notAllowed:

5>	consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [20];

4>	else:

5>	consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [20];

3>	else:

4>	forward the trackingAreaCode to upper layers;

3>	forward the PLMN identity to upper layers;

3>	if in RRC_INACTIVE and the forwarded information does not trigger message transmission by upper layers:

4>	if the serving cell does not belong to the configured ran-NotificationAreaInfo:

5>	initiate an RNA update as specified in 5.3.13.8;

3>	forward the ims-EmergencySupport to upper layers, if present;

3>	forward the uac-AccessCategory1-SelectionAssistanceInfo to upper layers, if present;

3>	apply the configuration included in the servingCellConfigCommon;

3>	apply the specified PCCH configuration defined in 9.1.1.3;

3>	if the UE has a stored valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with sub-clause 5.2.2.1:

4>	use the stored version of the required SIB;

3>	if the UE has not stored a valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s), in accordance with sub-clause 5.2.2.1:

4>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

5>	acquire the SI message(s) as defined in sub-clause 5.2.2.3.2;

4>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

5>	trigger a request to acquire the SI message(s) as defined in sub-clause 5.2.2.3.3;

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList in uplinkConfigCommon;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

4>	apply the additionalPmax in uplinkConfigCommon for UL;

3>	else:

4>	apply the p-Max in uplinkConfigCommon for UL;

3>	if supplementaryUplink is present in servingCellConfigCommon; and

3>	if the UE supports one or more of the frequency bands indicated in the frequencyBandList of supplementary uplink; and

3>	if the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList for a supported supplementary uplink band; and

3>	if the UE supports the bandwidth of the initial uplink BWP indicated in the locationAndBandwidth fields of supplementary uplink, and

3> if the UE supports an uplink channel bandwidth with a maximum transmission bandwith configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which
is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which
is wider than or equal to the bandwidth of the initial uplink BWP of the SUL:

4>	consider supplementary uplink as configured in the serving cell;

4>	apply a supported supplementary uplink channel bandwidth with a maximum transmission bandwidth which 
is contained withn the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which
is wider than or equal to the bandwidth of the initial BWP of the SUL;

4>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList for the supplementaryUplink;

4>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList for the supplementaryUplink:

5>	apply the additionalPmax in supplementaryUplink for SUL;

4>	else:

5>	apply the p-Max in supplementaryUplink for SUL;

2>	else:

3>	consider the cell as barred in accordance with TS 38.304 [20]; and

3>	perform barring as if intraFreqReselection is set to notAllowed;