3GPP TSG-RAN WG2 Meeting #107	R2-1909158

# Prague, Czech Republic, 26-30 August 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | CRnum | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for 2-step RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd, Nokia (Stage2 spec rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  |  | Date: |  | 2019-06-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of 2-step RACH |  |  |  |  |  |  |  |  |
| Summary of change: |  | The agreements from RAN2#105-bis and RAN2#106 are captured in this version: 1. RA triggers applicable for 2 step RACH; 2. 2 step RACH and 4 step RACH selection; 3. only CBRA for 2 step RACH; 4. MSGA structure; 5. Network response (MSGB) and fallback mechanism. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

Cell-Defining SSB: an SSB with an RMSI associated.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

MSGA: a preamble on PRACH and a payload on PUSCH transmitted as part of the 2-step contention-based random access procedure

MSGB: TBD

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Xn: network interface between NG-RAN nodes.

Next change

### 9.2.6	Random Access Procedure

The random access procedure is triggered by a number of events:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised";

-	UL data arrival during RRC_CONNECTED when there are no PUCCH resources for SR available;

-	SR failure;

-	Request by RRC upon synchronous reconfiguration (e.g. handover);

-	Transition from RRC_INACTIVE;

-	To establish time alignment for a secondary TAG;

-	Request for Other SI (see clause 7.3);

NOTE:	MSG1 based request for other SI is not supported with 2-step CBRA.

-	Beam failure recovery.

Three types of random access procedure are supported: A 4-step contention-based random access (CBRA) a 2-step CBRA and contention-free random access (CFRA) as shown on Figure 9.2.6-1 below. The UE selects the type of random access based on network configuration.

The MSGA of the 2-step CBRA includes a preamble on PRACH and a payload on PUSCH. After MSGA transmission, the UE monitors for a response from the network within a configured window. If contention resolution is received successfully in MSGB, it ends the random access procedure as shown in Figure 9.2.6-1(b); while if fallback indication is received, the UE falls back to 4-step CBRA and continues with MSG3 transmission as shown in Figure 9.2.6-2.

(a)	4-step CBRA                                (b)	2-step CBRA                                (c) CFRA

Figure 9.2.6-1: Random Access Procedures

Figure 9.2.6-2: Fallback from 2-step CBRA to 4-step CBRA

Editor’s Note:	The above pictures (9.2.6-1(b) and 9.2.6-2) may need to be updated once the details of MSGB format are finalised.

For random access in a cell configured with SUL, the network can explicitly signal which carrier to use (UL or SUL). Otherwise, the UE selects the SUL carrier if and only if the measured quality of the DL is lower than a broadcast threshold. Once started, all uplink transmissions of the random access procedure remain on the selected carrier.

When CA is configured, the first three steps of CBRA always occur on the PCell while contention resolution (step 4) can be cross-scheduled by the PCell. The three steps of a CFRA started on the PCell remain on the PCell. CFRA on SCell can only be initiated by the gNB to establish timing advance for a secondary TAG: the procedure is initiated by the gNB with a PDCCH order (step 0) that is sent on a scheduling cell of an activated SCell of the secondary TAG, preamble transmission (step 1) takes place on the indicated SCell, and Random Access Response (step 2) takes place on PCell.

Annex (not part of the specification): RAN2 Agreements

## RAN2#105bis

- Criteria on whether the UE uses 2-step RACH or 4-step RACH shall be clearly specified

- The start of the msgB reception window is after the PUSCH transmission opportunity of msgA.  Details are FFS for 2-step RACH and fallback.

- If CCCH SDU was included in MsgA, then the contention resolution will be based on the contention resolution ID included in MsgB.  FFS for other conditions.

- 2-step RACH is applicable for Msg3 based SI request.

- 2-step RACH is applicable for CB BFR.  FFS for CFRA

## RAN2#106

- From RAN2 perspective, 2-step RACH selection can be based on indicating to all UEs via SIB, or dedicated configuration in RRC_CONNECTED/INACTIVE/IDLE states. FFS if radio quality is used for 2-step RACH selection. - From RAN2 perspective, for msgA retransmission (i.e. preamble and PUSCH) we assume that the UE retries on 2-step RACH - FFS whether the UE can fallback to 4-step RACH after certain time. Ask RAN1 whether the preamble transmission performance for 2-step RACH and 4-step RACH is the same. - For MsgA with C-RNTI, the UE shall monitor the PDCCH addressed to C-RNTI for success response and msgB-RNTI (e.g. RA-RNTI or new RNTI) Contention resolution: a. If the PDU PDCCH addressed to the C-RNTI (i.e. C-RNTI included in MsgA) containing the 12 bit TA command is received, the UE should consider the contention resolution to be successful and stop the reception of MsgB or with UL grant if the UE is synchronized already. b. If the corresponding fallback RAR is detected, the UE should stop the monitoring of PDCCH addressed to the corresponding C-RNTI for success response and process the fallback operation accordingly. c. If neither corresponding fallback RAR nor PDCCH addressed C-RNTI is detected within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB. d. FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced - For CCCH, MsgB can include the SRB RRC message. The format should be designed for both with and without RRC message. - For CCCH, for success or fallback RAR MsgB can multiplex messages for multiple UEs. FFS if we can multiplex SRB RRC messages of multiple UEs. - Network response to msgA (i.e. msgB/msg2) can include the following: a. SuccessRAR b. FallbackRAR c. Backoff Indication FFS: format of successRAR and whether successRAR is split into more than one message and format of fallbackRAR and whether legacy msg2 can be reused for fallbackRAR - The following fields can be included in the successRAR when CCCH message is included in msgA. a. Contention resolution ID b. C-RNTI c. TA command - Upon receiving the fallbackRAR, the UE shall proceed to msg3 step of 4-step RACH procedure - FallbackRAR should contain the following fields a. RAPID b. UL grant (to retransmit the msgA payload). FFS on restrictions on the grant and UE behavior if different grant and rebuilding c. TC-RNTI d. TA command - From RAN2 perspective, no further offset is needed for the start of msgB monitoring window (i.e. no offset is needed to cover the RRC processing delay and/or F1 delay). - The UE will monitor for response message using the single msgB agreed window - MsgB containing the succcessRAR shall not be multiplexed with the legacy 4-step RACH RAR in the same MAC PDU