3GPP TSG-RAN WG2 Meeting #107	R2-1909309

Prague,  Czech Republic, 26 – 30 August 2019

Agenda item:	11.18

Source:	Nokia, Nokia Shanghai Bell

Title:	Adding Human-Readable network names for NPNs to SIB

WID/SID:	NG_RAN_PRN - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN agreed that the Non-Public Networks (NPN) related RAN work will be carried out within NG_RAN_PRN work item [1]. There is an option that human-readable network names are also advertised to help in manual SNPN or CAG selection. There can a human-readable network name for each advertised NID, and for each advertised CAG Identifiers [2]. This paper proposes how to add the human readable name to SIBs.

# 2	Discussion

The purpose of the optional human-readable names (HRN) is to present them to user during manual SNPN or CAG selection. The size of the human-readable names has not been specified, however it is expected that it will be similar as the size of SSIDs in Wi-Fi networks. The maximum size of SSIDs is 32 character. Therefore, if multiple NIDs and/or CAG are supported (the requirement is that a RAN node can support broadcasting a total of twelve NIDs or CAG identifiers) by a cell then the size of HRNs can be significant. HRNs are not essential information as they are not used during automatic network selection or during cell selection and reselection [2]. Therefore, our view is that defining a new, optional SIB for this purpose would be the best solution to advertise the optional HRNs.

Proposal 1: Define a new optional SIB for advertising human-readable names for SNPNs and CAGs.

A cell may advertise this new SIB in the same way as all other SI message: periodically or based on request coming from a UE. A UE can request the advertisement before attaching to the network. Thus, our conclusion is that this new SIB does not require any changes in the SIB transmission and acquiring mechanisms.

Observation 1: The introduction of a new optional SIB for advertising human-readable names for SNPNs and CAGs does not require any change in the SIB transmission and acquiring mechanisms.

The new SIB should contain a list of human-readable names assigned to different NPNs supported by the cell. No other information is needed. A possible structure is presented in Figure 1.

-- ASN1START

-- TAG-SIBX-START

SIBX ::=                            SEQUENCE {

humanReadableNameListNPN            HumanReadableNameListNPN,

...

}

HumanReadableNameListNPN ::= SEQUENCE (SIZE (1..maxHumanReadableName)) OF HumanReadableNameInfo

HumanReadableNameInfo ::=           SEQUENCE {

plmn-Identity                       PLMN-Identity,

npn-Identifier                      CHOICE {

nid                                 NID,

cag-Identifier                      CAG-Identifier

},

humanReadableName                   OCTET STRING (SIZE(1..NN)),

...

}

-- TAG-SIBX-STOP

-- ASN1STOP

Figure 1: Proposed structure for the new SIBX

Proposal 2: The structure of the new optional SIB for advertising human-readable names for SNPNs and CAGs follow the structure presented in Figure 1.

# 3	Conclusions

The observation and proposals made in this contribution are summarized below.

Proposal 1: Define a new optional SIB for advertising human-readable names for SNPNs and CAGs.

Observation 1: The introduction of a new optional SIB for advertising human-readable names for SNPNs and CAGs does not require any change in the SIB transmission and acquiring mechanisms.

Proposal 2: The structure of the new optional SIB for advertising human-readable names for SNPNs and CAGs follow the structure presented in Figure 1.

# References

[1]	RP-190729: "New Work Item Proposal on Private Network Support for NG-RAN"

[2]	3GPP TS 23.501, System Architecture for the 5G System; Stage 2 (Release 16)