3GPP TSG RAN WG2 #107	 R2-1909414

Prague, Czech Republic, 26 – 30th August 2019

Agenda Item:	12.3.3.1 Execution Details of CHO

Source: 	Ericsson

Title:  	Reconfiguration while monitoring CHO

Document for:	Discussion and Decision

# Introduction

In the work item for Further Enhanced Mobility Enhancements, one objective is to improve the robustness at handover. In RAN2#105bis it has been agreed to support Conditional handover (CHO) in LTE.

In this contribution we discuss some further stage-2 principles for Conditional Handover (CHO) in E-UTRAN.

# Discussion

## Measurement reporting while the UE is monitoring CHO conditions

In LTE reconfiguration with sync, measurement configuration/reporting and handovers are independent procedures specified in RRC, even if measurement configuration/reporting may be used by the network to assist handover decisions. It seems obvious that the same principle is considered for CHO and measurement configuration/reporting.

Measurement reporting and CHO are independent RRC procedures.

In RAN2#105, the following has been agreed concerning CHO in LTE

……………………………………………………………………………………..

3: The baseline operation for E-UTRAN Conditional HO assumes the source eNB remains responsible for RRC until UE successfully sends RRC Connection Reconfiguration Complete message to target eNB.

In RAN2#105bis, the following was agreed regarding conventional and conditional handover:

Agreements

. . .

2  Conventional handover overrides any configured conditional handover command

In email discussion [1] different proposals regarding CHO reconfiguration (e.g. update of CHO execution condition, target and source cell configuration) was discussed and agreed. To enable the source eNB to perform such possible reconfigurations, measurement report is required from UE. Consequently, it should be possible that while the UE monitors CHO triggering conditions, requiring the UE to perform measurements, the UE is also performing RRM measurements configured by the network and possibly triggering measurement reports.

UE sends measurement reports while monitoring CHO triggering conditions.

# Summary

RAN2 is kindly asked to discuss the following observations and proposals:

- Measurement reporting and CHO are independent RRC procedures.

- Proposal 1 	UE sends measurement reports while monitoring CHO triggering conditions.

# References

- R2-19xxxxx Summary of [106#42] [NR/LTE/mob enh] CHO configuration, OPPO, RAN2#107, Prague, Czech, 26-30th August 2019.