3GPP TSG-RAN WG2 #107	R2-1910664

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	11.8.2.1

Source:	Ericsson

Title:	View on Multi-Cell RTT

Document for:	Discussion, Decision

# Introduction

In RAN2#105bis, there was discussion on the scope of E-CID and how multi-cell RTT and/or angle based measurements should be captured in the specification.

In this contribution, we present our view on multi-cell RTT.

# Discussion

The main benefit with multi-cell RTT is that it is possible to operate without an accurate network synchronization or known/estimated relative time differences between network elements. In round trip time computations, positioning algorithms rely on the transmissions in both directions between UE and the network nodes involved in positioning of the UE. The method creates a system of equations solved by finding the point where all the circles created by each downlink/uplink pair intersect.

An alternative is to only perform RTT with the serving cell, and perform reference signal time difference (RSTD) measurements of DL PRS from serving and neighbor cells, and transmit UL PRS to be detected and measured by serving and neighbor cells.

Some limitations with these methods:

- Supporting transmissions to multiple cells in UL has a big impact on UE complexity

- UL transmissions to non-serving cells increase interference in the network

- Uplink signals are usually significantly weaker than their downlink counterparts, e.g., due to power control and also lower maximum output power for UEs.

- Controlling the UE transmit timing for transmissions to neighbor (non-serving) cells is challenging (in LTE, timing advance only for the UE’s serving cell is supported)

- A lot of signaling is needed to make the neighbor cells aware of the exact UE’s UL transmissions they need to receive

- Extensive signaling is needed between the UE and the different neighbors and serving cells.

An alternative simplified positioning method using RTT is based on RTT only to the serving cell, and rely on DL PRS RSTD measurements for the neighbor cells to be reported. That would however require a synchronized network or known RTDs. This can be avoided if the serving base station also measures the time of arrival of the neighbor DL PRS, and reports to the location server.

A simplified multi cell RTT is described below.

RxTx measurements are bidirectional measurements involving both DL and UL. Multi-cell RTT based on UE RxTx measurements could be based on measuring DL signals from the serving and neighbor cells but with the UL transmission to the serving cell only.

DL from serving and neighbor cells and UL transmission to the serving cell only is less complex for both UE and network, less power consuming, and requires less signalling), while having UL transmission to all neighbor cells is much more complex with unclear additional benefits. The above figure depicts a simplified procedure.

Considering above Figure; Rxn1, Rxn2 are the received neighbour cell time computed by UE, Rxs is the serving cell received signal time and Txs is the time when UE transmits in UL to serving cell.

Thus, UE would report

Rxn1 – Txs for Neighbor cell1

Rxn2 – Txs for Neighbor cell 2

Rxs – Txs for serving cell

As of gNB Rx-Tx, in order to avoid complex procedures and signalling, it is suggested that the gNB Rx-Tx measurement is performed only for the serving cell DL and serving cell UL.

Further, a simplified multi-cell RTT is performed only in the same frequency as serving cell. Thus, to avoid complexity inter-frequency layer measurements are avoided.

If multi-cell RTT based upon multiple cells where UE needs to perform Rx-Tx for each neighbor cells needs to be specified, then RAN2 should consult with RAN4 to ensure that the UE requirements to perform such measurements can be fulfilled.

It is complex procedure to perform RTT based upon multiple cells for both UE and for the gNBs which requires UE to be in synch in UL with multiple cells.

RAN2 to agree on definitions of multi-cell RTT based solutions, where complexity, signaling and interference concerns are addressed

UE Tx is only towards the serving cell. UE reports Rx-Tx for neighbor cell where Tx is based upon Tx of serving cell.

Only the serving cell gNB performs Rx-Tx.

Further, [2] discusses overhearing technique which can be also useful multi-cell RTT technique where only the serving cell/gNB and UE needs to perform RxTx and the neighbour gNBs overhear the UE and serving cell/gNB and report the TDOA.

RAN2 to agree on overhearing based multi-cell RTT.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is complex procedure to perform RTT based upon multiple cells for both UE and for the gNBs which requires UE to be in synch in UL with multiple cells.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to agree on definitions of multi-cell RTT based solutions, where complexity, signaling and interference concerns are addressed

Proposal 2	UE Tx is only towards the serving cell. UE reports Rx-Tx for neighbor cell where Tx is based upon Tx of serving cell.

Proposal 3	Only the serving cell gNB performs Rx-Tx.

Proposal 4	RAN2 to agree on overhearing based multi-cell RTT.

# References

[1] 	RP-182155, “Study on NR positioning support”, RAN1, Intel Corporation, Ericsson, June 2018.

R1-1901197, “On the use of RTT for positioning”, Ericsson, Ad-Hoc Meeting 1901