3GPP TSG-RAN WG2 Meeting #107	R2-1911101

Prague, Czech Republic, 26th - 30th August 2019

Agenda item:	11.2.1.2

Source:	Charter Communications

Title:	Handling of Persistent LBT Failures in NR-U

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Operation in unlicensed spectrum requires conformance to regulatory rules. Particularly, listen-before-talk (LBT) rule ensures that operating nodes do not transmit over each other. While LBT is important for operation in unlicensed bands, it affects operation of some of the NR baseline procedures such that it calls for additional attention in case of repetitive LBT failures. In NR-Unlicensed WID [1], the implications that LBT brings has been captured in a few of the objectives.

In RAN2 #105bis meeting [2], the following agreements related to the impact of UL LBT success/fail were reached:

- Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection

- R2 assumes that the configured grant timer is not started/restarted when configured grant is not transmitted due to LBT failure. PDU overwrite need to be avoided somehow.

- The configured grant timer is not started/restarted when UL LBT fails on PUSCH transmission for grant received by PDCCH addressed to CS-RNTI scheduling retransmission for configured grant

- The configured grant timer is not started/restarted when the UL LBT fails on PUSCH transmission for UL grant received by PDCCH addressed to C-RNTI, which indicates the same HARQ process configured for configured uplink grant

- Upon UL transmission on CG, bwp-InactivityTimer is restarted as today (at LBT success)

- Upon UL transmission on CG, sCellDeactivationTimer is restarted as today (at LBT success)

- dataInactivityTimer should be restarted when any MAC entity attempts transmission of a MAC SDU for DTCH logical channel, or DCCH logical channel regardless UL LBT outcome.

- drx-HARQ-RTT-TimerUL should be started/restarted regardless of the LBT outcome for PUSCH transmission with dynamic grant

- drx-HARQ-RTT-TimerUL should not be started/restarted when LBT fails for PUSCH transmission with configured grant

In this contribution, we express our views regarding potential issues that LBT failures, particularly consistent LBT failures, could bring in various MAC procedures in NR-U operation.

# 2	Discussion

There are several MAC procedures that get affected due to possible LBT failure. Particularly, if LBT failures happen persistently, it would trigger timers or counters that eventually may lead to significant delay.

## 2.1 LBT Failures in Uplink Transmission

In the baseline specification, the SR counter and SR prohibit timer are maintained by MAC and depending on their status, an SR transmission may be initiated, prevented, or cancelled. Since the baseline SR procedure (SR initiation and prevention of subsequent SR transmission) are designed considering operation in licensed spectrum, for operation in unlicensed bands, it is vital to make MAC entity aware of whether PHY has been able to transmit the SR as instructed. Particularly, this would help MAC to distinguish an LBT failure vs an unsuccessful transmission event.

In the baseline specification, during random access procedure the preamble transmission counter, power ramping counter, RA response window and RA contention resolution timer are maintained by MAC and depending on their status, the random-access procedure progresses differently. Due to the differences in operation in unlicensed bands, in RAN2#105bis [2], agreements were reached to treat transmission and power ramping counters differently than the baseline operation.

A UE is expected to perform LBT Cat-4 with appropriate CAPC in order to access a provided uplink grant. Persistent LBT failures in transmission of multiple uplink grants, should also be counted toward overall LBT failures. Similarly, persistent LBT failures in accessing configured grants (CG) resources should be counted toward overall LBT failures.

Beside PUSCH, SR and RACH, there are other uplink transmissions that PHY handles them, with RRC configuration and, without direct involvement of MAC. These include: CSI, HARQ feedback, and SRS transmissions. To count LBT failure for these types of uplink transmission, PHY has to inform MAC about any LBT failure so that MAC can track overall LBT failure occurrences. Such notification from PHY to MAC would be unsolicited.

It has been argued that for a simpler implementation, MAC should only track LBT failures of PUSCH, SR and RACH and to ignore LBT failures during transmission of CSI, HARQ feedback and SRS. We argue against this since ignoring any LBT failure would delay LBT failure counter to reach the threshold and that subsequently delays the action that UE is expected to take in the event of persistent LBT failure. We acknowledge the additional burden to consider unsolicited LBT failure from PHY to MAC, however we believe such notifications can be handled along LBT failure indications (for SR, RACH and PUSCH). Therefore, we suggest that LBT failures during all UL transmissions should be counted toward a possible persistent LBT failure event.

Observation 1: Ignoring any LBT failure would delay LBT failure counter to reach the threshold and that subsequently delays the action that UE is expected to take in the event of a persistent LBT failure event.

Also, it has been argued that only UE-initiated unlink transmissions to be considered and transmissions such as CSI report, HARQ feedback, and SRS need not be taken into account since gNB is aware and expecting such transmissions (hence gNB failing to receive these transmissions could be considered as LBT failure at gNB). While gNB becomes aware if CSI report, HARQ feedback, and SRS are not present, it is the UE that has to take an action for a persistent LBT failure and withholding an occurrence of LBT failure (for these types of transmission) from MAC does impairs the ability of the UE to take a timely action. Therefore, in order to aid a UE to act timelier in the event of persistent LBT failures, LBT failure during all types of uplink transmission should be accounted for.

Observation 2: While gNB becomes aware if CSI report, HARQ feedback, and SRS are not present, it is the UE that has to take an action for a persistent LBT failure and withholding an occurrence of LBT failure (for these types of transmission) from MAC does impairs the ability of the UE to take a timely action.

Proposal 1: In order to aid a UE to act timelier in the event of persistent LBT failures, LBT failure during all types of uplink transmission should be accounted for.

An occurrence of LBT failure during any uplink transmission should be considered in case a pattern of persistent LBT failures happen. PHY layer should notify MAC for LBT failure during any uplink transmission.

Proposal 2: Consider a common “LBT status” signalling from PHY to MAC to enable MAC to recognize persistent LBT failure patterns. Inform RAN1 to accommodate such signalling.

## 2.4 Procedure for Detection of Persistent LBT Failures

A persistent LBT failures during performing SR or RA procedures could lead to trigger RLF. This could happen due to following: 1) congested unlicensed channel due to presence of other NR-U or other technologies nodes, 2) presence of interfering nodes in the vicinity of the UE but hidden to gNB.

It is more straightforward and systematic to have a mechanism for systematic LBT failure in MAC without changing the baseline procedures. Enabling MAC to recognize persistent LBT failures, appropriate solutions can be considered to address the problem. Ultimately, if persistent LBT failure reaches a threshold the UE may initiate RLF, but before that for a timelier reaction other measures could be considered. Due to above consequences due to UL LBT failures, in RAN2 #105bis [2], an agreement was reached “to adopt a mechanism in MAC spec to handle the UL LBT failure, where consistent UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection”.

To identify a persistent LBT failures it may be required to initiate counter(s) and/or timer(s). A set of counter/timer would help to recognize how often and how persistently LBT failures happens. A related discussion then would be how to maintain these counter(s)/timer(s).

While it is possible to maintain multiple counter/timer for each type of uplink transmission (e.g. SR, PUSCH and RACH), we believe such level of details is unnecessary since the events that lead to an LBT failure is agnostic to the type of uplink transmission.

Observation 3: Maintaining multiple counter/timer for each type of uplink transmission (e.g. SR, PUSCH and RACH) is unnecessary since the events that lead to an LBT failure is agnostic to the type of uplink transmission.

Having a counter to count the number of LBT failures, a threshold should be configured such that approaching this threshold should be an indication of persistent LBT failures. However, reaching the threshold should be subject to a time duration. Therefore, we suggest a timer should also be initiated after each LBT failure where expiry of the timer indicates the prior LBT failures have been isolated events and the LBT failure counter should be reset. This is motivated by the fact that channel access by competing devices is patchy and in between patches LBT may not fail, but this would not indicate the overall status of channel access.

Proposal 3: Enable MAC to initiate and maintain a counter and a timer in order to detect the occurrence of a persistent LBT failure, where: a) the counter is incremented upon an LBT failure during each uplink transmission, b) the timer is reset after each LBT failure, c) upon expiry of the timer, the counter is reset.

While it is possible to maintain a counter/timer for each LBT category and even each CAPC of LBT Cat-4, we believe such level of details may not help more than having 1-2 counters. Having separate counter for each LBT category and CAPC offers additional information about LBT failure statistics, the details may not make a difference since Statistically, persistent LBT failure for a higher priority LBT category/CAPC should also trigger persistent LBT failure for a lower priority LBT category/CAPC.

Observation 4: Statistically, persistent LBT failure for a higher priority LBT category/CAPC should also trigger persistent LBT failure for a lower priority LBT category/CAPC.

An exception would be persistent LBT failures during LBT Category 2, which given the priority that Category 2 has, a persistent LBT failure should be (compared to a lower threshold and) acted upon in timelier manner.

Proposal 4: Given higher priority that LBT Category 2 has, a persistent LBT failure should be (compared to a lower threshold and) acted upon in timelier manner.

If RAN2 decides to have multiple counters/timers to identify persistent LBT failures during various types of uplink transmissions or various LBT categories, then it is important to consider a sum of LBT failures across the categories and to be compared with a separate threshold.

Proposal 5: If RAN2 decides to have multiple counters/timers to identify persistent LBT failures, then it is important to consider a sum of LBT failures across the categories and to be compared with a separate threshold.

In a wideband operation, given there is a possibility of varying number of competing devices per sub-band it’s logical to allow an event of persistent LBT failure to be detected per sub-band. Without per sub-band granularity, a UE could be declaring persistent LBT failures more often and unnecessarily.

Proposal 6: In a wideband operation, given there is a possibility of varying number of competing devices per sub-band, allow persistent LBT failure events to be detected per sub-band.

# 3	Conclusions

In this contribution, we highlighted our concerns regarding consequences that LBT could bring in several MAC procedures. We have proposed the following:

Observation 1: Ignoring any LBT failure would delay LBT failure counter to reach the threshold and that subsequently delays the action that UE is expected to take in the event of a persistent LBT failure event.

Observation 2: While gNB becomes aware if CSI report, HARQ feedback, and SRS are not present, it is the UE that has to take an action for a persistent LBT failure and withholding an occurrence of LBT failure (for these types of transmission) from MAC does impairs the ability of the UE to take a timely action.

Proposal 1: In order to aid a UE to act timelier in the event of persistent LBT failures, LBT failure during all types of uplink transmission should be accounted for.

Proposal 2: Consider a common “LBT status” signalling from PHY to MAC to enable MAC to recognize persistent LBT failure patterns. Inform RAN1 to accommodate such signalling.

Observation 3: Maintaining multiple counter/timer for each type of uplink transmission (e.g. SR, PUSCH and RACH) is unnecessary since the events that lead to an LBT failure is agnostic to the type of uplink transmission.

Proposal 3: Enable MAC to initiate and maintain a counter and a timer in order to detect the occurrence of a persistent LBT failure, where: a) the counter is incremented upon an LBT failure during each uplink transmission, b) the timer is reset after each LBT failure, c) upon expiry of the timer, the counter is reset.

Observation 4:  Statistically, persistent LBT failure for a higher priority LBT category/CAPC should also trigger persistent LBT failure for a lower priority LBT category/CAPC.

Proposal 4: Given higher priority that LBT Category 2 has, a persistent LBT failure should be (compared to a lower threshold and) acted upon in timelier manner.

Proposal 5: If RAN2 decides to have multiple counters/timers to identify persistent LBT failures, then it is important to consider a sum of LBT failures across the categories and to be compared with a separate threshold.

Proposal 6: In a wideband operation, given there is a possibility of varying number of competing devices per sub-band, allow persistent LBT failure events to be detected per sub-band.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

RAN2 #105b Chairman notes, Xi’an, China, RAN2#105b, April 2019