3GPP TSG-RAN WG2 Meeting #107	R2-1911575

Prague, Czech Republic, 26-30 August 2019

Source: 	Document Rapporteur (BlackBerry)

Title:	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC

Agenda Item:	12.1.1 and 12.2.1

Document for:	Email endorsement

# Introduction

This document lists RAN2 agreements made for Rel-16 Additional enhancements for NB-IoT (WI code NB_IOTenh3-Core; WID in RP-191513) and Rel-16 Additional MTC enhancements for LTE (WI code LTE_eMTC5-Core, WID in RP-191356), until and including the RAN2#107, 26-30 August 2019.

NOTE:	Email agreements for CRs/specific topics are not listed in this document.

# 1	Additional MTC enhancements for LTE

## 1.1	Organisational

RAN2#103bis agreements: To capture the agreements from all Rel-16 IoT meetings in the same document as agreed in Rel-16 NB-IoT session. The draft can be provided in R2-1815744. [103bis#xx][ eMTC/NB-IoT R16] To capture the agreements from the meeting in a document for information. (Blackberry) Intended outcome: Endorsed report Deadline: Thursday 2018-10-19 RAN2#104 to RAN2#107 agreements: None (except for this document, see clause 5).

## 1.2	Mobile-terminated (MT) early data transmission (EDT)

Treated jointly with 2.2 under this subclause.

RAN2#103bis agreements: RAN2 intends to support MT-initiated EDT for both CP and UP solutions. The intention to use MT-EDT is for user data, i.e. not for NAS signalling. RAN2#104 agreements: MT EDT are evaluated at least based on battery life, network resource efficiency, security, reliability and potential impact on core network. MT-EDT is intended for DL data which can be transmitted in one transport block. Use cases that require DL data transmission with or without UL data transmission as a response should be supported for MT-EDT. RAN2#105 agreements: DL data in paging message is excluded (Opt A). RNTI in paging message to schedule the DL data is excluded (Opt B). Working assumption: DL data scheduled, i.e. DL grant, in paging message is excluded (Opt C). Working assumption: DL data scheduled in paging occasion is excluded (Opt D). RAN2#105bis agreements: For both UP and CP solutions, an MT-EDT indication is needed in the S1 paging message to eNB. It is up to RAN3 to decide how such indication is provided to the eNB. RAN2 assumes that MME initiates MT-EDT. It is up to eNB to use MT-EDT based on e.g., UE capability. RAN2 assumes that DL data information is needed from S-GW/SCEF to the MME to assist MME to initiate MT-EDT. For the UP solution, the DL data are ciphered and sent over DTCH. For both UP and CP solutions eNB sends MT EDT indication to the UE via paging. For Msg-2 based solution (if agreed): A CF RACH resource is provided in the paging message to page the UE for MT-EDT. FFS whether/how security related concerns are addressed and how number of repetitions required and RNTI are provided. For Msg-4 based solution (if agreed): For UP solution, RRCConnectionResume is used in Msg4 in case UL transmission is expected in response. R2-1905276 LS on Mobile-terminated Early Data Transmission RAN2 LS out Rel-16 To:RAN3, SA2, CT1, and CT4. RAN2#106 agreements: DL data scheduled, i.e. DL grant, in paging message is excluded (Opt C). DL data scheduled in paging occasion is excluded (Opt D). DL data transmitted in Msg4 option B is excluded (Msg4-B). For Msg2-based solution (if agreed): No need to cipher the dedicated RACH resource provided in the paging message. RAN2 intends to introduce a mechanism to acknowledge that Msg2 was received by the intended UE. At least one dedicated RACH resource is provided in the paging message for MT-EDT. FFS what other parameters, e.g., CE level, subcarrier index, are needed to be provided. FFS: whether RNTI is provided in the paging message. For Msg4-based solution (if agreed): Working assumption: UE should be able to indicate in Msg3 if it intends to send data in the UL. RAN2#107 agreements: Msg2 based solution is not pursued in this release. Working assumptions: - For CP solution UE triggers RA using a Rel-15 EDT preamble. - For UP solution, UE may trigger RA using a non-EDT preamble. - For CP and UP solutions no new TB size for Msg3 is introduced. No indication is sent in Msg3 regarding whether an UL grant needs to be provided in Msg4, i.e. up to network implementation. In Msg4, same as MO-EDT, the network may move the UE to connected mode or release the UE to idle mode. FFS how to include user data in Msg4 for the CP solution if the UE is moved to connected mode. Fallback is supported in the case there is more downlink data to be sent, i.e. network can move the UE to connected mode in Msg4. From RAN2 stand point indicating the DL data size in the S1 paging message would be beneficial and sufficient, i.e. separate indication is not required R2-1911603 Reply LS on Mobile-terminated Early Data Transmission NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2, RAN3 Cc:SA3, CT1, CT4.

## UE-group wake-up signal (WUS)

See subclause 2.3 where this topic is treated jointly.

## Transmission in preconfigured resources

See subclause 2.4 where this topic is treated jointly.

## Scheduling multiple DL/UL transport blocks

Treated jointly with 2.5 under this subclause.

RAN2#103bis agreements: Working assumption: For MTCH in SC-PTM, configuration for multiple scheduling is transmitted in MCCH. Backwards compatibility is FFS. SPS is not supported for NB-IoT in Release 16. Enhancements for “SR with SPS for BSR” can be considered. RAN2#104 agreements: RAN2 intends to support separate/shared SC-MTCH transmission. Multiple TB scheduling is supported for UEs in connected mode. It is FFS if it is supported for EDT. UEs in connected mode are configured with multiple TB scheduling via dedicated RRC signalling assuming that a new DCI format is introduced. This is pending RAN1 agreement. UEs report capability to indicate support of multiple TB scheduling in connected mode. RAN2#105 to RAN2#105bis agreements: None. RAN2#106 agreements: Multi-TB scheduling for Msg4 during EDT (and MT-EDT) is not considered in Rel-16. For multi-TB scheduling, when HARQ ack bundling is configured, HARQ RTT timer is not started until the last TB has been received. FFS: For multi-TB scheduling for both LTE-M and NB-IoT, how to handle drx-InactivityTimer when HARQ Ack bundling is configured. UE capability for multiple TB is indicated separately for CE Mode A and CE mode B. UE capability for multiple TB is indicated separately for uplink and downlink. Wait for RAN1 before further discussion on multiple TB support for SC-PTM. RAN2#107 agreements: None.

## 1.6	Quality report in Msg3

RAN2#103bis agreements: Quality report in Msg3 is introduced for EDT. FFS for non-EDT. RAN2#104 agreements: RAN2 waits for progress on the discussion of MT-EDT before deciding on whether channel quality report in Msg3 is introduced for EDT. Channel quality report in Msg3 is introduced for non-EDT. RAN2#105 agreements: UE reports at most one DL quality measurement in Msg3 transmission. This is pending RAN1 agreement. For EDT, new MAC CE will be defined to report the channel quality in Msg3. FFS whether an LCID (lowest priority) or eLCID is used. Channel quality in Msg3 is reported with no explicit differentiation on whether the measurement was made in T1 or T2. RAN2#105bis agreements: eLCID based solution is not supported. A new LCID is used for eMTC pending approval in the main room. IoT informs the main room regarding the pending agreement above and the alternatives, e.g. to use one of LCID values reserved for NB-IoT or sidelink. Report from main session: Agreement made in IoT/MTC breakout session to use one of the reserved LCID values is confirmed. R2-1905277 LS on quality report in Msg3 for LTE-M RAN2 LS out Rel-16 To:RAN1 Cc:RAN4 RAN2#106 agreements: None. RAN2#107 agreements: Explicit signaling is introduced to trigger the aperiodic DL quality report in connected mode. FFS whether DCI or MAC CE. MAC CE is used to provide the aperiodic DL quality report in connected mode. DL quality report in Msg3 in idle mode is enabled via system information broadcast. UE capability reporting is not needed for the DL quality report in Msg3 in idle mode. UE capability reporting is introduced for the aperiodic DL quality report in connected mode.

## 1.7	MPDCCH performance improvement using CRS

RAN2#103bis agreements: eNB enables CRS for improving channel estimation on MPDCCH via broadcasted RRC signalling. RAN2#104 agreements: CRS for improving channel estimation on MPDCCH is configured via SIB2. This agreement can be revisited if RAN2 identifies that it would be more reasonable to configure it via SIB1-BR. It is FFS whether configuration would implicitly mean to enable the feature. UEs report capability to indicate support of using CRS for improving MPDCCH channel estimation. It is FFS if enabling/disabling is needed for the UE. RAN2#105 to RAN2#105bis agreements: None. RAN2#106 agreements: No need to introduce a separate indication to enable the feature. Configuration of CRS for MPDCCH performance improvement enables the feature implicitly for UEs in Idle mode. Working assumption: If dedicated configuration is not provided in connected mode, UE can use the broadcasted configuration if it exists. RAN2#107 agreements: None.

## 1.8	Improvements for non-BL UEs

RAN2#103bis agreements: RAN2 intends to support CMAS/ETWS for non-BL UEs in CE mode in connected mode. RAN2#104 agreements: None. RAN2#105 agreements: SIBs for ETWS and CMAS are not provided via dedicated signaling. RAN2#105bis agreements: Working assumption: In connected mode non-BL UEs in CE monitor MPDCCH to receive ETWS indication and/or CMAS indication using CSS in the same narrowband where unicast transmission can be received. R2-1905278 LS on ETWS/CMAS in connected mode narrowband RAN2 LS out Rel-16 To:RAN1 RAN2#106 agreements: None. RAN2#107 agreements: In connected mode, non-BL UEs in CE monitor MPDCCH to receive ETWS and/or CMAS notification using Type 0 CSS in the same narrowband where unicast transmission can be received. UE stays in RRC_CONNECTED after receiving the indication for ETWS/CMAS notification. It is up to UE implementation how ETWS/CMAS notification is acquired. RAN2 does not intend to introduce any signalling optimization regarding MIB, SIB1-BR and SIB10/11/12 acquisition. RAN2 understands that UEs in CE Mode B may not be able to acquire ETWS notification within 4 seconds requirement.

## 1.9	Stand-alone deployment

RAN2#103bis agreements: The standalone deployment supports legacy operations for legacy BL UEs or UEs in CE mode. LTE control channel region for DL transmission is supported for Rel-16 BL UEs and UEs in CE mode. RAN2#104 agreements: None. RAN2#105 agreements: Rel-16 non-BL UEs supporting CE mode can operate in CE mode in both idle and connected mode in standalone deployment regardless of whether criterion S for normal coverage is fulfilled. FFS if legacy non-BL UEs supporting CE mode can operate in CE mode in both idle and connected mode in standalone deployment regardless of whether criterion S for normal coverage is fulfilled. Introduce signalling in SIB1-BR to enable using the LTE control channel region for broadcast transmission assuming that only an indicator is needed. FFS: Introduce signalling in a dedicated message to enable using the LTE control channel region for DL unicast transmission. Introduce UE capability to indicate that UE supports unicast PDSCH reception in LTE control channel region. RAN2#105bis agreements: RAN2 agrees with the intention that non-BL UEs should be able to camp in a standalone cell when it is not possible to acquire SIB1. RAN2#106 agreements: None. RAN2#107 agreements: LTE Control Channel Region for DL transmission: FFS if, from Rel-16, it should be possible for a non-BL UE that fullfills S criteria for normal coverage to camp in a “normal” cell, i.e. not standalone, in enhanced coverage. This discussion will continue as part of the AI 12.1.8 Improvements for non-BL UEs.

## Mobility Enhancements

RAN2#103bis agreements: None. RAN2#104 agreements: An LS is sent to inform RAN1 that the overhead introduced by signalling agreed parameters per cell is excessive, and ask which parameters are carrier specific for potential optimization. Relaxation of RRM measurements for UEs using WUS: RAN2 will wait for RAN4 to progress. R2-1818662 LS on RSS based measurements signalling LS out Rel-16 To: RAN1. LTE_eMTC5-Core. RAN2#105 to RAN2#105bis agreements: None. RAN2#106 agreements: Add num-DRX-CyclesRelaxed to RadioResourceConfigCommon for eMTC. Capture the following in the corresponding running 36.331 CR: - “numDRX-CyclesRelaxed: Maximum number of consecutive DRX cycles during which the UE can use WUS for synchronisation and skip serving cell measurements, see TS 36.213 [23]. Value n1 corresponds to 1 DRX cycle, value n2 corresponds to 2 DRX cycles and so on. The actual duration during which the UE can skip serving cell measurements is: Min (signalled value x paging cycle, 2.56s), where paging cycle is the value of defaultPagingCycle parameter provided in SystemInformationBlockType2.” FFS if the value in Min() is 2.56sec. RAN2#107 agreements: None.

## Coexistence with NR

RAN2#103bis to RAN2#107 agreements: None.

## Connection to 5GC

RAN2#105 agreements: R2-1902436 Response LS on Completion of Study on Cellular IoT support and evolution for the 5G System LS out Rel-16 NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2. RAN, RAN3. Cc:SA, CT. R2-1902431 Response LS on on eDRX cycles for CM-CONNECTED with RRC inactive LS out Rel-16 NB_IOTenh3-Core, LTE_eMTC5-Core To:SA2, CT1, CT4, RAN3. RAN2#105bis agreements: Support of eDRX in CM-IDLE and EDT: Working assumption: Introduce a new IE “cp-EDT-5GC-r16” in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports CP-EDT optimization. Rel-15 eMTC/NB-IoT EDT PRACH Configuration parameters can be commonly used for both EPC and 5GC. Use critical extension for RRCEarlyDataRequest/RRCEarlyDataRequest-NB messages for CP-EDT eMTC/NB-IoT UEs connecting to 5GC. Introduce 48-bit 5G S-TMSI in critical extension of RRCEarlyDataRequest /RRCEarlyDataRequest-NB messages. For eMTC, in critical extension of RRCEarlyDataRequest message, add establishment cause of mo-Data. For NB-IoT, in critical extension of RRCEarlyDataRequest-NB message, add establishment cause of mo-Data and mo-ExceptionData. Working assumption: For eMTC/NB-IoT, delayTolerantAccess is not applicable for 5GC. Update stage 2 spec to capture that idle mode eDRX feature is supported for eMTC and NB-IoT connected to 5GC. For eMTC, hyperSFN in SIB1-BR is commonly used for both EPC and 5GC. For NB-IoT, hyperSFN-LSB in MIB-NB and hyperSFN-MSB in SIB1-NB are commonly used for both EPC and 5GC. For eMTC connected to 5GC, introduce a flag in SIB1-BR to indicate if idle mode eDRX is allowed in the cell. Working assumption: No indication of support for idle mode eDRX is needed for NB-IoT connected to 5GC. For eMTC and NB-IoT, systemInfoModification-eDRX is commonly used in paging messages for both EPC and 5GC. Use 5G S-TMSI as input for Hash ID to calculate PH and PTW_start. Support of RRC_INACTIVE and eDRX in CM-CONNECTED: RRC_CONNECTED Mode extended long DRX cycle, e.g. 5.12 and 10.24 sec, is supported for eMTC connected to 5GC. FFS whether new optional UE capability is required. RRC_CONNECTED Mode long DRX cycle, e.g., 5.12 and 10.24 sec, is supported for NB-IoT connected to 5GC. Mandatory to support without new UE capability. For eMTC, eDRX-Config-CycleStartOffset and drx-StartOffset are commonly used for both EPC and 5GC. For NB-IoT, DRX-Cycle and drx-StartOffset are commonly used for both EPC and 5GC. RAN2#106 agreements: Support of RRC_INACTIVE and eDRX in CM-CONNECTED: RRC_INACTIVE state in NB-IoT connected to 5GC is not supported. RRC_INACTIVE state with short eDRX cycles is optionally supported for eMTC connected to 5GC with capability signalling. FFS if EDT in RRC_INACTIVE state is not supported in Rel-16. UP optimization solution is supported for both eMTC and NB-IoT connected to 5GC with capability signalling. eLTE is assumed to be the baseline for RRC_INACTIVE state for eMTC UEs connected to 5G. Working assumption: Same cell selection/reselection criteria and measurement rules for BL UEs and non-BL UEs in CE in RRC_IDLE when attached to EPC are applied to BL UEs and non-BL UEs in CE in RRC_INACTIVE or RRC_IDLE when attached to 5GC. For transmission over DRB, NB-IoT/eMTC UE connected to 5GC uses the same AS security algorithms as eLTE regardless of PDCP type. KeNB root key is derived from Kamf as specified in TS 33.501. For NB-IoT/eMTC UE connected to 5GC, AS security algorithms are indicated by using LTE code points, e.g., EIA, EEA. R2-1908202 Reply LS on Conclusion on 5G CIoT user plane small data solution LS out Rel-16 FS_CIoT_5G To:SA2, RAN3 Cc:RAN, SA, CT RAN2#107 agreements: Support of eDRX in CM-IDLE and EDT: Support of User Plane CIoT 5GS optimization is optional for both eMTC and NB-IoT devices connected to 5GC without capability signaling. Indication for support is provided in Msg5, i.e. RRCConnectionSetupComplete. For NB-IoT and eMTC, introduce a new parameter, up-CIoT-5GS-Optimisation-r16, in SIB1-BR/SIB1-NB to indicate per PLMN support of User plane CIoT 5GS optimisations. For eMTC, broadcast signaling is not required to indicate support of N3 data transfer since it is mandatory for both the UE and the network. For NB-IoT, introduce a new parameter, n3-5GS-DataTransfer-r16, in SIB1-NB to indicate per PLMN support of N3 user plane data transfer. For eMTC, introduce new parameters, up-CIoT-5GS-Optimisation-r16 and cp-CIoT-5GS-Optimisation-r16 in RRCConnectionSetupComplete when accessing 5GC. For NB-IoT, introduce new parameters, up-CIoT-5GS-Optimisation-r16 and n3-5GS-DataTransfer-r16 in RRCConnectionSetupComplete-NB when accessing 5GC. For NB-IoT and eMTC, when the UE in RRC_IDLE using the UP CIoT optimisation reselects to another CN type, it discards the UE AS context and the UE identity. For NB-IoT and eMTC, UE goes to RRC_IDLE and initiates NAS recovery procedure when the UE switches between CN types at RRC Connection re-establishment. Introduce a new IE up-EDT-5GC-r16 in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports UP MO-EDT. Introduce a new UE capability earlyData-UP-5GC-r16 in UE-EUTRA-Capability, UE-Capability-NB to indicate support of UP MO-EDT in 5GC. Other: From RAN2 standpoint it is feasible to introduce LTE-M indication in RRCConnectionSetupComplete message if UE selected 5GC. R2-1911601 Reply LS on LTE-M identification in 5GC LS out Rel-16 5G_CIOT To:SA2, RAN3 Cc:RAN

## Other

RAN2#103bis to RAN2#107 agreements: None.

# Additional enhancements for NB-IoT

## Organisational

RAN2#103bis agreements: To create document agreed after each meeting capturing agreements from all meetings. Copy/paste of chair notes, not intended to be a further discussion of agreements. One week email agreement after each meeting BlackBerry will be rapporteur of the document. RAN2#104 to RAN2#107 agreements: None (except for this document, see clause 5).

## Mobile-terminated (MT) early data transmission (EDT)

See subclause 1.2 where this topic is treated jointly.

## UE-group wake-up signal (WUS)

Treated jointly with 1.3 under this subclause.

RAN2#103bis agreements: The aim of UE grouping for WUS is reducing the false alarm probability. At least UE_ID based grouping is supported for UE-Group based WUS. This doesn’t exclude other options. Further discuss whether the following are supported: - Service based grouping - DRX/eDRX based grouping - Gap based grouping. - Coverage based grouping RAN2#104 agreements: Further discuss the benefit and feasibility of using service based parameters for grouping in addition to UE-ID. Can discuss group distribution further, including Rel-15/16 mechanism interaction, once we know more about number of groups and more about the grouping solution (e.g. service based parameters). RAN2 will decide on the UE to WUS group mapping. RAN2#105 agreements: Feasibility of the solution based on the following attributes for deriving the service-type for GWUS can be studied further: - Paging Probability - Mobility. RAN2#105bis agreements: Additional grouping based on DRX/eDRX is not supported. Coverage based grouping is not supported. Additional grouping based on gap is not supported. - FFS whether number of groups can depend on gap duration. RAN2#106 agreements: WUS grouping based on paging probability is beneficial for improving false wake-up probability for UEs which are not frequently paged, but may increase false wake-up probability for UEs which are frequently paged. Send an LS to SA2, cc RAN3, CT1, to ask about the feasibility and impact for the MME to provide information on UE paging probability. R2-1908260 LS on assistance indication for WUS grouping LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:SA2 Cc: CT1, R3 RAN2#107 agreements: Paging probability information is negotiated between the UE and MME via NAS signalling. This paging probability information needs to be provided by S1 paging. eNB configures, via broadcast, the relation between this paging probability information and WUS group on Uu interface. R2-1911589 Reply LS on assistance indication for WUS LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To: SA2,CT1, RAN3

## Transmission in preconfigured resources

Treated jointly with 1.4 under this subclause.

RAN2#103bis agreements: Transmission in dedicated preconfigured uplink resources in IDLE mode is supported for UEs with a valid timing advance. Initially we will focus on dedicated preconfigured uplink resources in idle mode - Shared resources can also be discussed RAN2#104 agreements: The eNB configures the dedicated preconfigured uplink resources via RRC dedicated signaling. Methods for eNB to obtain information used to help configuring the dedicated preconfigured uplink resource to the UE is FFS. Periodic D-PUR with duration is supported FFS if one shot D-PUR is supported. Release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS. RAN2#105 agreements: Multi-shot D-PUR is supported with the possibility to configure as a single shot. UE may perform a D-PUR request/information, if D-PUR is indicated as enabled in the cell. Network makes the decision on the D-PUR configuration. Request/information can include: - Requested TBS - Requested periodicity - Other information FFS. The eNB can (re)configure and release D-PUR by dedicated RRC signalling. D-PUR configuration is released when the eNB doesn’t detect “m” consecutive UE transmissions. The UE must release the D-PUR when it does a RA procedure on a new cell. D-PUR configuration can be set up without a pre-defined end (infinite). RAN2#105bis agreements: From RAN2 point of view it is possible to configure TBS for D-PUR for both NB-IoT and eMTC up to the maximum supported based on the UE category and TBS capability. For UP the UE may transmit D-PUR release request/(re)configuration request when transmitting using D-PUR. FFS For CP. The following are FFS: Whether a UE can have more than one parallel D-PUR configurations Ability of the UE to request (indicate) a time offset The range of values and limits of preconfigured D-PUR allocations. Whether mechanisms for a UE to reject a D-PUR configuration including by explicitly or implicitly releasing are needed. RAN2#106 agreements: RRC response message needs to be supported by the UE and could be used in all cases. For some cases L1 signalling is sufficient to acknowledge, i.e. RRC response message is not needed. RAN2 assumes the L1 signalling for acknowledgement is sent only after the eNB determines there is no pending downlink data or signalling. RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling, and should check with RAN1 what parameters are planned to be (re)configured using DCI. It is feasible to provide the UE with a UE-specific RNTI for D-PUR. Common or shared RNTI is also feasible. The RNTI used for D-PUR is signalled together with other D-PUR configuration. R2-1908196 Reply LS on PUR Working Assumptions for NB-IoT and LTE-M LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To: RAN1. RAN2#107 agreements: D-PUR procedural steps: Valid TA is a requirement in order to initiate D-PUR transmission. The UE may use the D-PUR resource to send RRCConnectionRequest or RRCConnectionResumeRequest to establish or resume RRC connection. - FFS: whether the UE can send part of the data using the padding in this case. FFS: whether the UE can segment and send part of the data using the D-PUR resource. For the CP solution, the uplink data are encapsulated as a NAS PDU in an uplink RRC message transmitted in CCCH. For the UP solution, the uplink data are transmitted in DTCH. After the uplink D-PUR transmission, the UE monitors PDCCH under the control of a timer: - The timer starts after D-PUR transmission. - The timer restarts if a scheduling for D-PUR retransmission is received. - The UE considers that the D-PUR transmission has failed if the timer expires. - The timer is stopped when D-PUR procedure ends/succeeds. The downlink RRC response message, if needed, for the CP solution may include the following optional information: - downlink data encapsulated as a NAS PDU (downlink application layer response or pending data in MME). - redirection information. - D-PUR (re-)configuration and release. - FFS extendedWaitTime. The downlink RRC response message for the UP solution may include the following optional information: - Resume ID. - NCC (mandatory) - the downlink RRC response message for the UP solution is always provided. - redirection information. - D-PUR (re-)configuration and release. - FFS extendedWaitTime. The MAC CE for TA update can be sent along with the RRC transmission of the downlink RRC response message for the CP solution and UP solution. - FFS for CP solution if MAC CE for TA update can be sent without a downlink RRC response message. After reception of D-PUR transmission, the eNB can move the UE to RRC connection by RRCConnectionSetup message or RRCConnectionResume message. Fallback after D-PUR transmission is not successful is not specified i.e. it is up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion. - FFS how to handle the skip in case of failure (UL or DL). D-PUR TA validation criteria: TA validation criterion “Serving cell changes” is implicitly always enabled, which means that TA is considered invalid when the UE initiates RA procedure in a different cell than where TA was last validated. Configuration for TA validation criteria is provided in dedicated RRC signaling. - It should be possible to disable each or all of the optional TA validation criteria (i.e., TA timer, (N)RSRP change) via RRC signaling. UE keeps the PUR configuration while TA is considered invalid, but PUR cannot be used until eNB validates the existing TA/provides a new TA. Working assumption: Counter for D-PUR occasions, i.e., “n”, is not introduced and “indefinite” or “one-shot” are the only possible configurations. A new TA timer is defined for UEs configured with D-PUR in idle mode. - The (re)starting times for TA timer need to be aligned between UE and eNB. The details of the mechanism are FFS. - TA timer is restarted after TA is updated. - The value range for the TA timer is FFS. Value of “infinity” is possible. D-PUR Request, (re)configuration and release mechanism: D-PUR request can be sent only by BL UE, UE in CE or NB-IoT UE; and which are capable of D-PUR. D-PUR request can be sent when the UE is in RRC_CONNECTED. D-PUR request includes number of PUR grant occasions requested with possibility to request infinite. FFS other values. UE can request D-PUR release. FFS how. A new RRC message is introduced for transmission of PUR request when UE is in RRC_CONNECTED (i.e., not for the cases of sending PUR request during EDT and during PUR). UE-specific PUR (re)configuration can be provided while UE is in RRC_CONNECTED. PUR (re)configuration can be included in RRC Connection Release. At least the following information can be included in PUR (re)configurations: - “m” consecutive missed allocations before release, FFS values. - Time Alignment Timer for idle mode. - RSRP change threshold for Serving cell. For UP solution, when PUR request is being piggybacked in the PUR transmission, same RRC message used for PUR transmission is used to include PUR request. PUR (re)configuration can be provided in DL RRC response message (message FFS) of the D-PUR procedure. Explicit reject message (NW-> UE) in response to PUR request is not introduced. Delta configuration is supported for PUR reconfiguration. If the UE performs EDT or moves to RRC_CONNECTED and comes back to RRC_IDLE in the same cell, PUR configuration remains valid unless specifically released or reconfigured by network or other triggers. PUR can be released explicitly by RRCConnectionRelease message and DL RRC response message (FFS message) of the D-PUR procedure. - FFS: RRCEarlyDataComplete. FFS: When UE initiates RACH/EDT, whether it has D-PUR configuration(s) is not explicitly notified to the network. EDT cannot be initiated solely for the purpose of sending PUR request in EDT Msg3. UE is not restricted from initiating RRC Connection for the purpose of sending PUR request (i.e. this agreement has no impact to legacy RRC Connection Establishment / Resume procedures). L1 ACK: RAN2 confirm the intention of the previous agreement as follows: - If RRC response message is not needed, eNB may send L1 ACK to acknowledge the PUR transmission in UL. The L1 ACK concludes the PUR procedure and UE moves to Idle.

## 2.5	Scheduling multiple DL/UL transport blocks

See subclause 1.5 where this topic is treated jointly.

## 2.6	Network management tool enhancement

RAN2#103bis agreements: ANR reporting for NB-IoT only uses idle-mode measurements (i.e. we won’t introduce connected mode measurements) Support RACH report for NB-IoT Will consider whether information in LTE RACH report is extended RAN2#104 agreements: SON-ANR: RAN2 understanding is that the purpose of SON/ANR reporting in NB-IoT is network optimisation rather than immediately updating neighbour relations like with LTE ANR, and is therefore not time critical. SON reporting does not trigger RRC connection establishment/resume - FFS whether this includes EDT. SON information can be reported along with EDT, FFS what and how. - RACH report: In addition to legacy parameters for RACH reporting, the first selected resource pool (E.g. CE level, EDT) is included in the RACH report. This agreement is also applicable for eMTC. RAN2#105 agreements: - RLF report: The UE informs network about having an RLF report available via rlf-InfoAvailable indication during the initial RRC connection re-establishment process after RLF. The rlf-InfoAvailable parameter is included in Msg5 of the RRC connection re-establishment process. It is up to the network to decide when to request for the RLF report, using UEInformationRequest/UEInformationResponse procedure. In case of no/failed RRC connection re-establishment, the UE may store the RLF report for indicating rlf-InfoAvailable in Msg5 of later connection establishment. The following legacy parameters are supported for RLF reporting in NB-IoT: - Failed cell ECGI - Last Serving Cell RSRP/RSRQ (Whether OK for CP-solution is FFS) - Absolute Time Stamp (optional, if available) - Location Info (optional, if available) - FFS: further information. RLF reporting does not trigger RRC connection establishment/resume, including EDT. Neither rlf-InfoAvailable indication nor RLF report is included in the Msg3 of the EDT procedure to indicate to the network about an available RLF report. - RACH report: For NB-IoT RACH report, range of numberOfPreamblesSent is INTEGER (1..64). contentionDetected is Boolean. Initial CEL is included in NB-IoT RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. Take the ASN.1 structure provided in the report (proposal 5) as baseline for NB-IoT running CR. Initial CEL is included in eMTC RACH report. Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. Take the ASN.1 structure provided in the report (proposal 8) as baseline for eMTC running CR. RAN2 aims on standardizing a single method of sending NB-IoT RACH report, i.e., avoid multiple solutions. Proposed solutions should be discussed considering signalling overhead, UE power consumption, security aspects and other reports being considered in other email discussions. NB-IoT RACH report includes the information from the latest successful RACH procedure. NB-IoT RACH report information is discarded when the UE goes to IDLE. FFS how to report. - SON-ANR: Solution direction based on option a: - Single set of measurements only. - No new measurement requirements. - ANR measurement reporting using the UE Information Request / Response framework is supported. Other methods FFS. - ANR reporting for the CP solution is not supported in Rel-16. RAN2#105bis agreements: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. At least RACH report for NB-IoT based on UE information request/response procedure is supported. It is up to the network to decide when to request for the RACH report. RAN2#106 agreements: - SON-ANR: ANR report does not trigger EDT. ANR report is not carried in EDT Msg3. anr-InfoAvailable indication can be reported in EDT Msg3. ANR procedure steps: - 1) UE receives “measurement” configuration in dedicated signalling. - 2) When the UE is released to idle: The UE performs the instructed measurements according to existing RAN4 cell reselection measurement performance requirements. CGI reading is limited to cells being above a certain threshold. Threshold details FFS. 3) Reporting takes place, upon network request, next time UE is connected mode again. - RACH report: Not support EDT RACH report in EDT Msg3. RAN2#107 agreements: - SON-ANR: - Configuration: Dedicated ANR measurement configuration is provided in RRCConnectionRelease message only. The list of frequencies where to perform the measurements can be optionally provided in dedicated signalling, as a subset of the frequencies signalled in system information. Per default, the list frequencies signalled in system information is used. Maximum of 8 carriers can be configured. A blacklist of cells can be optionally signalled per carrier. - Measurements: ANR measurements are only performed while UE remains camped on the same cell from which the ANR measurement configuration was received. (i.e. When the UE changes cell the ANR measurements are stopped). Neighbour cell relaxed monitoring criteria and Ssearch criteria don’t apply while performing an ANR measurement. ANR measurement doesn’t affect DRX / eDRX operation. CGI-reading applies to strongest cell on each frequency, if above the RSRP threshold. The RSRP threshold is an absolute threshold that is common to all frequencies. - Reporting: anr-InfoAvailable indication is reported in and only in RRCConnectionSetupComplete, RRCConnectionResumeComplete, RRCConnectionReestablishmentComplete and RRCConnectionResumeRequest for EDT. The ANR report might be empty for a given carrier, if a measurement is performed and no strong cell is found. UE only indicates the availability of the report if 1) a report is available and 2) if the RPLMN is included in the list of EPLMNs stored by the UE when the ANR measurement was configured. The ANR report is discarded after: - Timer expiry. Value FFS. - Power off or detach. - Reporting anr-InfoAvailable and returning to idle. UE Information Request / Response is the only reporting procedure for ANR measurement. For a measured carrier the following information is included: - Carrier frequency. - Strongest cell PCI, RSRP/RSRQ. - Strongest cell CGI-info if RSRP above the threshold. Serving cell RSRQ/RSRP and CGI are included in the ANR report. ANR reporting is optional for NB-IoT UEs with capability signaling. FFS: ANR applicability to 5GC.

## 2.7	Improved multi-carrier operation

RAN2#103bis agreements: None. RAN2#104 agreements: Re-use the code points defined in Rel-14. Study the impact of re-using the Rel-14 RRC reporting mechanism and consider whether a MAC mechanism should be used instead. RAN2 further study how to support the use case of enabling measurements in non-anchor carrier while reducing measurement on anchor carrier. RAN2#105 agreements: None. RAN2#105bis agreements: - Channel Quality Reporting: Existing Msg3 signalling is used for reporting downlink channel quality when UE makes access on non-anchor carrier. Existing SIB2 signalling is used to enable UE to report downlink channel quality on non-anchor carrier. The DL channel quality reported in connected mode corresponds to the carrier used for the unicast transmission (i.e. configured by MSG4 or by a subsequent reconfiguration procedure). eNB enables the reporting of the DL Channel quality in connected mode. - FFS whether dedicated or broadcast signaling is used for enabling. Periodic reporting or on-demand reporting are not supported. DL channel quality reported in connected mode is optional for the UE. Working assumption that the same code points as Msg3 reporting is used. FFS whether MAC or RRC is used for reporting. R2-1905263 LS on non-anchor carrier CQI reporting in MSG3 Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN1 Cc: RAN4 - Non-anchor carrier measurements for RRM: If RAN4 confirms feasibility, and NRS is present in non-anchor carrier, RAN2 can make use of that for RRM measurements in non-anchor carrier. R2-1905264 LS on non-anchor carrier idle mode measurements for RRM Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc: RAN1 RAN2#106 agreements: - Channel Quality Reporting: The DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE. Use the same LCID as agreed for eMTC. We need to wait for RAN4 before deciding when to report. FFS whether broadcast or dedicated signalling is used for NW to enable the reporting. R2-1908262 LS on DL channel quality report for configured carrier in RRC connected mode LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc:RAN1 RAN2#107 agreements: - Channel Quality Reporting: Quality report in connected mode is transmitted on demand. No implicit trigger is supported. The trigger command is defined as a MAC CE with empty payload and one of the reserved LCID is used. No feature enable indication is defined in RRC signalling. UE capability signalling for quality reporting in connected mode is defined. RAN2 confirm that DL Channel quality in MSG3 applies to initial access, i.e. when MAC MSG3 contain a CCCH SDU, and that the report corresponds to the carrier where the UE performs the random access procedure. From RAN2 point of view, there is no need for a physical layer trigger for DL channel quality in connected mode. From RAN2 point of view, there is no need for DL channel quality reporting (for RAR or unicast carrier) in MSG3 in connected mode. CCCH SDU in the MSG3 buffer needs to be updated after receiving RAR. Support of DL channel quality in MSG3 for non-anchor carrier is optional without capability reporting and is a separate capability from support of DL channel quality in MSG3 for the anchor carrier. Use the codepoint/index of “10001” for the MAC CE DL channel quality report. Introduce a one-byte MAC CE DL channel quality report to support a maximum of 8 bits report. FFS: The new MAC CE DL channel quality report has the next priority after MAC CE for BSR. When the measurement is available, the MAC CE DL channel quality report is sent at the first opportunity according to the MAC logical channel prioritisation rules. Support of DL channel quality in MSG3 for non-anchor carrier and support of DL channel quality in connected mode also apply when the UE is accessing via 5GC. R2-1911587 Reply LS on channel quality report in connected mode for NB-IoT LS out Rel-16 NB_IOTenh3-Core To:RAN1, RAN4

## 2.8	Inter-RAT cell selection

RAN2#103bis agreements: RAN2 understands that cell reselection between NB-IoT and any other RAT is not within the scope of this WI NB-IoT network may indicate eMTC/LTE/GERAN assistance information for inter-RAT cell selection eMTC/LTE network may indicate NB-IoT assistance information for inter-RAT cell selection Intention is not to provide cell reselection parameters It is up to UE implementation how and when to perform measurements for cell selection of the target RAT RAN2#104 agreements: NB-IoT network may indicate frequency identifiers of neighbouring eMTC/LTE/GERAN carriers to assist inter-RAT selection. eMTC/LTE network may indicate frequency identifiers of neighbouring NB-IoT carriers to assist inter-RAT selection. Indicate whether each LTE neighbour frequency supports eMTC/LTE/both. Indicate whether each GERAN neighbour frequency supports EC-GSM/PEO. RAN2#105 agreements: Priorities of eMTC/LTE/GERAN frequencies are not provided by NB-IoT network as assistance information for inter-RAT cell selection. Priorities of NB-IoT frequencies are not provided by eMTC/LTE network as assistance information for inter-RAT cell selection. Suitability criteria of eMTC/LTE/GERAN frequencies are not provided by NB-IoT network as assistance information for inter-RAT cell selection. Suitability criteria of NB-IoT frequencies are not provided by eMTC/LTE network as assistance information for inter-RAT cell selection. RAN2#105bis to RAN2#106 agreements: None. RAN2#107 agreements: In NB-IoT and in LTE, a new System Information block should be defined for IRAT cell selection assistance information containing the parameters agreed previously. - The normal SI update procedures apply. R2-1911583 IRAT Cell Selection Assistance information LS out Rel-16 NB_IOTenh3-Core To:RAN6

## 2.9	Coexistence with NR

RAN2#103bis to RAN2#107 agreements: None.

## 2.10	Connection to 5GC

RAN2#105 agreements: See subclause 1.12 RAN2#105bis agreements: For NB-IoT: SIB1-NB extended to include 5GC PLMN list, per PLMN indication of Cell Reserved for Operator Use, common 5GC Tracking Area Code and 5GC Cell Identity across all PLMNs, common Cell Barring for 5GC connectivity across all PLMNs. For NB-IoT/eMTC: SIB-NB/SIB1-BR extended to include per PLMN indication of the supported CIoT 5GS Optimisations. For NB-IoT: Update Paging-NB message to include 5G S-TMSI as UE Identity for core network paging. For NB-IoT: Adopt critical extension of RRCConnectionRequest-NB message for 5GC connectivity and include 48 bit 5G S-TMSI and random value as Initial UE identity along with specific RRC establishment cause. For NB-IoT: Update RRCConnectionSetupComplete-NB to include RegisteredAMF-r15, full 5G S-TMSI (48 bit long). For NB-IoT/eMTC: FFS :Whether s-NSSAI-list-r15 is applicable. For NB-IoT/eMTC: FFS: Applicability of NR PDCP for SRB1, DRBs For eMTC: UAC feature is supported. For NB-IoT: FFS how to support access control. For NB-IoT: FFS how to support slicing. For NB-IoT: FFS whether to adopt SDAP as user plane protocol, and whether AS reflective QoS is applicable. For eMTC: Adopt SDAP as user plane protocol, and AS reflective QoS is optional. For NB-IoT/eMTC: Working assumption that CN type is not used in RRCConnectionRelease/ RRCConnectionRelease-NB. For NB-IoT: How to support RRC Connection Re-establishment for the UP and CP optimization. For NB-IoT/eMTC: FFS whether for data transfer through DRBs, use AS security algorithms same as LTE AS security algorithms, and KeNB root key is derived from Kamf as specified in TS 33.501. For NB-IoT/eMTC: FFS whether AS security algorithms are identified by using LTE code points. For eMTC: mt-Access, mo-Signaling, mo-Data, emergency, highPriorityAccess and mo-VoiceCall are applicable establishment causes. For NB-IoT/eMTC: Release cause loadBalancingTAURequired at RRC connection release is not applicable. RAN2#106 agreements: Indication of supported CIoT features and other common aspects: RAN2 agrees to send a LS to SA2, CC: CT1,RAN3 - ask how many slices NB-IoT/eMTC UEs can support simultaneously and whether S-NSSAI will be used by the ng-eNB in addition to the indications of the supported 5G CIoT optimisations for AMF selection. - ask about applicability 5GC QoS frame work for NB-IoT/5GC - ask how QoS mapping is handled during mobility between NB-IoT and other RATs. NB-IoT UE AS forwards all CIoT feature support indications broadcast in the cell to NAS as assistance info for CN type selection by NAS. eMTC UE AS forwards all CIoT feature support indications broadcast in the cell to NAS as assistance info for CN type selection by NAS. For NB-IoT, CN Type is not used in RRCConnectionRelease-NB message as NAS based redirection between EPC and 5GC is used for CIoT. eMTC re-uses the existing CN type in RRCConnectionRelease message For eMTC intra RAT connected mode mobility (for intra 5GC case), adopt R15 LTE connected to 5GC introduced changes for RRCConnectionReconfiguration (including Mobility Control Info) including SecurityConfigHO-v1530 -> intra5GC-r15 IE. For eMTC intra RAT connected mode mobility (for inter system mobility between EPC and 5GC), adopt R15 LTE connected to 5GC introduced MobilityFromEUTRACommand carrying RRCConnectionReconfiguration (including Mobility Control Info) including SecurityConfigHO-v1530 -> fivegc-ToEPC-r15 and epc-To5GC-r15 IEs. For eMTC, re-use SIB25 and the procedure in subclause 5.3.16 of 36.331 as a baseline. Ask SA1: - whether UAC needs to be supported for NB-IoT. - whether a new UAC access category corresponding to mo-ExceptionData can be defined as one of the reserved standardised access categories in the range 9-31. - Whether up to 64 access categories need to be defined for support of UAC, given the currently supported establishment causes and necessity for operator defined categories and potential future extension - Whether access classes 0-9 (mapped to access identity 0) are applicable or not - Whether barring for access identities 1, 2, 11 to 15, is common for all access categories or independently controlled for different access categories. Use the same Enhanced Coverage authorisation mechanism of UEs connected to EPC to the NB-IoT/eMTC UEs connected to 5GC and no impacts on RAN2 specification are identified. R2-1908190 LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:SA2 Cc:CT1, RAN3. R2-1908263 LS on UAC for NB-IoT LS out Rel-16 NB_IOTenh3-Core, 5G_CIoT To:SA1, CT1 Cc:SA2, RAN3 Other: RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is supported, assuming feasibility is confirmed. Send a LS to SA3, SA2 CC: RAN3, CT1, CT4 - Ask whether they can introduce RRC Connection re-establishment for the Control plane for NB-IoT connected to 5GC. - Ask the feasibility that a truncated 5G S TMSI (e.g. 40 bits) needs to be defined. R2-1908264 LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC LS out Rel-16 NB_IOTenh3-Core, 5G_CIoT To:SA3, SA2 Cc:CT1, RAN3, CT4 RAN2#107 agreements: Indication of supported CIoT features and other common aspects: SDAP is not supported for NB-IoT. Maximum 8 slices are supported by both NB-IoT and eMTC. In Msg5, both NB-IoT and eMTC UEs provide S-NSSAI to indicate network slice that UE wants to connect. AS reflective QoS is not supported for NB-IoT connected to 5GC. NB-IoT UEs connected to 5GC support a maximum of 2 DRBs. Each NB-IoT DRB is mapped to a default QoS flow of that PDU session. Control Plane optimization feature is optional for eMTC devices connecting to 5GC and mandatory for NB-IoT devices connected to 5GC. Introduce a new IE “cp-CIoT-5GC-Optimisation-r16” in SIB1-BR to indicate ng-eNB connected to 5GC is supporting control plane optimisation and no need of any indicator in SIB1-NB to indicate support for CP optimisation. When eMTC UE is initiating RRC connection setup procedure for connecting to 5GC, use R15 critical extension of RRCConnectionRequest message including 5GC UE Identity and 5GC establishment cause values. In R15, RRC Connection Setup Complete Message is enhanced to include registeredAMF-r15, s-NSSAI-list-r15, ng-5G-S-TMSI-Bits-r15, gummei-Type-v1540, guami-Type-r15 and all these changes are applicable for eMTC 5GC UEs as well. In R16 NB-IoT, RRCConnectionSetupComplete-NB message needs also to include gummei-Type-v16, guami-Type-r16. In RRCConnectionSetupComplete message, introduce a new IE “cp-CIoT-5GC-Optimisation-r16” to assist ng-eNB for the selection of proper AMF supporting control plane optimization. Working assumption: We will introduce a AS Release Assistance Information (RAI) that conveys at least the following information: - No further UL/DL higher layer PDUs. - No further UL, 1 single DL higher layer PDU expected. - Further details including CP/UP applicability can be discussed in the next meeting. R2-1911865 Reply LS on UAC for NB-IoT LS out Rel-16 5G_CIOT, NB_IOTenh3-Core To:SA1; Cc:CT1, SA2, RAN3

## 2.11	Other

RAN2#103bis to RAN2#107 agreements: None.

# References: List of approved outgoing Rel-16 LSs

| Tdoc | Meeting | Title | WI | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-1818662 | RAN2-104 | LS on RSS based measurements signalling | LTE_eMTC5-Core | R1 |  |
| R2-1902431 | RAN2-105 | Response LS on on eDRX cycles for CM-CONNECTED with RRC inactive | FS_CIoT_5G | SA2, CT1, CT4, RAN3 |  |
| R2-1902436 | RAN2-105 | Response LS on Completion of Study on Cellular IoT support and evolution for the 5G System | FS_CIoT_5G | SA2, RAN, R3 | SA, CT |
| R2-1905263 | RAN2-105bis | LS on non-anchor carrier CQI reporting in MSG3 | NB_IOTenh3-Core | R1 | R4 |
| R2-1905264 | RAN2-105bis | LS on non-anchor carrier idle mode measurements for RRM | NB_IOTenh3-Core | R4 | R1 |
| R2-1905276 | RAN2-105bis | LS on Mobile-terminated Early Data Transmission | LTE_eMTC5, NB_IOTenh3 | R3, SA2, CT1, CT4 |  |
| R2-1905277 | RAN2-105bis | LS on quality report in Msg3 for LTE-M | LTE_eMTC5-Core | R1 | R4 |
| R2-1905278 | RAN2-105bis | LS on ETWS/CMAS in connected mode narrowband | LTE_eMTC5-Core | R1 |  |
| R2-1908190 | RAN2-106 | LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2 | CT1, R3 |
| R2-1908196 | RAN2-106 | Reply LS on PUR Working Assumptions for NB-IoT and LTE-M | LTE_eMTC5-Core, NB_IOTenh3-Core | R1 |  |
| R2-1908202 | RAN2-106 | Reply LS on Conclusion on 5G CIoT user plane small data solution | FS_CIoT_5G | SA2, R3 | RAN, SA, CT |
| R2-1908260 | RAN2-106 | LS on assistance indication for WUS grouping | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2 | CT1, R3 |
| R2-1908262 | RAN2-106 | LS on DL channel quality report for configured carrier in RRC connected mode | NB_IOTenh3-Core | R4 | R1 |
| R2-1908263 | RAN2-106 | LS on Access Categories for NB-IoT | NB_IOTenh3-Core, 5G_CIoT | SA1, CT1 | SA2, R3 |
| R2-1908264 | RAN2-106 | LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC | NB_IOTenh3-Core, 5G_CIoT | SA3, SA2 | CT1, R3, CT4 |
| R2-1911583 | RAN2-107 | LS to RAN6 on IRAT Cell Selection Assistance information | NB_IOTenh3-Core | R6 |  |
| R2-1911587 | RAN2-107 | Reply LS on channel quality report in connected mode for NB-IoT | NB_IOTenh3-Core | R1, R4 |  |
| R2-1911589 | RAN2-107 | Reply LS on assistance indication for WUS | LTE_eMTC5-Core, NB_IOTenh3-Core | SA2, CT1, R3 |  |
| R2-1911601 | RAN2-107 | Reply LS on LTE-M identification in 5GC | 5G_CIOT | SA2, RAN3 | RAN |
| R2-1911603 | RAN2-107 | Reply LS on Mobile-terminated Early Data Transmission | NB_IOTenh3-Core, LTE_eMTC5-Core | SA2, R3 | SA3, CT1, CT4 |
| R2-1911865 | RAN2-107 | Reply LS on UAC for NB-IoT | 5G_CIOT, NB_IOTenh3-Core | SA1 | CT1, SA2, R3 |

# References: Reports from breakout sessions on NB-IoT and MTC

| Meeting | Date | Title | Tdoc |
| --- | --- | --- | --- |
| RAN2-103bis, Chengdu, China | 08-12 October 2018 | Report NB-IoT breakout session | R2-1815603 |
| RAN2-103bis, Chengdu, China | 08-12 October 2018 | Report eMTC breakout session | R2-1815604 |
| RAN2-104, Spokane, USA | 12-16 November 2018 | Report NB-IoT breakout session | R2-1818523 |
| RAN2-104, Spokane, USA | 12-16 November 2018 | Report eMTC breakout session | R2-1818524 |
| RAN2-105, Athens, Greece | 25 Feb - 01 March 2019 | Report NB-IoT breakout session | R2-1902233 |
| RAN2-105, Athens, Greece | 25 Feb - 01 March 2019 | Report eMTC breakout session | R2-1902234 |
| RAN2-105bis, Xi’an, China | 08-12 April 2019 | Report NB-IoT breakout session | R2-1905183 |
| RAN2-105bis, Xi’an, China | 08-12 April 2019 | Report eMTC breakout session | R2-1905184 |
| RAN2-106, Reno, USA | 13-17 May 2019 | Report NB-IoT breakout session | R2-1908103 |
| RAN2-106, Reno, USA | 13-17 May 2019 | Report eMTC breakout session | R2-1908104 |
| RAN2-107, Prague | 26-30 August 2019 | Report NB-IoT breakout session | R2-1911513 |
| RAN2-107, Prague | 26-30 August 2019 | Report eMTC breakout session | R2-1911514 |

# Change history

| Tdoc | Title | Comment |
| --- | --- | --- |
| R2-1816063 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-103bis |
| R2-1818633 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-104 |
| R2-1902405 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-105 |
| R2-1905262 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-105bis |
| R2-1908188 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-106 |
| R2-1911575 | RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC | Post RAN2-107 |