3GPP TSG-RAN WG2 Meeting #107bis		R2-1912111

Chongqing, P. R. China, 14th–18th October 2019

Source:	CATT

Title:	Maximum MIMO layers configuration in power saving

Agenda Item:	6.11.4

Document for:	Discussion and Decision

# Introduction

In RAN1#98 meeting [1] and RAN2#107 meeting [2], the following agreements on how to configure maximum MIMO layers to reduce power consumption were achieved.

RAN1#98 meeting

Agreements:

- Support per-DL-BWP configuration of maximum number of DL MIMO layers

  - Signalling details up to RAN2

RAN2#107 meeting:

Agreements

- Maximum number of MIMO layers can be configured at least per DL BWP including initial/default and others.

- If maximum number of MIMO layers is configured for a BWP, the UE uses this value and ignores the cell-specific value provided in the PDSCH-ServingCellConfig IE, when operating in the BWP.

- If maximum number of MIMO layers is not configured for a BWP, the UE uses the cell-specific value provided in the PDSCH-ServingCellConfig IE, when operating in the BWP.

According to the agreements, it can be seen that per-DL-BWP configuration of maximum MIMO layer will be supported in Rel-16. However, the design of this IE remains to be specified. In this document, we address the potential formats of the signalling structure for per-DL-BWP configuration of maximum DL MIMO layers.

# Discussion

In current specification, IE maxMIMO-Layers is configured under the IE PDSCH-ServingCellConfig, which is at cell level. In order to achieve backward compatibility, this cell level IE maxMIMO-Layers is not changed. There were mainly 2 options proposed in RAN2#107 meeting documents which provided RRC signaling structure examples.

Option 1: UE dedicated configuration - PDSCH-Config [3]-[6]

Option 2: Cell common configuration - PDSCH-ConfigCommon [7]

Compared with cell common configuration, it is more flexible to configure maximum MIMO layer with UE dedicated signaling. Besides, different UEs may have different services or can be distributed at different coverage locations in the cell, so it is more reasonable to configure maximum MIMO layer using UE dedicated configuration. Thus one new parameter, i.e. maximum MIMO layer for BWP level, which also can be named maxMIMO-Layers could be configured under the IE PDSCH-Config. Meanwhile, according to the agreements, maxMIMO-Layers of cell level will be ignored, if it is also configured in PDSCH-Config under a given BWP; otherwise cell level value will be used. Possible signaling structure is illustrated in the Figure 1.

Figure 1 Possible signaling structure of maxMIMO-Layers

Regarding the need code, we think the best way to capture the above RAN2 agreement is to explicitly specify the UE behavior regarding maxMIMO-Layers in the IE PDSCH-ServingCellConfig when the same field is present/absent in PDSCH-Config. Hence this falls in the category of “Need S” code.

One example of ASN.1 format is shown in Annex A.

Hence, it is proposed that:

Proposal 1: RAN2 agrees the ASN.1 format in Annex A.

# Conclusion

This contribution discusses the ASN.1 format issues related to per-BWP maximum MIMO layer. And we propose that:

Proposal 1: RAN2 agrees the ASN.1 format in Annex A.

# Reference

- RAN1#98 meeting, chairman notes, 3GPP TSG RAN WG1 #98, Czech Republic, 26th - 30th August 2019;

- RAN2#107 meeting, chairman notes, 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech Republic, 26th - 30th August 2019;

- R2-1908891	, Per BWP configuration of maxMIMOLayer, CATT, 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech Republic, 26th - 30th August 2019;

- R2-1909987, UE adaptation to maximum number of MIMO layers, Ericsson, RAN2#107 meeting, 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech Republic, 26th - 30th August 2019;

- R2-1908777, MIMO layer adaptation for power saving, OPPO, 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech Republic, 26th - 30th August 2019;

- R2-1910531, On MIMO layer adaptation, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech Republic, 26th - 30th August 2019;

- R2-1910084, On maximum number of MIMO layers, MediaTek Inc., 3GPP TSG-RAN WG2 Meeting #107, Prague, Czech Republic, 26th - 30th August 2019;

# Annex

#### PDSCH-Config

The PDSCH-ConfigIE is used to configure the UE specific PDSCH parameters.

PDSCH-Configinformation element

-- ASN1START

-- TAG-PDSCH-CONFIG-START

PDSCH-Config ::=                        SEQUENCE {

dataScramblingIdentityPDSCH             INTEGER (0..1023)                                                   OPTIONAL,   -- Need S

dmrs-DownlinkForPDSCH-MappingTypeA      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

dmrs-DownlinkForPDSCH-MappingTypeB      SetupRelease { DMRS-DownlinkConfig }                                OPTIONAL,   -- Need M

tci-StatesToAddModList                  SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-State                  OPTIONAL,   -- Need N

tci-StatesToReleaseList                 SEQUENCE (SIZE(1..maxNrofTCI-States)) OF TCI-StateId                OPTIONAL,   -- Need N

vrb-ToPRB-Interleaver                   ENUMERATED {n2, n4}                                                 OPTIONAL,   -- Need S

resourceAllocation                      ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch},

pdsch-TimeDomainAllocationList          SetupRelease { PDSCH-TimeDomainResourceAllocationList }             OPTIONAL,   -- Need M

pdsch-AggregationFactor                 ENUMERATED { n2, n4, n8 }                                           OPTIONAL,   -- Need S

rateMatchPatternToAddModList            SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL,   -- Need N

rateMatchPatternToReleaseList           SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL,   -- Need N

rateMatchPatternGroup1                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rateMatchPatternGroup2                  RateMatchPatternGroup                                               OPTIONAL,   -- Need R

rbg-Size                                ENUMERATED {config1, config2},

mcs-Table                               ENUMERATED {qam256, qam64LowSE}                                     OPTIONAL,   -- Need S

maxNrofCodeWordsScheduledByDCI          ENUMERATED {n1, n2}                                                 OPTIONAL,   -- Need R

prb-BundlingType                        CHOICE {

staticBundling                          SEQUENCE {

bundleSize                              ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

},

dynamicBundling                     SEQUENCE {

bundleSizeSet1                          ENUMERATED { n4, wideband, n2-wideband, n4-wideband }       OPTIONAL,   -- Need S

bundleSizeSet2                          ENUMERATED { n4, wideband }                                 OPTIONAL    -- Need S

}

},

zp-CSI-RS-ResourceToAddModList                  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-Resource

OPTIONAL,   -- Need N

zp-CSI-RS-ResourceToReleaseList                 SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-Resources)) OF ZP-CSI-RS-ResourceId

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList    SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

aperiodic-ZP-CSI-RS-ResourceSetsToReleaseList SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToAddModList   SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSet

OPTIONAL,   -- Need N

sp-ZP-CSI-RS-ResourceSetsToReleaseList  SEQUENCE (SIZE (1..maxNrofZP-CSI-RS-ResourceSets)) OF ZP-CSI-RS-ResourceSetId

OPTIONAL,   -- Need N

p-ZP-CSI-RS-ResourceSet                 SetupRelease { ZP-CSI-RS-ResourceSet }

OPTIONAL,   -- Need M

...

[[

maxMIMO-Layers                          INTEGER (1..8)                                                      OPTIONAL,   -- Need S

]]

}

RateMatchPatternGroup ::=               SEQUENCE (SIZE (1..maxNrofRateMatchPatternsPerGroup)) OF CHOICE {

cellLevel                               RateMatchPatternId,

bwpLevel                                RateMatchPatternId

}

-- TAG-PDSCH-CONFIG-STOP

-- ASN1STOP

PDSCH-Configfield descriptions

aperiodic-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring aperiodically triggered zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network configures the UE with at most 3 aperiodic ZP-CSI-RS-ResourceSets and it uses only the ZP-CSI-RS-ResourceSetId 1 to 3. The network triggers a set by indicating its ZP-CSI-RS-ResourceSetId in the DCI payload. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetId 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetId 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetId 3 (see TS 38.214 [19], clause 5.1.4.2)

dataScramblingIdentityPDSCH Identifier used to initialize data scrambling (c_init) for PDSCH. If the field is absent, the UE applies the physical cell ID. (see TS 38.211 [16], clause 7.3.1.1).

dmrs-DownlinkForPDSCH-MappingTypeA DMRS configuration for PDSCH transmissions using PDSCH mapping type A (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

dmrs-DownlinkForPDSCH-MappingTypeB DMRS configuration for PDSCH transmissions using PDSCH mapping type B (chosen dynamically via PDSCH-TimeDomainResourceAllocation). Only the fields dmrs-Type, dmrs-AdditionalPosition and maxLength may be set differently for mapping type A and B.

maxMIMO-Layers Indicates the maximum MIMO layer to be used for PDSCH of the BWP (see TS 38.212 [17], clause 5.4.2.1). The IE maxMIMO-Layers configured per serving cell should be ignored when IE maxMIMO-Layers is configured for the BWP; otherwise cell level maxMIMO-Layers is used.

maxNrofCodeWordsScheduledByDCI Maximum number of code words that a single DCI may schedule. This changes the number of MCS/RV/NDI bits in the DCI message from 1 to 2.

mcs-Table Indicates which MCS table the UE shall use for PDSCH. (see TS 38.214 [19], clause 5.1.3.1). If the field is absent the UE applies the value 64QAM.

pdsch-AggregationFactor Number of repetitions for data (see TS 38.214 [19], clause 5.1.2.1). When the field is absent the UE applies the value 1.

pdsch-TimeDomainAllocationList List of time-domain configurations for timing of DL assignment to DL data (see table 5.1.2.1.1-1 in TS 38.214 [19]).

prb-BundlingType Indicates the PRB bundle type and bundle size(s) (see TS 38.214 [19], clause 5.1.2.3). If dynamic is chosen, the actual bundleSizeSet1 or bundleSizeSet2 to use is indicated via DCI. Constraints on bundleSize(Set) setting depending on vrb-ToPRB-Interleaver and rbg-Size settings are described in TS 38.214 [19], clause 5.1.2.3. If a bundleSize(Set) value is absent, the UE applies the value n2.

p-ZP-CSI-RS-ResourceSet A set of periodically occurring ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList). The network uses the ZP-CSI-RS-ResourceSetId=0 for this set.

rateMatchPatternGroup1 The IDs of a first group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternGroup2 The IDs of a second group of RateMatchPatterns defined in PDSCH-Config->rateMatchPatternToAddModList (BWP level) or in ServingCellConfig ->rateMatchPatternToAddModList (cell level). These patterns can be activated dynamically by DCI (see TS 38.214 [19], clause 5.1.4.1).

rateMatchPatternToAddModList Resources patterns which the UE should rate match PDSCH around. The UE rate matches around the union of all resources indicated in the nested bitmaps (see TS 38.214 [19], clause 5.1.4.1). FFS: RAN1 indicates that there should be a set of patterns per cell and one per BWP => Having both seems unnecessary.

rbg-Size Selection between config 1 and config 2 for RBG size for PDSCH. The UE ignores this field if resourceAllocation is set to resourceAllocationType1 (see TS 38.214 [19], clause 5.1.2.2.1).

resourceAllocation Configuration of resource allocation type 0 and resource allocation type 1 for non-fallback DCI (see TS 38.214 [19], clause 5.1.2.2).

sp-ZP-CSI-RS-ResourceSetsToAddModList AddMod/Release lists for configuring semi-persistent zero-power CSI-RS resource sets. Each set contains a ZP-CSI-RS-ResourceSetId and the IDs of one or more ZP-CSI-RS-Resources (the actual resources are defined in the zp-CSI-RS-ResourceToAddModList) (see TS 38.214 [19], clause 5.1.4.2).

tci-StatesToAddModList A list of Transmission Configuration Indicator (TCI) states indicating a transmission configuration which includes QCL-relationships between the DL RSs in one RS set and the PDSCH DMRS ports (see TS 38.214 [19], clause 5.1.5).

vrb-ToPRB-Interleaver Interleaving unit configurable between 2 and 4 PRBs (see TS 38.211 [16], clause 7.3.1.6). When the field is absent, the UE performs non-interleaved VRB-to-PRB mapping.

zp-CSI-RS-ResourceToAddModList A list of Zero-Power (ZP) CSI-RS resources used for PDSCH rate-matching. Each resource in this list may be referred to from only one type of resource set, i.e., aperiodic, semi-persistent or periodic (see TS 38.214 [19]).