3GPP TSG-RAN2 Meeting #107bis	R2-1912139

# Chongqing, P.R.China, 14 - 18 Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1266 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the premises of E-UTRA HO to NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-09-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1) In 38.331,on the E-UTRA HO to NR premise, it is stated that: The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only after integrity has been activated in E-UTRA. However, the premise here is in contradictory with the initiation description of mobility from E-UTRA in 36.331, which states that: E-UTRAN initiates the mobility from E-UTRA procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport message or in response to reception of CS fallback indication for the UE from MME, by sending a MobilityFromEUTRACommand message. E-UTRAN applies the procedure as follows: - the procedure is initiated only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended; It is clear that the premise of E-UTRA HO to NR in 38.331 is less than the description in 36.331, which would lead to an inter-protocol conflict. As one example, when RRC: SecurityModeCommand is sent out, the integrity is activated in E-UTRA, but at this time E-UTRA will not initiate HO to NR, as neither SRB2 nor DRB is setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Replace the 38.331 premises of E-UTRA HO to NR with 36.331 descriptions. Impact analysis Impacted 5G architecture options: SA Impacted functionality: HO to NR Inter-operability: 1) This CR concerns only about gNB. 2) If the NW is implemented according to this CR, no impact on UE is forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) There is an inter-protocol conflict between 38.331 and 36.331 on the premises of E-UTRA HO to NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.4.2.1	General

Figure 5.4.2.1-1: Handover to NR, successful

The purpose of this procedure is to, under the control of the network, transfer a connection between the UE and another Radio Access Network (e.g. E-UTRAN) to NR.

The handover to NR procedure applies when SRBs, possibly in combination with DRBs, are established in another RAT. Handover from E-UTRA to NR applies only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended in E-UTRA.

END OF CHANGE