3GPP TSG-RAN WG2 #107bis	Tdoc R2-1912827

# Chongqing, China, 14th – 18th October 2019

Agenda Item:	6.12.2

Source:	Ericsson

Title:	Leftover issues for Logged MDT in NR

Document for:	Discussion, Decision

# 1	Introduction

This paper discusses the following open issues in logged MDT:

- Basic principles to introduce a framework for event-based logged measurement for MDT in NR.

- MDT memory size, potential issues of logged MDT concerning device memory, in particular when UE suffers from memory issue and logs multiple consecutive measurement at the same location.

- MDT configuration for UEs in Inactive State

# 2	Background

In LTE, a UE is configured to perform periodical MDT logging during RRC_IDLE state after receiving the MDT configurations from the network. The UE shall report the DL pilot strength measurements (RSRP/RSRQ) together with time information, detailed location information if available, and WLAN, Bluetooth to the network using the UE information framework when it moves back to RRC_CONNECTED state. The DL pilot strength measurement of Logged MDT is collected based on the existing measurements required for cell reselection purpose, without imposing UE to perform additional measurements.

Table 1. Key parameters sent to UE in the configuration message for Logged MDT in LTE

| Parameters | Notes |
| --- | --- |
| areaConfiguration | It defines the area where the logged measurements are taken by the UE. |
| plmn-IdentityList | It is used to indicate a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs. |
| Trace reference | It is assigned by OAM. |
| Trace recording session reference | It is assigned by MME in signaling-based MDT; it is assigned by eNB in the management-based MDT. |
| Trace Collection Entity (TCE) identifier | It is mapping to the TCE IP address and will be sent to the TCE together with the logged measurements. |
| absoluteTimeInfo | It is used to indicate the absolute time in the current cell |
| loggingDuration | It is the configured duration for a UE to perform measurement logging. |
| loggingInterval | It indicates the periodicity for UE to perform the measurement logging. |
| bt-NameList | It is used to indicate the names of the Bluetooth beacon which the UE is configured to measure. |
| wlan-NameList | It is used to indicate the names of the WLAN AP for which the UE is configured to measure. |

## 2.1 The existing event-based logged measurements for RRC_IDLE UE in LTE

There is an event-based logged measurement for UE in RRC_IDLE in LTE, i.e., accessibility measurement [1]. The UE logs failed RRC connection establishments for LTE, i.e. a log is created when the RRC connection establishment procedure fails. The UE logs failed RRC connection establishments without the need for prior configuration by the network. The trigger for creating a log related to a failed RRC connection establishment is for LTE when timer T300 expires.

## 2.2 The problems of combining events based and periodically based logged measurements

After UE being configured with Logged MDT parameters including the loggingDuration and loggingInterval, UE starts the periodical logging when the UE is in RRC_IDLE.

LoggingDuration information element

-- ASN1START

LoggingDuration-r10 ::=			ENUMERATED {

min10, min20, min40, min60, min90, min120, spare2, spare1}

-- ASN1STOP

LoggingInterval information element

-- ASN1START

LoggingInterval-r10 ::=			ENUMERATED {

ms1280, ms2560, ms5120, ms10240, ms20480,

ms30720, ms40960, ms61440}

-- ASN1STOP

In Rel-15, the out-of-coverage detection (i.e., “anyCellSelectionDetected”) was introduced into the existing Logged MDT mechanism, i.e., periodical logging with the configured logging duration and logging interval.

In LTE, there is no starting mechanism at UE for Logged MDT. UE executes the periodical logging when UE is in RRC_IDLE after receiving the Logged MDT configuration. When it is started, UE logs both the DL pilot strength measurements when UE is in Camped normally state and “anyCellSelectionDetected” when UE is in Any Cell Selection state.

In LTE, UE supports a maximum number of logged measurements (i.e., the periodical MDT logging) entries. When the memory reserved for the logged measurement information becomes full, the UE shall stop the duration timer, T330, and perform the same actions as performed upon expiry of T330, i.e., the UE shall release the configuration.

-- ASN1START

maxLogMeas-r10				INTEGER ::= 4060-- Maximum number of logged measurement entries

-- that can be stored by the UE

-- ASN1STOP

For the out-of-coverage detection in LTE, the number of the out-of-coverage detection that a UE can log is limited, since UE logs both of the out-of-coverage detections and periodical DL pilot strength measurements using the same configured parameters: logging interval and logging duration; and the reserved memory is used for both of the purposes. The reserved memory can be full due to the records for the DL pilot strength measurements if the logging duration is large, and UE cannot log the events when the duration is expired if the duration is small.

Figure 1. Example of the limited number of out-of-coverage detection that a UE can record

In LTE, the out-of-coverage detections and periodical DL pilot strength measurements logging are configured with the same logging interval and logging duration; and the reserved memory is used for both purposes.

For the out-of-coverage detection in LTE, the number of the out-of-coverage detection that a UE can log is limited, since the reserved memory can be full due to the records for the DL pilot strength measurements if the logging duration is large, and UE cannot log the events when the duration is expired if the duration is small.

If the logging interval is large, e.g., 61.44 seconds, the UE may miss the out-of-coverage events.

# 3	Discussion

## 3.1 Event Triggered MDT:

In RAN2 #106 [2] and RAN2#107, there are the following agreements for out-of-coverage event in Logged MDT.

1	The out-of-coverage detection and logging in NR can be configured to be independent of the periodical DL pilot strength logged measurements.

2	For MDT in NR, UE can be configured to only perform the logging for out-of-coverage detection, i.e., UE is not required to log the DL pilot strength measurements if the configuration is only out-of-coverage detection.

3    For Logged MDT, introduce periodical logged measurements as in LTE and event-triggered logged measurements (including out-of-coverage event). Details of configuration are FFS

This section discusses the basic principles to introduce a framework for event-based logged measurement for MDT in NR.

- RAN2 has agreed that the event-triggered out-of-coverage logging can be configured to be independent of periodical logging. The event-triggered logging framework (i.e., configuration, storage and reporting) need to be specified.

### 3.1.1 Configuration

It has already agreed that the Logged MDT is the baseline and the running CR on logged MDT have adopted this configuration from LTE.

As indicated in background, it has been agreed to allow network to configure UE with the periodical logging and the event-triggered logging. There could be three cases are discussed below:

### Case 1: only the legacy periodical logging is configured.

- Case 2: only the event-based logging is configured.

### Case 3: both the event-based logging and legacy periodical logging are configured.

Case 1 is already covered in the running CR. We would focus on Case 2 and 3.

#### Case 2: only the event-based logging is configured.

If only event-based logging is configured, the logging is performed by a UE when the entering condition of an event of the configured events is satisfied. Then, UE performs the periodical logging following a configured eventLoggingInterval until the leaving condition of that event is fulfilled or the reserved memory is full.

If there are more than one configured events which have overlaps (e.g., Event B starts after the entering condition of Event A is fulfilled but before the leaving condition of Event A is fulfilled). For simplicity, we can only configure a common eventLoggingInterval for all of the configured events. In the overlapping case, only the entering of the first event triggers the event-based periodical logging. An example in Figure 2 shows that two events configured for event-based logged MDT.

Figure 2. Only event-based logging is configured. UE starts logging when entering Event A and stops logging when leaving Event B.

The proposed ASN.1 messages are discussed as follows.

The proposed LoggedMeasurementConfiguration containing LoggingDuration and LoggingInterval.

LoggedMeasurementConfiguration-r16-IEs ::= SEQUENCE {

traceReference-r16				TraceReference-r16,

traceRecordingSessionRef-r16	OCTET STRING (SIZE (2)),

tce-Id-r16						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16			AbsoluteTimeInfo-r16,

areaConfiguration-r16			AreaConfiguration-r16					             OPTIONAL,	--Need R

loggingDuration-r16				LoggingDuration-r16 		OPTIONAL,

loggingInterval-r16				LoggingInterval-r16			OPTIONAL,

eventTriggeredConfigList     	EventTriggeredConfigList     OPTIONAL,

eventLoggingInterval-r16		EventLoggingInterval-r16     OPTIONAL,

plmn-IdentityList-r16			PLMN-IdentityList3-r16			OPTIONAL,	--Need R

bt-NameList-r16					BT-NameList-r16					OPTIONAL, 	--Need R

wlan-NameList-r16				WLAN-NameList-r16				OPTIONAL, 	--Need R

nonCriticalExtension			SEQUENCE {}                                			 OPTIONAL

}

Proposed EventTriggeredList

EventTriggeredConfigList::= SEQUENCE (SIZE (1..maxNrofLoggedMeasEvents)) OF EventTriggerLoggedMeasConfig

EventTriggerLoggedMeasConfig::=                       CHOICE {

eventA2Trigger  							EventA2Trigger,

eventX1Trigger  							EventX1Trigger,

...

}

EventA2Trigger  ::=                                  SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

...

}

EventX1Trigger	::=					BOOLEAN

Event A2 (Serving becomes worse than threshold) has already been defined in TS 38.331 with the following entering and leaving conditions:

Inequality A2-1 (Entering condition) Ms + Hys < Thresh Inequality A2-2 (Leaving condition) Ms – Hys > Thresh

Event X1: UE is out of the coverage. The entering condition is considered to be satisfied if the UE enters the “Any Cell Selection” state as specified in TS 38.304. The leaving condition is considered to be satisfied when the UE leaves the “Any Cell Selection” state.

#### Case 3: both the event-based logging and legacy periodical logging are configured.

If both of the event-based logging and legacy periodical logging are configured and there is an overlap between them, UE uses the smaller interval between the eventLoggingInterval and loggingInterval to perform the logging during the overlapping part.

Figure 3. Both event-based logging and legacy periodical logging are configured, UE uses the smaller interval between eventLoggingInterval and loggingInterval during the overlapping part.

Based on the above analysis, we conclude that the UE should be able to support both periodical and event triggered MDT without any configuration problems.

- A UE can be configured with the periodical logging or the event-triggered logging or both.

### 3.2 Logging, storage and reporting

The logged measurements information is same for both periodical logging and the event-triggered logging. The difference between them is the determination on when the logging is performed. Therefore, the UE variable (i.e., VarLogMeasReport) to store the logged measurements information and the corresponding parameters in the variable can be common for the both types of logging.

VarLogMeasReport UE variable in LTE

-- ASN1START

VarLogMeasReport-r10 ::=				SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10			OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-Identity-r10					PLMN-Identity,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

VarLogMeasReport-r11 ::=			SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-IdentityList-r11				PLMN-IdentityList3-r11,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

LogMeasInfoList2-r10 ::=				SEQUENCE (SIZE (1..maxLogMeas-r10)) OF LogMeasInfo-r10

-- ASN1STOP

- For Logged MDT, the UE variable (i.e., VarLogMeasReport) to store the logged measurements information and its parameters are common for the both periodical logging and event-triggered logging.

The event-triggered logging and periodical logging can share the same indicator, logMeasAvailable, which indicates to a RAN node that there is logged measurements available at UE. Furthermore, the report IE for logged measurements (i.e., LogMeasReport) and its parameters are common for both.

LogMeasReport in UEInformationResponse message in LTE

LogMeasReport-r10 ::=				SEQUENCE {

absoluteTimeStamp-r10				AbsoluteTimeInfo-r10,

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

logMeasInfoList-r10					LogMeasInfoList-r10,

logMeasAvailable-r10				ENUMERATED {true}				OPTIONAL,

...,

[[	logMeasAvailableBT-r15			ENUMERATED {true}				OPTIONAL,

logMeasAvailableWLAN-r15		ENUMERATED {true}				OPTIONAL

]]

}

LogMeasInfoList-r10 ::=		SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF LogMeasInfo-r10

LogMeasInfo-r10 ::=		SEQUENCE {

locationInfo-r10					LocationInfo-r10				OPTIONAL,

relativeTimeStamp-r10				INTEGER (0..7200),

servCellIdentity-r10				CellGlobalIdEUTRA,

measResultServCell-r10				SEQUENCE {

rsrpResult-r10						RSRP-Range,

rsrqResult-r10						RSRQ-Range

},

measResultNeighCells-r10			SEQUENCE {

measResultListEUTRA-r10				MeasResultList2EUTRA-r9		OPTIONAL,

measResultListUTRA-r10				MeasResultList2UTRA-r9		OPTIONAL,

measResultListGERAN-r10				MeasResultList2GERAN-r10	OPTIONAL,

measResultListCDMA2000-r10			MeasResultList2CDMA2000-r9	OPTIONAL

}	OPTIONAL,

...,

[[	measResultListEUTRA-v1090			MeasResultList2EUTRA-v9e0	OPTIONAL

]],

[[	measResultListMBSFN-r12				MeasResultListMBSFN-r12		OPTIONAL,

measResultServCell-v1250			RSRQ-Range-v1250			OPTIONAL,

servCellRSRQ-Type-r12				RSRQ-Type-r12				OPTIONAL,

measResultListEUTRA-v1250			MeasResultList2EUTRA-v1250	OPTIONAL

]],

[[	inDeviceCoexDetected-r13			ENUMERATED {true}			OPTIONAL

]],

[[	measResultServCell-v1360			RSRP-Range-v1360			OPTIONAL

]],

[[	logMeasResultListBT-r15				LogMeasResultListBT-r15		OPTIONAL,

logMeasResultListWLAN-r15			LogMeasResultListWLAN-r15	OPTIONAL

]],

[[	anyCellSelectionDetected-r15		ENUMERATED {true}			OPTIONAL

]]

}

- For Logged MDT, the report IE for logged measurements (i.e., LogMeasReport) and its parameters are common for the both periodical logging and event-triggered logging.

- Adopt the Text Proposal providef in Annex A to adopt event based MDT framework in 38.331.

## 3.2 UE Memory for logged MDT

For logged MDT, UE receives the MDT configurations including logginginterval and loggingduration in the RRC message, i.e., LoggedMeasurementConfiguration, from the network as shown in the following:

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			LoggedMeasurementConfiguration-v1080-IEs	OPTIONAL

}

A timer (T330) is started at the UE upon receiving the configurations and UE sets the logging duration according to the loggingduration (10 min – 120 min). The UE shall perform periodical MDT logging with the interval set to logginginterval (1.28 s – 61.44 s). An example of the MDT logging is shown in the Figure 1.

Figure 1. Example of Logged MDT procedure

The logged measurement report consists of measurement results for the serving cell (the measurement quantity), available UE measurements performed in idle for intra-frequency/inter-frequency/inter-RAT, time stamp and location information. When the UE performs logging at the regular logging intervals, it adds a logged measurement entry in a UE variable called VarLogMeasReport. The detailed logged measurements in the VarLogMeasReport are described below.

- A UE can be configured to perform MDT logged measurement in a periodic intervals for a certain period of time.

-- ASN1START

VarLogMeasReport-r10 ::=				SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10			OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-Identity-r10					PLMN-Identity,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

VarLogMeasReport-r11 ::=			SEQUENCE {

traceReference-r10					TraceReference-r10,

traceRecordingSessionRef-r10		OCTET STRING (SIZE (2)),

tce-Id-r10							OCTET STRING (SIZE (1)),

plmn-IdentityList-r11				PLMN-IdentityList3-r11,

absoluteTimeInfo-r10				AbsoluteTimeInfo-r10,

logMeasInfoList-r10					LogMeasInfoList2-r10

}

LogMeasInfoList2-r10 ::=				SEQUENCE (SIZE (1..maxLogMeas-r10)) OF LogMeasInfo-r10

-- ASN1STOP

As depicted above, the maxLogMeas-r10 is the maximum number of logged measurement entries which can be stored by UE. Since Rel-10, the maximum number, maxLogMeas-r10, is 4060 entries.

-- ASN1START

maxLogMeas-r10				INTEGER ::= 4060-- Maximum number of logged measurement entries

-- that can be stored by the UE

-- ASN1STOP

- The maximum number (i.e., maxLogMeas-r10) of MDT logged measurements entries that can be stored by the UE has NOT been changed since Rel-10.

The UEs today have much more memory size than that 10 year ago. MDT has been used and verified in commercial network to identify and analyze coverage issues, e.g., poor coverage and over shooting [2]. It would be beneficial that UE can store more entries in Logged MDT in NR.

RAN2 to discuss introduction of a greater value of the maximum number of MDT logged measurements entries that can be stored by the UE in NR compared to LTE maxLogMeas-r10. The value of maxLogMeas-r16 is FFS.

For the Logged MDT in LTE, when the memory reserved for the logged measurement information becomes full, UE stops timer T330 and performs the same actions as performed upon expiry of T330 (i.e., the UE shall release the MDT configuration). In this way, the network may not get the expected measurements data from the UE. The solution which considers conserving UE memory can be to introduce another reserved memory for logged measurements at UE; if the primary reserved memory is full, the UE only logs the data in this reserved memory when certain conditions are fulfilled, or the triggering conditions of certain events are fulfilled. Some examples are illustrated as follows.

- UE continues performing the out-of-coverage entries in this secondary memory upon out-of-coverage events are detected when the primary reserved memory is full.

- UE can prioritize the logged entries in this secondary memory, e.g., only keeping the measurements from the serving cell and skipping the collected data of the neighbor cells.

- UE can adjust the logging interval based on the remaining reserved secondary memory.

- UE can use a selection/filtering mechanism to discard/remove the measurement captured at the same location (in case of detailed location information) to avoid redundancy of the measurements. This is discussed in the next section.

RAN2 to discuss introduction of another reserved memory in UE for logged measurements; the UE only logs the data in this memory when certain conditions or the triggering conditions of the certain events are fulfilled.

Relation between measurement and location information

In logged MDT it is possible to tag each measurement entry with the location information, if available. The location information can be derived either by ECGI or Cell-Id of the serving cell or can be taken as detailed location information (e.g. GNSS location information) that can be included if available and valid when the measurement is taken. If detailed location information is available, the reporting shall consist of latitude and longitude. Depending on availability, altitude, uncertainty and confidence may be also additionally included. UE tags available detailed location information only once with upcoming measurement sample, and then the detailed location information is discarded, i.e. the validity of detailed location information is implicitly assumed to be one logging interval.

- In logged MDT it is possible to tag each measurement entry with the location information, if available. The location information can be derived either by ECGI or Cell-Id of the serving cell or can be taken as detailed location information (e.g. GNSS location information) that can be included if available and valid.

As already mentioned, one interesting use case for performing logged MDT is coverage optimization. Generally, the coverage optimization requires measurements to be performed in all possible locations within the coverage area to provide a complete picture of the radio propagation environment to the network nodes for e.g., AAS based optimization of the capacity and coverage. On the other side, a limited memory can be allocated to the UE to perform the logged measurement. For example, in LTE, the maximum number of logged measurement entries (i.e., the memory reserved for Logged MDT, maxLogMeas), which can be stored by UE is 4060 entries. Therefore, a delicate MDT configuration is required to capture only the necessary and useful measurement at interested and target locations.

Although the radio signal propagation and measurement results are highly dependent on the location and mobility pattern of the UE, MDT configuration is currently oblivious to the mobile UEs’ location (mobility) and is blindly performed right at each MDT logging interval. To exemplify, for a stationary UE, the radio signal propagation may barely change and hence the logged data reported to the network may bring not much information. Hence, all the 4060 entries may include similar radio condition belonging to a given location of the UE. In other words, a UE may be configured to perform logged measurement while it might be in a fixed location throughout the measurement period (loggingDuration), hence providing redundant data to the network that may not be useful for coverage optimization. Therefore, network should lend more careful consideration on the location information when configuring the UE with logged MDT measurement, by configuring the UE to possibly discard redundant measurement when the location information is fixed for some consecutive measurements.

- A UE may be configured to perform logged measurement, while it might be in a fixed location throughout the measurement period (loggingDuration), hence providing redundant data to the network that may not be useful for coverage optimization or any other purpose.

Therefore, in our understanding, RAN2 can discuss a selection/filtering mechanism for UE to discard/remove measurement (or entries of measurement in the trace) wherein the location information is the same or at least very similar to the location information of the preceding measurements. In this way, the UE may provide more meaningful measurement data captured at different locations which can be used by network for different use-cases such as coverage or mobility optimization. On the other side, the current logged MDT method may result buffer bloating in a short period of time before expiry of T330 timer.  A selection/filtering mechanism can assist the UE to better utilize the memory allocated to the logged measurement by discarding consecutive measurements captured at the same location. Hence the risk of having a full memory before expiration of T330 timer significantly reduces. In addition, it helps the UE to reduce energy consumption for logged MDT.

A filtering mechanism based on the location information can be enabled by the MDT configurations to allow the UE to discard logging/storing the MDT measurements based on the UE location information. In other words, UE discards the measurement if the location information is the same or very similar to the location information of the preceding measurement. This can be very helpful for the UE to conserve memory for more valuable measurement from other locations ( in particular, when UE deals with memory shortage).

RAN2 discuss a selection/filtering mechanism to discard/remove the measurement captured at the same location (in case of detailed location information) to avoid redundancy of the measurements (when dealing with memory shortage), when performing logged MDT. How to configure and identify/measure the changes in the location is FFS.

## Logged MDT configuration for UE’s in Inactive State

In LTE, for signaling-based MDT, OAM sends the trace session activation a specific UE in CM-CONNECTED to MME. If the UE has already attached to the network, MME sends Trace Start message containing the MDT configuration to the serving gNB. Then, the gNB can configure the UE to perform the MDT by sending the RRC messages to the UE since the UE is in RRC_CONNECTED. However, in Rel-15, RRC Inactive is introduced and the UE might be in RRC_INACTIVE state when the serving gNB receives the Trace Start message.

The activation/configuration method for signaling-based MDT in LTE does not work if the UE is in RRC Inactive when the gNB receives the MDT configuration.

It is possible for AMF to subscribe the RRC state for a specific UE. AMF can send RRC Inactive Transition Report Request to NG-RAN to request NG-RAN node to report the RRC state and user location information of a specific UE when the UE enters or leaves RRC_INACTIVE.

It is possible for AMF to be informed when a UE moves from RRC_INACTIVE to RRC_CONNECTED.

Therefore, one possible option to consider the RRC Inactive case is that the NG-RAN nodes can assume that AMF is always aware of the RRC state of the UE for the signaling-based MDT.

There is a Trace Failure Indication procedure which allows the NG-RAN node to inform the AMF that a Trace Start procedure or a Deactivate Trace procedure has failed due to an interaction with a handover procedure. We can extend the usage of the Trace Failure Indication to support NG-RAN node to inform AMF that the failure of the MDT activation/configuration due to the UE is in RRC Inactive state.

OPTION 1:

- NG-RAN assumes that AMF sends MDT activation/configuration for signaling-based MDT for a specific UE when the UE is in RRC Connected.

- NG-RAN initiates Trace Failure Indication procedure to inform AMF the failure of the MDT activation/configuration when a NG-RAN node receives the MDT configuration while the UE is in RRC Inactive state.

- AMF stores the MDT configuration and sends to another serving NG-RAN node. An example is shown in the Figure 1.

Figure 1. example of option 1: NG-RAN assumes that AMF sends MDT activation/configuration for signaling-based MDT when the UE is in RRC Connected.

Another option is that NG-RAN assumes that AMF may or may not know the RRC state of the UE for the signaling-based MDT.

OPTION 2:

- NG-RAN assumes that AMF may or may not know the RRC state of the UE for the signaling-based MDT.

- When the last serving gNB receives the MDT configuration, the gNB stores the MDT configuration in Inactive UE context when the UE is RRC Inactive.

- It is up to NG-RAN to determine when to configure the UE with the configuration.

  - A new serving gNB can retrieve the MDT configuration from the old serving gNB and configure the UE. An example is shown in the Figure 2.

  - The serving gNB can also trigger a RAN paging procedure to move the UE to RRC_CONNECTED state. Once the UE is connected (either in the old serving gNB or in a new serving gNB), the serving gNB configures the UE to perform MDT measurement and reporting.

Figure 2. example of option 2: NG-RAN assumes that AMF may or may not know the RRC state of the UE for the signaling-based MDT.

A third option can be that an indicator can be included in the MDT configuration to indicate whether NG-RAN should send the Trace Failure indication or store the configuration if the UE is RRC Inactive state.

Based on the discussions and observations, we have the following proposals.

- RAN2 to send a LS to RAN3 about the problem and potential solution.

- Problem: for signaling-based MDT activation/configuration in the case that the UE is in RRC Inactive when the gNB receives the MDT configuration, gNB cannot configure the UE with MDT configurations.

- RAN 2 preferred solution: AMF sends MDT activation/configuration to CM Connected UE regardless of the RRC state of the UE; NG-RAN stores the configuration in Inactive UE context if the UE is in RRC Inactive state. It is up to NG-RAN to determine when to configure the UE with the configuration.

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 has agreed that the event-triggered out-of-coverage logging can be configured to be independent of periodical logging. The event-triggered logging framework (i.e., configuration, storage and reporting) need to be specified.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE can be configured with the periodical logging or the event-triggered logging or both.

Proposal 2	For Logged MDT, the UE variable (i.e., VarLogMeasReport) to store the logged measurements information and its parameters are common for the both periodical logging and event-triggered logging.

Proposal 3	For Logged MDT, the report IE for logged measurements (i.e., LogMeasReport) and its parameters are common for the both periodical logging and event-triggered logging.

Proposal 4	Adopt the Text Proposal providef in Annex A to adopt event based MDT framework in 38.331.

Proposal 5	RAN2 to discuss introduction of a greater value of the maximum number of MDT logged measurements entries that can be stored by the UE in NR compared to LTE maxLogMeas-r10. The value of maxLogMeas-r16 is FFS.

Proposal 6	RAN2 to discuss introduction of another reserved memory in UE for logged measurements; the UE only logs the data in this memory when certain conditions or the triggering conditions of the certain events are fulfilled.

Proposal 7	RAN2 discuss a selection/filtering mechanism to discard/remove the measurement captured at the same location (in case of detailed location information) to avoid redundancy of the measurements (when dealing with memory shortage), when performing logged MDT. How to configure and identify/measure the changes in the location is FFS.

Proposal 8	RAN2 to send a LS to RAN3 about the problem and potential solution.

Problem: for signaling-based MDT activation/configuration in the case that the UE is in RRC Inactive when the gNB receives the MDT configuration, gNB cannot configure the UE with MDT configurations.

RAN 2 preferred solution: AMF sends MDT activation/configuration to CM Connected UE regardless of the RRC state of the UE; NG-RAN stores the configuration in Inactive UE context if the UE is in RRC Inactive state. It is up to NG-RAN to determine when to configure the UE with the configuration.

# References

TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT), 3GPP, V15.0.0, 2018-07

RAN2-106-Notes LTE-5GC NR-idle RAN-DCU Hu-2019-05-16-1930, 3GPP RAN2#106 meeting, Reno, USA, May 2019

# Annex A

#### –	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used by NR to configure the UE to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::= SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration               LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                     SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::= SEQUENCE {

traceReference-r16				TraceReference-r16,

traceRecordingSessionRef-r16	OCTET STRING (SIZE (2)),

tce-Id-r16						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16			AbsoluteTimeInfo-r16,

areaConfiguration-r16			AreaConfiguration-r16			OPTIONAL,	--Need R

plmn-IdentityList-r16			PLMN-IdentityList3-r16			OPTIONAL,	--Need R

bt-NameList-r16					BT-NameList-r16					OPTIONAL, 	--Need R

wlan-NameList-r16				WLAN-NameList-r16				OPTIONAL, 	--Need R

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

}

PeriodicalReportConfig ::=                  SEQUENCE {

loggingDuration-r16				LoggingDuration-r16 		OPTIONAL,

loggingInterval-r16				LoggingInterval-r16			OPTIONAL,

}

EventTriggerConfig ::=						SEQUENCE {

eventLoggingInterval-r16		EventLoggingInterval-r16     OPTIONAL,

eventTriggeredConfigList     	EventTriggeredConfigList     OPTIONAL,

}

}

EventTriggeredConfigList::= SEQUENCE (SIZE (1..maxNrofLoggedMeasEvents)) OF EventTriggerLoggedMeasConfig

EventTriggerLoggedMeasConfig::=                       CHOICE {

eventOOCTrigger  							EventOOCTrigger,

eventX1Trigger  							EventX1Trigger,

...

}

EventOOCTrigger  ::=                                  SEQUENCE {

OOC-Threshold                                MeasTriggerQuantity,

...

}

EventX1Trigger	::=					FFS

nonCriticalExtension			SEQUENCE {}                                			 OPTIONAL

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

tce-Id Parameter Trace Collection Entity Id: See TS XX.XXX [YY].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS XX.XXX [YY].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT configuration or Event Triggered MDT.

EventTriggerLoggedMeasConfig Parameter configures with UE with specific events for triggering MDT configuration. Current specified event is based on out of coverage detection.