3GPP TSG-RAN2 Meeting #107-Bis	R2-1912976

# Chongqing, China, 14th Oct 2019 - 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4111 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on RSSI and Channel Occupancy measurement in LAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Core |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 36.214 subclause 5.1.24, the RSSI measurement BW is not specified to a particular value. In TS 36.133 subclause 9.1.18.5, measurement accuracy requirements do not specifiy a measurement BW either. Furthermore, ChannelOccupancyThreshold is defined in units of dBm (as opposed to dBm/Hz) which creates a problem for the NW in properly setting the threshold if different UEs use different RSSI measurement BWs. The existing specifications specify the RSRP/RSRQ measurement bandwidths, but for the case of RSSI, it is assumed that the RSSI measurement values correspond to 6 RB equivalent, but it is not clarified anywhere. A UE is not required to measure only 6 RB for RSSI; it may measure RSSI according to AllowedMeasBandwidth. A UE which measures with a bandwidth other that 6 RB needs to scale the measured RSSI to report a nominal RSSI equivalent to 6 RB measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that a UE which measures RSSI with a bandwidth other that 6 RB scales the measured RSSI and thresholds to a nominal RSSI equivalent to 6 RB. Impact Analysis: Impacted Functionality: RSSI measurement for LAA Interoperability: - No interoperability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 specification remains unclear on the RSSI threshold and reported RSSI value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 6.3.5	Measurement information elements

#### <<unchanged text skipped>>

#### –	RSSI-Range-r13

The IE RSSI-Range specifies the value range used in RSSI measurements and thresholds assuming RSSI measurement bandwidth of 6 RB. A UE which performs RSSI measurement on bandwidth other that 6 RB shall scale the measurements and thresholds accordingly. Integer value for RSSI measurements is according to mapping table in TS 36.133 [16].

RSSI-Range information element

-- ASN1START

RSSI-Range-r13 ::=						INTEGER(0..76)

-- ASN1STOP

End of changes