3GPP TSG-RAN WG2 Meeting #107bis	R2-1912998

Chongqing, China, 14th – 18th October 2019

Agenda Item:	4.1

Source: 	MediaTek Inc., LG Electronics

Title:  	Reselection to a Cell with Valid Dedicated Frequency Offset

Document for:	Discussion and decision

Introduction

The network may redirect a UE to a desired frequency by configuring Dedicated Frequency Offset, which makes UE discriminate other frequencies. However, Despite the Dedicated Frequency Offset, UE may perform inter-frequency cell reselection away from the desired frequency. In this case, UE may not trigger cell reselection back to the desired frequency, because in current cell-ranking criteria, Dedicated Frequency Offset is never applied for the serving cell (or intra-frequency neighbours).

The related text in TS 36.304 is provided below for reference:

#### 5.2.4.6 Intra-frequency and equal priority inter-frequency Cell Reselection criteria The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by: where: Qmeas; RSRP measurement quantity used in cell reselections.. Qoffset; For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Except for NB-IoT, equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. For NB-IoT equals to QoffsetDedicatedfrequency for any frequency other than the frequency of the dedicated frequency offset, if QoffsetDedicatedfrequency is valid, otherwise this equals to Qoffsetfrequency (if QoffsetDedicatedfrequency is valid Qoffsetfrequency is not used).. Qoffsettemp; Offset temporarily applied to a cell as specified in TS 36.331 [3]. QoffsetSCPTM; Offset temporarily applied to an SC-PTM frequency as specified below. The offset is applied to all cells on the SC-PTM frequency. If QoffsetSCPTM is valid, Qoffset for inter-frequency neighbour cells is not used..

Discussion

Details of the problem

The Dedicated Frequency Offset provided by the network makes UE favour the (or, as described by 36.304 disfavour all other frequencies) frequency the UE is redirected to for the given time duration.

- When the UE camps on a cell that is on desired frequency, in ranking algorithm for cell reselection, the UE disfavours all cells that are on different frequencies, as long as an associated timer is running (i.e. the Rn value for ranking in 36.304 disfavours inter-frequency cells by QoffsetDedicatedfrequency).

- However, if the UE, due to cell reselection, camps on a cell on some other frequency than the desired frequency, then the current specification does NOT disfavour the current serving cell (even though it is not on the desired frequency), nor its intra-frequency neighbours.

Such ranking criteria result in asymmetric UE behaviour: A non-desirable frequency is sometimes disfavoured but sometimes not. Then, once a UE camps on a non-desirable frequency, UE may not trigger cell reselection back to the desired frequency because the serving cell, even not desirable, is not disfavoured by UE with current formula (i.e., “-Qoffset” is not applied)

Proposed solution

To ensure symmetric UE behaviour, the UE should disfavour all cells on frequencies other than X in all cases (regardless where the UE is camping) as long as the dedicated frequency offset is valid. In this way, the UE is more likely to trigger cell reselection back to the desired frequency, which aligns better with the configurations by the network. Therefore, we propose to apply Qoffset in the cell-ranking criterion Rs for serving cell. In the corresponding field description, state that for serving and intra-frequency that are different from the desired frequency (with dedicated frequency offset), Qoffset equals to QoffsetDedicatedfrequency, if valid.

Proposal 1:	Apply Qoffset in the cell-ranking criterion Rs for serving cell. In the corresponding field description, state that for serving and intra-frequency that are different from the desired frequency (with dedicated frequency offset), Qoffset equals to QoffsetDedicatedfrequency, if valid.

A corresponding TS 36.304 CR is provided in [1].

Conclusion

It is proposed to discuss and decide on the following proposal:

Proposal 1:	Apply Qoffset in the cell-ranking criterion Rs for serving cell. In the corresponding field description, state that for serving and intra-frequency that are different from the desired frequency (with dedicated frequency offset), Qoffset equals to QoffsetDedicatedfrequency, if valid.

Reference

- R2-19xxxxx, TS 36.304 draftCR, Reselection to a Cell with Valid Dedicated Frequency Offset, MediaTek Inc.

- TS 36.304