3GPP TSG-RAN WG2 #107bis	R2-1913071

# Chongqing, China, 14th – 18th October 2019

Agenda Item:	7.2.4 Transmission in preconfigured resources

Source:	Institute for Information Industry (III)

Title:	Further consideration on D-PUR aspect

Document for:	Discussion, Decision

1	Introduction

In previous RAN2 meetings, the release mechanism of D-PUR has been discussed. In this contribution, we discuss whether an enhancement for current scenarios is needed.

2	Discussion

The agreement of the D-PUR release mechanism in previous RAN2 meeting is given in [1] and [2] and captured below:

RAN2 #105 agreements:

- The UE must release the D-PUR when it does a RA procedure on a new cell.

RAN2 #107 agreements:

- If the UE performs EDT or moves to RRC_CONNECTED and comes back to RRC_IDLE in the same cell, PUR configuration remains valid unless specifically released or reconfigured by network or other triggers.

- PUR can be released explicitly by RRCConnectionRelease message and DL RRC response message (FFS message) of the D-PUR procedure.

According to agreements mentioned above, the D-PUR resource configured by the original cell remains valid until the UE performs a RACH procedure on a new cell when the UE moves to its neighbouring cell. However, this scenario is not described enough. There is no corresponding D-PUR resource mechanism for a slow moving NB-IoT UE, e.g. a tracker, to send periodic data. To send data, a slow moving NB-IoT UE has to perform a RACH procedure each time when it moves to a new cell. This NB-IoT UE has to complete data transmission within 10s even when it moves to a new cell with a CE level Two coverage. If dedicated resources can be pre-allocated among cells, the duration for each data transmission and overall signalling overhead can be reduced. Hence, dedicated D-PUR resource(s) across cells may be necessary.

Meanwhile, a UE can stay in the RRC_IDLE mode but has registered to the core network if this UE is connecting to a 5GC. It is possible that a 5GC can maintain a mobile NB-IoT UE information when this NB-IoT UE moves around different base stations. Hence, mechanisms for D-PUR used in mobile scenarios may be achievable.

Discuss the possibility to allocate dedicated D-PUR resource(s) among cells.

3	Conclusion

In this contribution, we discuss the D-PUR resource is released when a NB-IoT UE performs a RACH procedure in a new cell. However, the existing mechanism to allocate a dedicated D-PUR resource in one single cell would be not enough for moving scenarios. Based on the discussion above, we propose the following:

Proposal 1	Discuss the possibility to allocate dedicate D-PUR resource(s) among cells.

4	References

R2-1902233, RAN2 #105, Report NB-IoT breakout session

R2-1911513, RAN2 #107, Report NB-IoT breakout session