3GPP TSG-RAN2 Meeting #107bis 	R2-1913391

# Chongqing, China, 14 – 18 October, 2019

Title:	[Draft] LS on RRC establishment cause value in voice fallback from NR to E-UTRAN

Response to:

Release:	Rel-16

Work Item:	TEI-16

Source:	Qualcomm Incorporated [RAN WG2]

To:	CT1

Cc:

Contact Person:

#### Name:	Masato Kitazoe

E-mail Address:	mkitazoe [at] qti.qualcomm.com

1. Overall Description:

In the current RRC redirection-based voice fallback from NR to E-UTRAN, RAN2 understands that:

In EPS voice fallback:

The UE NAS indicates the call type “originating signalling” to initiate the TAU procedure in E-URAN. As a result, the UE RRC uses “mo-Signaling” as the cause value for RRC connection establishment in E-UTRAN.

In inter-RAT voice fallback:

The UE NAS indicates “mo-Signalling” as the establishment cause value to initiate the TAU procedure in E-UTRAN. The UE RRC directly uses the establishment cause value in RRC connection establishment in E-UTRAN.

RAN2 found that this prevents the target eNB from being able to prioritize the UE for voice call. RAN2 therefore agreed that the UE should instead use “mo-VoiceCall” as the establishment cause value in RRC Connection Request, and applied necessary changes to RAN2 specifications.

2. Actions:

To RAN2:

ACTION: RAN2 respectfully asks CT1 to take the above information into consideration and inform RAN2 if any issue is found.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN2 #108 		18 - 22 November 2019, 	Reno, US