3GPP TSG-RAN2 Meeting #107-Bis	R2-1913393

# Chongqing, China, 14-18 Oct 2019

Agenda item:		6.15

Source: 		Qualcomm Incorporated

Title: 		Too strong interference during CLI-RSSI/SRS-RSRP measurement

Document for:	Discussion/Decision

# Introduction

RAN2 was requested by RAN1 in LS R2-1905203 to define CLI-RSSI and SRS-RSRP measurements. RAN2#106 and RAN2#107 made some agreements however some issues are still open.

# Performing CLI-RSSI and SRS-RSRP measurements

In a network with dynamic TDD configuration, strong CLI may occur to a UE if it is close to multiple UEs from another cell that have strong UL transmissions colliding with its DL reception. In this case, the strong interference power may push the UE analog circuit to saturation. For the UE that is configured to measure the SRS-RSRP or the CLI-RSSI, in such case either the measurement is not accurate, or the UE is not even able to detect the SRS sequence ID.

If the interference is too strong, either the measurement is not accurate or the UE is not even able to detect the SRS sequence ID.

Therefore, UE should be allowed to indicate in the CLI SRS-RSRP/CLI-RSSI measurement report that the interference is too strong to measure.

UE can indicate ‘too strong interference to measure’ in CLI-RSSI/SRS-RSRP measurement result(s).

# Summary

Based on the above discussion, we have the following observation:

Observation 1.	If the interference is too strong, either the measurement is not accurate or the UE is not even able to detect the SRS sequence ID.

Based on the discussion and observation, we propose:

Proposal 1.	UE can indicate ‘too strong interference to measure’ in CLI-RSSI/SRS-RSRP measurement result(s).

# References

- R2-1905203/R1-1903836, RAN1 LS on RRC parameters for NR CLI-handling

- RP-190667, Status Report to TSG, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR