3GPP TSG-RAN2 Meeting #107-bis		    	 					 							R2-1913449

Chongqing, China, 14–18 October, 2019

Source: 		email discussion Rapporteur (ZTE Corporation)

Title: 	Summary of running MAC CR review issue list

Agenda item:	6.13.1

Document for: 	Discussion and Decision

# Review Issue list (Rev1)

| # | Brief description of the issue | Suggested resolution | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| 0 | ### 5.1.4a MsgB reception for 2-step CBRA …. 4> Once Msg3 is transmitted, proceed to Contention Resolution step (see subclause 5.1.5). [ZTE(EV)] The above sentence is not needed since the UE behaviour is obvious | Delete this sentence [Nokia] Agree. [LG] Agree | => Delete the sentence |
| 1 | ### General comments: It would be good to align the terminology, since in some places, e.g., 5.1.1: 1> if the BWP selected for Random Access procedure is configured with 2-step RACH resources as specified in TS 38.331 [5]: It’s named as 2-step RACH, while some other places, it’s named as 2-step CBRA: 2> perform the Random Access Resource selection procedure for 2-step CBRA (see clause 5.1.2a). | [OPPO]: We prefer to have the aligned terminology, e.g., 2-step CBRA, as aligned with stage 2 spce. [LG] : We have a similar view and prefer terminology ‘2-step CBRA’ aligned with other sub-clauses. [OPPO]:Regarding msgA and msgB terminolgy, it would be good to align them. In stage-2, we use MSGA and MSGB, it would be good to align the stage-2 spec terminology. [Ericsson] We believe the term should be “2-step RA” as we are doing 2-step Random Access and not 2-step Random Access Channel. RACH is an abbreviation for Random Access Channel. | - Agree to harmonise the wording in various places. - We need to keep the terminology a bit more generic, now that the plenary has agreed to also work on CFRA - Basically, same view as Ericsson => Agree to align the terminology => but, keep “2-step RA” for now and harmonise the terminology across the spec. |
| 2 | ### 5.1.2 Random Access Resource selection for 4-step CBRA/CFRA | [OPPO]: No strong view, but slightly prefer to not change the title here, since in 5.1.2a, you have explicitly mentioned it’s applied for “2-step CBRA”, so I think it’s clear this section will apply to 4-step CBRA and CFRA. [Nokia] We don’t prefer to change the names for existing subclauses. [LG] We have a same view as OPPO and Nokia. [Ericsson] We agree with Nokia. If the rapporteur feels a need to clarify that this clause is only for 2-step RA, an editor’s note could be added immediately following the name of the subclause. | => undo the change and keep the original title |
| 3 | ### 5.1.2a Random Access Resource selection for 2-step CBRA 1> determine the corresponding PUSCH resource to the selected preamble and PRACH occasion according to subclause x of TS 38.213 [6]; | [OPPO]: we’re not sure whether some MAC layer behaviour should be defined here for PO selection, since when UE performs PO selection, it should not only consider the configured mapping relationship but also the payload size, i.e., the payload size should match the size of PO. Maybe RAN1 can handle the entire PO selection part but it would be good if some editor notes are added here in case some MAC behaviour should be added later. [Nokia] From our point of view the proposed text is OK but we could indeed have an Editor’s note saying detailed procedural description about PUSCH resource selection will be added once agreed. [Ericsson] Agree with Nokia | - Agree, something is needed for payload size handling: - e.g: It may be necessary to select the preamble group based on the payload size (unless RAN1 agrees to use the UCI to indicate the payload size – FFS for now). - Given that these details are FFS, we can for now, add a note as proposed by Nokia. => add an Editor’s note for now |
| 4 | ### 5.1.3a MsgA transmission Editor's Note: The handling of the counters in this section are FFS for now. The description below is just for information and will be updated based on the input from RAN1 on how to handle the power ramping counters. Also, the calculation of RA-RNTI will also be updated based on further agreements (both in RAN1 and RAN2 on whether to use a new RNTI etc), so, this is also for information only … 2> obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer. | [OPPO]: we are ok on the editor notes here. But wonder we may need an agreements saying, e.g., the legacy counters can be reused for 2-step RACH. [OPPO]: regarding the msg3 buffer, it should be noted that using msg3 buffer is not agreed yet. It would cause some issue when fallback grant does not match the MAC PDU in the msg3, the UE would perform some behaviours specified for legacy 4-step RACH, however we have agreed it’s up to UE implementation to handle the mis-match case. So, it would be good to add an FFS here. | - Yes, whether legacy counters can be used is indeed FFS (as already captured in the current Editor’s note). - We can add that it is FFS whether to reuse Msg3 buffer. However, there are only two options: either to reuse Msg3 buffer or to define a new buffer for MsgA payload. It is unclear which is better (possibly it is just a matter of taste in the end). - The issues with legacy procedure when Ul grant size mismatch happens as mentioned here is a bit unclear to the rapporteur. We can discuss this further. => add a note that using Msg3 buffer is FFS |
| 5 | ### 5.1.4 Random Access Response reception (4-step CBRA and CFRA) | [OPPO]: No strong view, but slightly prefer to not change the title here, since in 5.1.2a, you have explicitly mentioned it’s applied for “2-step CBRA”, so I think it’s clear this section will apply to 4-step CBRA and CFRA. [Nokia] We don’t prefer to change the names for existing subclauses. [LG] We have a same view as OPPO and Nokia. [Ericsson] We agree with the above. If the rapporteur feels a need to clarify that this clause is only for 2-step RA, an editor’s note could be added immediately following the name of the subclause. | => Keep the original title |
| 6 | ### 5.1.4a MsgB reception for 2-step CBRA ### ... 1> monitor the PDCCH of the SpCell while the msgB-ResponseWindow is running; … 2> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider the Random Access procedure successfully completed. 2> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 3> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 4> stop MsgB-ResponseWindow; 4> consider this Random Access Response reception successful; 4> consider the Random Access procedure successfully completed. … 2> if the MsgB contains a MAC subPDU with Backoff Indicator: 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. 2> else: 3> set the PREAMBLE_BACKOFF to 0 ms. … 2> if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see subclause 5.1.3a): 3> stop MsgB-ResponseWindow; … HARQ operation is not applicable to the Random Access Response reception through PDCCH addressed to MsgB-RNTI. | Issue 6.1 [OPPO]: as agreed, the PDCCH monitoring behaviours are different for C-RNTI and CCCH SDU case. For C-RNTI case, UE should not only monitor msgB-RNTI scrambled PDCCH but also C-RNTI scrambled PDCCH. But for CCCH SDU case, UE only needs to monitor msgB-RNTI scrambled PDCCH. We may need to specify these two different behaviour in stage-3 spec. Issue 6.2: [OPPO]: regarding contention resolution for C-RNTI case, several comments: - In RAN2-106, we left an FFS and didn't handle this in RAN2#107, so we prefer to leave this to FFS. - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced - Issue 6.3: The current “if .. else if” branch only handles two cases, the first case is the uplink is in-sync case, and the other case is uplink is out-sync case. We’re wondering in the case when uplink is out-of-sync and there is uplink data arrival, what’s the UE behaviour. In legacy RACH procedure, UE obtains not only UL grant but also TAC (in RAR), so here, similarly, we may need to discuss whether we need to have both UL grant and TAC for the case when UL is out-of-sync and UE has UL data available. Issue 6.4: [OPPO]: It seems UE always apply the prioritized RACH when BI is received in MSGB, but we never discussed this aspects? [Ericsson] Prioritized RA is supported for 4-step and is supposed to be the baseline. We think Prioritized RA is supported for 2-step unless decided otherwise. Related to this it has been agreed to modify the Prioritized RA as part of TEI16 so simply copying the current text related to Prioritized RA might not be sufficient. [Ericsson] We agree with Oppo, the proposed text is not aligned with the agreements made. Issue 6.5: [OPPO]: In RAN2#107, we agreed that UE may stops monitoring PDCCH during the window if fallbackRAR is received. But the implemented text seems always mandate the UE to stop is fallbackRAR is received. Issue: 6.6: [OPPO]: In RAN2#107, we agreed HARQ feedback for msgB would be needed from RAN2 point of view. It seems the implemented text does not align with this agreements? [Ericsson] We agree with Oppo. The proposed text should be removed. If the text is kept it shall be written in active form, i.e. which node/entity does not apply HARQ operation. Also, what is “HARQ operation”? | Issue 6.1: => add additional text to describe this behaviour Issue 6.2: - Previously we left it ffs because there were proposals to use legacy RAR to provide timing advance, but now we don’t have this, so a new MAC CE is needed in any case to provide the TA. => keep the description about new MAC CE Issue 6.3: - When the UE is not in sync (TAT not running), even if UL data is pending (e.g. if BSR is included in MsgA etc), the UE anyway needs TAC. So, it seems including TAC is minimum. So, it seems the current wording (which is according to the agreements) is enough. But we can discuss this to confirm. => Discuss in RAN2 whether we need both an UL grant and TAC in case of UL data + Async case. Issue 6.4: => Remove the SCALING_FACTOR_BI, which is not applicable to CBRA. Issue 6.5: - More feedback on this is needed perhaps (although for now, we implemented it as “may” – i.e. deleted the “stop MsgB-ResponseWindow” - It is a bit unclear why the UE will continue monitoring MsgB window even after receiving fallbackRAR (given that it will anyway need to process the received fallbackRAR and proceed with the UL transmission of msg3). - Is this, for example, to cover the scenario where two UEs transmit the same preamble and collide, but the gNB is able to decode one of the payload but not the other? In this case, it is enough if the gNB includes the successRAR also for the UE for which the decoding has succeeded in the same MACPDU as the fallbackRAR. => Remove the “stop MsgB-ResponseWindow” but continue discussion per above. Issue 6.6: - It is a bit unclear to us whether the feedback for successRAR is same as HARQ feedback. This also depends on the RAN1 decisions in the end. For now, we can delete this sentence and add an Editor’s note: => Delete the sentence and add an Editor’s note to clarify that feedback for reception of successRAR is FFS pending RAN1 input. |
| 7 | #### 6.1.3.4a Timing Advance Command Type 2 MAC CE | [OPPO]: maybe we should wait the agreements on the following FFS: - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced | => Keep this description (see above issue 6.2) |
| 8 | ### [Huawei (GuoYinghao)] Regarding the general structure of the running CR, it is OK for us to have a section 5.1.4a to merge what is previous in section 5.1.4 and 5.1.5. But we are not sure how much difference there will be between section the RACH resource selection of 2-step/4-step (i.e., section 5.1.2 and 5.1.2a) and msgA/Preamble transmission (i.e., section 5.1.3 and 5.1.3a). We propose to leave this part to FFS and consider to merge these two sections to the R15 RACH procedure if there is no large difference. | Leave it as FFS regarding whether to create separate section for RACH resource selection and msgA transmission for 2-step RACH. Consider to merge these two sections to the R15 RACH procedure if there is no large difference. [Ericsson] We are fine with the proposed structure and we disagree with Huawei in this aspect. | - This is a matter of taste. Hence, no strong view on this. - My slight concern is that already, the current procedure section in 4-step RACH is fairly complex and adding the 2-step RACH procedures within it may reduce the readability of this part. => For now, keep the current structure as baseline |
| 9 | ### [Huawei (GuoYinghao)] Editorial issues: some of the text have the font size 10.5, such as the following, while all text should be 10. #### 6.1.3.4a Timing Advance Command Type 2 MAC CE The Timing Advance Command Type 2 MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.4a-1): - TAG Identity (TAG ID): This field indicates the TAG Identity of the addressed TAG. The TAG containing the SpCell has the TAG Identity 0. The length of the field is 2 bits; - Timing Advance Command: This field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6]. The size of the field is 12 bits; | change the size of the font to 10 | => fix the style |
| 10 | - rsrp-ThresholdSSB-2stepRach: an RSRP threshold for selection between 2-step RACH and 4-step RACH ### [Huawei(GuoYinghao)] The name of the above RRC parameter seems incorrect. It should be rsrp-ThresholdSSB-2stepRACH ? Also, RAN1 may give RAN2 a final table for the RRC parameters for 2-step RACH and we can finalize the name when we get the excel table from RAN1 | Change the name to rsrp-ThresholdSSB-2stepRACH, also put a bracket around the name to indicate that the exact name is FFS [Ericsson] As stated in the beginning we think the name of this work is “2-step RA”, hence we would also like to update the name accordingly. Also there are rules for how names in TS 38.331 are constructed and we should follow them. Putting brackets around contentious parameter/variable/field names is a good idea. | => Change the name to rsrp-ThresholdSSB-2stepRA => Add and Editor’s Note that the exact names are FFS |
| 11 | - rsrp-ThresholdSSB-2stepRach: an RSRP threshold for selection between 2-step RACH and 4-step RACH ### [Huawei(GuoYinghao)] I think the threshold is just for the selection between CBRA 2-step RACH and CBRA 4-step RACH. This is also independent from the decision in RANP on 2-step CFRA. | Change the description to “ selection between 4-step CBRA and 4-step CBRA” [LG] We are fine with this change for now. However, if RAN2 has enough time to discuss 2-step CFRA after completing 2-step CBRA, this issue needs to be dealt with. | - The current description is indeed not perfect (for instance, it is not good to introduce the terminology of 4-step RACH, which doesn’t yet exist) - On the other hand, it would be good to keep this a bit more generic to allow for CFRA addition later (i.e. do not narrow it down to 2-step CBRA as proposed in the comment) - Redefine as below (avoiding mentioning of 4-step RACH and avoiding restriction to CBRA only) => - rsrp-ThresholdSSB-2stepRACH: an RSRP threshold for selection of 2-step random access |
| 12 | ### 5.1.2 Random Access Resource selection for 4-step CBRA/CFRA [Huawei(GuoYinghao)] Since another section for the RACH resource selection for 2-step RACH has been added, we think the specification has been clear enough and we don't prefer to change the existing R15 spec. The same rationale also follows for section 5.1.4 | remove the added "for 4-step CFRA/CFRA" and also for title for section 5.1.4a [LG] We have same view. [Ericsson] This is same as issue 5. We agree with this issue. | => Keep the original title |
| 13 | 1> determine the corresponding PUSCH resource to the selected preamble and PRACH occasion according to subclause x of TS 38.213 [6]; ### [Huawei(GuoYinghao)] From our view, the PUSCH configuration for msgA will generally follow that of the configured grant. While the determination of PUSCH for configured grant is specified in the MAC specification, similarly, the PUSCH for msgA should be specified in the MAC specification. In addition, PUSCH resource selection probably should not be specified under the section for PRACH resource selection | Leave it as FFS in a different section from RACH resource selection and finalize the specification of PUSCH resource selection when the RAN1 agreements for this is fully determined. | See comment#3 (pasted below) - Agree, something is needed for payload size handling: - e.g: It may be necessary to select the preamble group based on the payload size (unless RAN1 agrees to use the UCI to indicate the payload size – FFS for now). - Given that these details are FFS, we can for now, add a note as proposed by Nokia. => add an Editor’s note for now |
| 14 | ### [Huawei(GuoYinghao)] It seems that the following part of condition for contention resolution is not aligned with the legacy LTE UE behavior. 1> if the C-RNTI MAC CE was included in MsgA: 2> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider the Random Access procedure successfully completed. 2> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 3> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 4> stop MsgB-ResponseWindow; 4> consider this Random Access Response reception successful; 4> consider the Random Access procedure successfully completed. In the last meeting, Huawei has proposed a tdoc R2-1910671 about the condition for contention resolution when C-RNTI is included in msgA. Please refer to Proposal 3-6 | specify the following condition for contention resolution when C-RNTI is included in msgA - For synchronized UE in RRC_CONNECTED and the 2-step RACH is triggered by MAC sublayer itself or by RRC sublayer, the UE considers the contention resolution successful if an UL grant for new transmission is included in msgB and PDCCH addressed to C-RNTI is received. - For synchronized UE in RRC_CONNECTED and the 2-step RACH is triggered by PDCCH order or BFR, the UE considers the contention resolution successful if PDCCH addressed to C-RNTI is received. - For un-synchronized UE in RRC_CONNECTED and 2-step RACH is triggered by PDCCH order or BFR, the UE considers the contention resolution successful if 12-bit TA command is received and PDCCH addressed to C-RNTI is received. - For un-synchronized UE in RRC_CONNECTED and 2-step RACH is triggered by MAC or RRC sublayer, the UE considers the contention resolution successful if 12-bit TA command and UL grant for new transmission are received and PDCCH addressed to C-RNTI is received. [LG] Regarding the last case (i.e., un-synchronized UE in RRC_CONNECTED and 2-step RACH is triggered by MAC or RRC sublayer) specified by Huawei, we are not sure why UE considers the contention resolution successful after receiving UL grant as well as 12-bit TA. We think that contention resolution is successful if un-synchronized UE receives 12-bit TA regardless of event triggering RA procedure. | Same as issue 6.3 above. Issue 6.3: - When the UE is not in sync (TAT not running), even if UL data is pending (e.g. if BSR is included in MsgA etc), the UE anyway needs TAC. So, it seems including TAC is minimum. So, it seems the current wording (which is according to the agreements) is enough. But we can discuss this to confirm. => Discuss in RAN2 whether we need both an UL grant and TAC in case of UL data + Async case. |
| 15 | ### [Huawei(GuoYinghao)] one minor comment, the title of 5.1.4a might be changed to "MsgB reception and contention resolution for 2-step CBRA" | change the title to "MsgB reception and contention resolution for 2-step CBRA" [Ericsson] We don’t think there is a need to have separate sections for CBRA and CFRA. [Huawei] reply to E//, the intention here is to add “contention resolution” because it is not only reception of msgB. Not to confuse with any discussion related to CFRA | - Not sure I understood the comment from Ericsson (to be checked) => Okay in general => Change the “CBRA” to “RA” to keep this generic though per above |
| 16 | 1> if the C-RNTI MAC CE was included in MsgA: 2> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider the Random Access procedure successfully completed. 2> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 3> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 4> stop MsgB-ResponseWindow; 4> consider this Random Access Response reception successful; 4> consider the Random Access procedure successfully completed. [Huawei(GuoYinghao)] While according to the running CR, the condition is only applicable for the PTAG, we think it should also be applicable for STAG, since it can be triggered both internally by UE and externally by the PDCCH order from the network. | When TAT for the STAG is running, the UE does not need to take receiving TAC type2 MAC CE as a condition for contention resolution [Nokia] So far only CBRA is agreed and this applies only for SpCell, ie., PTAG. Hence, the proposal by the rapporteur is correct. [LG] We have the same understanding as Nokia. [Ericsson] We agree with Nokia. | - As Nokia/Ericsson pointed out this seems correct. Even in case of CFRA, this is only agreed for HO. So, referring to PTAG here seems enough. => Keep the sentence as is |
| 17 | 1> if a downlink assignment has been received on the PDCCH for the MsgB-RNTI and the received TB is successfully decoded: ### [Huawei(GuoYinghao)] For this the condition "CCCH SDU is included in msgA" should be added | [Nokia] No, CONNECTED mode UE has to also decode MsgB-RNTI for possible fallback / backoff. The rapporteur proposal is hence correct. [LG] We have the same understanding as Nokia. [Huawei] OK, I know the condition is already implicit in the next condition. No strong view on this. | - Yes, this is for fallback as Nokia/LG say. => Keep the sentence as is |
| 18 | 3> set the PREAMBLE_BACKOFF to 0 ms. ### [Huawei(GuoYinghao)] there should not be unit for this value | remove the "ms" [Nokia] We use “ms” also in 4-step. [Ericsson] Agree with Nokia. | - Keep it unless 4-step case is also changed => Keep the sentence as is |
| 19 | 2> if the MsgB contains a MAC subPDU with fallbackRAR; and 2> if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see subclause 5.1.3a): 3> stop MsgB-ResponseWindow; ### [Huawei(GuoYinghao)] In the last meeting, the following agreement was made: - For MsgA with C-RNTI or CCCH SDU, upon receiving fallbackRAR corresponding to random access preamble transmitted by UE, UE may stop monitoring PDCCH addressed to msgB-RNTI. | follow the legacy NR spec and also add a note for this. [Ericsson] Same as one of the issue 6. | Issue 6.5: - More feedback on this is needed perhaps (although for now, we implemented it as “may” – i.e. deleted the “stop MsgB-ResponseWindow” - It is a bit unclear why the UE will continue monitoring MsgB window even after receiving fallbackRAR (given that it will anyway need to process the received fallbackRAR and proceed with the UL transmission of msg3). - Is this, for example, to cover the scenario where two UEs transmit the same preamble and collide, but the gNB is able to decode one of the payload but not the other? In this case, it is enough if the gNB includes the successRAR also for the UE for which the decoding has succeeded in the same MACPDU as the fallbackRAR. => Remove the “stop MsgB-ResponseWindow” but continue discussion per above. [Huawei] agree, for us the reason behind this is not crystal clear either. Continue the discussion in the next meeting |
| 20 | 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers; and 1> if the C-RNTI MAC CE was included in MsgA: ====omittted==== 1> if a downlink assignment has been received on the PDCCH for the MsgB-RNTI and the received TB is successfully decoded: [Huawei(GuoYinghao)] The notification of PDCCH transmission of the SpCell is applicable for both the case of C-RNTI and msgB-RNTI. So the highlighted paragraph in yellow for msgB-RNTI should be one level lower than the condition in green | Change the level of the highlighted yellow spec. [Ericsson] We could also change 1> if a downlink assignment has been received on the PDCCH for the MsgB-RNTI and the received TB is successfully decoded: To 1> else if a downlink assignment has been received on the PDCCH for the MsgB-RNTI and the received TB is successfully decoded: This is more similar to NR Rel-15. | => fixed as proposed by HW - Need to check if “else” is needed on top as proposed by Ericsson |
|  | 2> if the MsgB contains a MAC subPDU with success RAR; and 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: 3> set the C-RNTI to the value of C-RNTI in the successRAR; 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 4> process the received Timing Advance Command (see subclause 5.2); 4> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 3> if the next MAC subPDU contains a SRB SDU; 4> forward the SRB SDU to higher layer; 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider this Random Access procedure successfully completed. [Huawei(GuoYinghao)] For this part, we think it only applies to the case when CCCH SDU is included in msgA, so a condition that " CCCH SDU is included in msgA should be added | add the condition that "CCCH SDU is included in msgA". [Ericsson] We don’t think this change is necessary as the matching assumes the presence of a CCCH SDU in msgA | - for now implemented as proposed by Huawei as it doesn’t hurt either way (no strong view). => agree the proposal from HW |
| 21 | 2> if the MsgB contains a MAC subPDU with success RAR; and 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: 3> set the C-RNTI to the value of C-RNTI in the successRAR; 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 4> process the received Timing Advance Command (see subclause 5.2); 4> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 3> if the next MAC subPDU contains a SRB SDU; 4> forward the SRB SDU to higher layer; 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider this Random Access procedure successfully completed. [Huawei(GuoYinghao)] For the highlighted part, the condition should be the "the contention resolution MAC CE matches the first 48 bits of the CCCH SDU" | change the condition to " the CR MAC CE matches the first 48 bits of the CCCH SDU" [Ericsson] This was discussed as part of Rel-15 corrections (to 4-step) and not agreed. No need to add it here. [Huawei] Actually, some agreements have been captured in the chairman note in the discussion for R15 CP corrections. R2-1911510 Correction on contention resolution with CCCH SDU in msg3 Huawei, HiSilicon CR Rel-15 38.321 15.6.0 0655 1 F NR_newRAT-Core - ZTE think this is already captured in the contention resolution MAC CE text. LG agrees and think this CR is not needed. - Huawei think the text is not strictly correct but would be ok to just capture in Chair notes R2 confirms that the CR reflects the intended behaviour No support to change anything, Not pursued And also, in LTE, it is “MAC CE matches the first 48bits of the CCCH SDU”. But as correctly pointed out by the rapporteur, we may want to align the two types of RACHes although the intention may be correct, at least from our understanding | - It would be good to align this text with what we have for 4-step RACH. => keep the existing wording which is aligned with the corresponding text in 4-step RACH. |
| 22 | 2> if the MsgB contains a MAC subPDU with success RAR; and 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: 3> set the C-RNTI to the value of C-RNTI in the successRAR; 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 4> process the received Timing Advance Command (see subclause 5.2); 4> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 3> if the next MAC subPDU contains a SRB SDU; 4> forward the SRB SDU to higher layer; 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider this Random Access procedure successfully completed. [Huawei(GuoYinghao)] No need for indicating the preamble power to the lower layer | Remove the paragraph"4> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);" [Ericsson] Question to Huawei; In 4-step we indicate the power to lower layer. Why is there no need to do it here? [Huawei] We are fine that the UE indicate these to the lower layer when fallbackRAR is received, as in this case, there is still a msg3 to be transmitted. However, we are not fine for the UE to indicate to the lower layer when successRAR is received, as the paragraph in the left. But indeed, for the indication of power at the msgA transmission, maybe the MAC should not indicate a lump-sum power to the UE, but the indication should be like here: the initial received target power and power ramping are indicated separately. We can discuss about this. | - We, (like Ericsson), also think this might be needed for determining the tx power of the PUSCH. - We can keep this for now but add an editor’s note to clarify that this is FFS. => keep the text but add an Editor’s note that this is FFS pending RAN1 input |
| 23 | [Huawei(GuoYinghao)] msgB reception for SI request is not added | add a similar procedure as R15 3> if the Random Access Response includes a MAC subPDU with RAPID only: 4> consider this Random Access procedure successfully completed; 4> indicate the reception of an acknowledgement for SI request to upper layers. [LG] Since MSG3 based request for other SI is only supported in 2-step RACH, the text should be modified as follows: 3> if this Random Access procedure was initiated for SI request: 4> indicate the reception of an acknowledgement for SI request to upper layers. 3> else: 4> set the C-RNTI to the value of C-RNTI in the successRAR; [Ericsson] We have a similar concern. Perhaps the rapporteur could clarify why this was not included? | - Agree with LG proposal => cover the case of Msg3 based SI Req |
| 24 | 3> consider this Random Access Response reception successful; [Huawei(GuoYinghao)] all the "RAR reception" in this section should be "msgB reception" | change "RAR reception" in this section to "msgB reception" [Ericsson] We agree with Huawei, unless the rapporteur had a specific thought behind this. | -I thought that the problem with using MsgB here is that the “MsgB reception” is not applicable to the C-RNTI case. “Random access response” on the other hand is more generic and covers both C-RNTI case (where there is no MSGB) and normal case. => Keep the current wording [Huawei] From our view, msgB is not only for the case of CCCH SDU included in msgA, but also includes the case of C-RNTI, as we have discussed this in the stage-2 running CR. Note that msgB can be both uplink grant or downlink assignment, but I don't think there will not be any issue to call it msgB reception even if it is a UL grant. msgB essentially is a signal for contention resolution. So, even if it is an ul grant, it is still a reception of contention resolution signal. |
| 25 | HARQ operation is not applicable to the Random Access Response reception through PDCCH addressed to MsgB-RNTI. [Huawei(GuoYinghao)] This contradicts the agreement in the last meeting => HARQ feedback for msgB would be needed from RAN2 point of view | specify the previous agreement [Ericsson] Same as one of the issue 6. | Issue 6.6: - It is a bit unclear to us whether the feedback for successRAR is same as HARQ feedback. This also depends on the RAN1 decisions in the end. For now, we can delete this sentence and add an Editor’s note: => Delete the sentence and add an Editor’s note to clarify that feedback for reception of successRAR is FFS pending RAN1 input. |
| 26 | 1> perform the BWP operation as specified in clause 5.15; 1> if the RSRP of the downlink pathloss reference is above rsrp-ThresholdSSB-2stepRach; and 1> if the BWP selected for Random Access procedure is configured with 2-step RACH resources as specified in TS 38.331 [5]: 2> select 2-step CBRA procedure; Editor’s Note: The initialization of variables for 2-step RACH to be added here 2> perform the Random Access Resource selection procedure for 2-step CBRA (see clause 5.1.2a). 1> else: (i.e. for 4-step CBRA/CFRA) [Huawei(GuoYinghao)] Whether to allow 2-step RACH only configuration on a certain BWP is FFS and this impacts BWP switching | BWP switching and whether 2-step RACH only configuration is possible should be FFS [Ericsson] we should first determine general cases before looking at variants with 2-step only or where no RACH config at all is available for a BWP | - To discuss whether 2-step RACH can be configured on a BWP without 4-step RACH. - Add Editor’s note that this is FFS => Add Editor’s Note: It is FFS whether 2-step CBRA resource can be configured on a BWP without 4-step CBRA resource. If it is allowed, then some extra description is needed to address such case. |
| 27 | The MAC entity shall: 1> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 2> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 1> else: 2> select any SSB. [Huawei(GuoYinghao)] It appears what the parameter for 4-step RACH is reused here, while this has not been discussed before | FFS whether this parameter rsrp-ThresholdSSB is the same or different between 2-step RACH/4-step RACH | - Yes, reuse of the counters and timers is FFS pending RAN1 input. => Editor's Note: All the parameters and counters mentioned in the following section are FFS, and will be updated based on RAN1&RAN2 agreements. |
| 28 | [Huawei(GuoYinghao)] There have been the different wordings for the same thing in the following part of the running CR, we suggest to unify the wording as "UL grant configured for msgA payload" Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined as specified in subclause 5.1.2a for the transmission of the MsgA payload. ======= 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant or a transmission of the MsgA payload: ======= 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell or for transmission of MsgA payload in case of 2-step RACH: ===== Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response or for UL transmission for MsgA payload, HARQ process identifier 0 is used. === 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or 2> if the uplink grant was received in a Random Access Response; or 2> if the uplink grant was configured for MsgA transmission for 2-step CBRA; or | unify the wording as " UL grant configured for msgA payload " [Ericsson] We support unifying the wording. | => aligned the wording |
| 29 | ### 5.1.1 Random Access procedure initialization 1> if the RSRP of the downlink pathloss reference is above rsrp-ThresholdSSB-2stepRach; and 1> if the BWP selected for Random Access procedure is configured with 2-step RACH resources as specified in TS 38.331 [5]: 2> select 2-step CBRA procedure; [Nokia-1] It seem more natural if the availability of 2-step RACH is checked before comparing to threshold. 1> else: (i.e. for 4-step CBRA/CFRA) [Nokia-2] BFR is agreed to be supported for 2-step as well whereas the BFR in the running CR applies to 4-step only. | [Nokia-1] Swap the order of the conditions. [Nokia-2] Include BFR support for 2-step. [Ericsson] We agree with the Nokia comments. | => okay to swap the order of conditions =>Addition of BFR is FFS |
| 30 | ### 5.1.2a Random Access Resource selection for 2-step CBRA [Nokia-1] It is obvious from the description this applies only for CBRA, hence, we don’t prefer to cultivate this abbreviation where it is not absolutely necessary. Furthermore, we anyway would need to remove this when CFRA support is added whether or not it is in this release or in the future. (…) 1> select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB for 2-step RACH; [Nokia-2] Is the intention to say “and 2-step Random Access”? | [Nokia-1] We can just have the subclause as “Random Access Resource selection for 2-step Random Access”. [Nokia-2] “…from the Random Access Preambles associated with the selected SSB and 2-step Random Access;” [Ericsson] We support the Nokia proposal. | => Agree to make the title and the text generic (more so because of addition of CFRA now) How about : “1> select a Random Access Preamble randomly with equal probability from the 2-step Random Access Preambles associated with the selected SSB;” => implemented as above, can be changed if better wording is found. |
| 31 | ### 5.1.3a MsgA transmission 1> if SSB selected is not changed from the selection in the last Random Access Preamble transmission: [Nokia-1] Should it be the last MSGA transmission? RA-RNTI [Nokia-2] Since no details have been agreed, we could leave this part out. | [Nokia-1] Fix to “..in the last MSGA transmission”. [Ericsson] We are not sure about this. MsgA consists of a preamble and a PUSCH, so the existing text would be ok. [Huawei] Same concern as E//. Some RAN1 confirmation on this is needed [Nokia-2] Could use “MsgB-RNTI” for now and have an Editor’s note the MsgB-RNTI computation will be added once agreed. [Ericsson] We agree with Nokia. | => Depends on whether the SSB selection is common to preamble and PUSCH part. Propose to leave this as is for now and we can discuss this based on RAN1 output => Okay |
| 32 | ### 5.1.4a MsgB reception for 2-step CBRA [Nokia-1] See comments above to 5.1.2a, ie, prefer not to use “CBRA” 3> consider this Random Access Response reception successful; [Nokia-2] we receive MsgB in all cases under this sub-clause. 3> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: [Nokia-3] We don’t currently have “Type 1” TAC MAC CE. 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. [Nokia-4] No prioritized RA has been yet agreed for 2-step. 2> if the MsgB contains a MAC subPDU with fallbackRAR; and 2> if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see subclause 5.1.3a): [Nokia-5] “fallbackRAR” is not currently defined anywhere. 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: [Nokia-6] Seems this condition is not needed at all for now regardless even if RAN would agree on CFRA as it would be only performed for HO? 2> if the MsgB contains a MAC subPDU with success RAR; and 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: [Nokia-7] Same comment as in [Nokia-5] 3> set the C-RNTI to the value of C-RNTI in the successRAR; [Nokia-8] Editorial. 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: [Nokia-9] Same as [Nokia-6]. 3> if the next MAC subPDU contains a SRB SDU; 4> forward the SRB SDU to higher layer; [Nokia-10] seems obvious? 1> if MsgB -ResponseWindow configured in RACH-ConfigCommon-2stepRACH expires, and the Random Access Response Reception has not been considered as successful based on descriptions above: [Nokia-11] Seems the yellow highlight is not needed since we use “MsgB-ResponseWindow”? For green highlight, this is “MsgB reception”. 3> if the Random Access Preamble is transmitted on the SpCell: [Nokia-12] Unnecessary condition for now without CFRA. | 32.1 [Nokia-1] Can just use “MsgB reception for 2-step Random Access” 32.2 [Nokia-2] “Consider this MsgB reception successful” [Ericsson] Same as one of the Huawei issues. 32.3 [Nokia-3] No strong views on this but would just “Long TAC MAC CE” suffice? [Ericsson] Suggest put the name in brackets for now. 32.4 [Nokia-4] Remove “multiplied with SCALING_FACTOR_BI” from the sentence for now. [Ericsson] Same as one of the issue 6. We disagree with Nokia. 32.5 [Nokia-5] We suppose fallbackRAR will be defined in the MAC PDU section when we agree the fallbackRAR content and PDU format? Editorial proposal: “if the MsgB contains a fallbackRAR MAC subPDU; and” [Ericsson] Add some Editorial note. We think this will clear up once the formats are in place. 32.6 [Nokia-6] Remove the condition and correct indention of the subsequent procedural text to “3>”. 32.7 [Nokia-7] Same comment as for [Nokia-5]. BTW, it seems that we could correct here to the “…first 48 bits of the CCCH SDU” which should have been also there for 4-step. Editorial proposal: “if the MsgB contains a successRAR MAC subPDU; and” 32.8 [Nokia-8] remove the second “C-RNTI”: 3> set the C-RNTI to the value received in the successRAR; [Ericsson] We agree. 32.9 [Nokia-9] Same as [Nokia-6] 32.10 [Nokia-10] No strong view but could just remove the part related to SRB SDU. [LG] Regarding Nokia-10, we also suggest removing this part because RAN2 hasn’t made any agreements on SRB SDU of msgB, [Ericsson] We agree the current text is not optimal. An alternative would be to use some legacy text, e.g. “deliver to the disassembly and demultiplexing entity” or something. 32.11 [Nokia-11] Could just remove the yellow highlighted part. Change to “…and the MsgB reception has not been…”. [Ericsson] We also don’t understand the need to highlight which IE carries the msgB-ResponseWindow. Perhaps the rapporteur can explain or we can remove it. We agree with Nokia on the RAR/MsgB part. However, with a proper definition of MsgB, this will sort it self out (i.e. if there is a RAR in MsgB). [Nokia-12] Remove the condition. | 32.1: => Okay, more generic (includes CFRA) 32.2: Seems same as comment 24 from HW -I thought that the problem with using MsgB here is that the “MsgB reception” is not applicable to the C-RNTI case. “Random access response” on the other hand is more generic and covers both C-RNTI case (where there is no MSGB) and normal case. => Keep the current wording [Huawei] See my comment above. No strong view though 32.3 => No strong view, but kept it as is for now as it seems to match other fields such as PHR type2 etc, but can be changed later if more companies prefer it. [Huawei] A more straightforward and self-explanatory wording would be “12-bit TAC MAC CE” 32.4 => removed (also see 6.4 above) - To E///: we can discuss if prioritization is applicable to 2-step RA 32.5 => define fallbackRAR format (e.g. to reuse legacy format based on the outcome of email discussion 107#68) Editorial proposals => okay 32.6 => Keep it. seems it is cleaner to encapsulate these bunch of conditions for the serving cell in a conditional section. No strong view though 32.7 => keep the current text as it is aligned with Rel-15 text (if we change both 4-step and 2-step RACH text at the same time, this is also okay) => Editorial proposals seem fine 32.8 => okay 32.9 => Keep it. seems it is cleaner to encapsulate these bunch of conditions for the serving cell in a conditional section. No strong view though 32.10: - No strong view, but looking at LG’s comment, it seems it might be actually helpful to keep this (i.e. it is unclear what additional agreements are needed here – obviously it has to be passed to upper layers if this is not clear then we better keep this) => keep the text 32.11: => okay 32.12: => okay. can be removed even with CFRA because it is only for HO |
| 33 | ### 5.1.5 Contention Resolution 3> else if 2-step CBRA procedure was selected; and [Nokia-1] Similarly to above, can just refer to Random Access 3> if msgATransMax is configured and PREAMBLE_TRANSMISSION_COUNTER < (msgATransMax + 1): 4> perform the Random Access Resource selection for 2-step CBRA procedure (see clause 5.1.2a) after the backoff time. 3> else: (i.e. 4-step CBRA or PREAMBLE_TRANSMISSION_COUNTER ≥ msgATransMax + 1) 4> perform the Random Access Resource selection for 4-step CBRA/CFRA procedure (see clause 5.1.2) after the backoff time. [Nokia-2] Seems to become clumsy this way. | 33.1 [Nokia-1] Fix to “else if 2-step Random Access procedure was selected; and” [Ericsson] We agree with Nokia. 33.2 [Nokia-2] Similarly to 5.1.4a section, flip the order and put first condition as: 3> if msgATransMax is configured, and PREAMBLE_TRANSMISSION_COUNTER = msgATransMax + 1: which avoids using the long “i.e.” sentence after the “else:”. [Ericsson] We agree with Nokia. We think i.e. after else should be avoided as it is hard to maintain in the future. Also the indentation/style is incorrect. | => both look okay |
|  | ## 5.2 Maintenance of Uplink Time Alignment [Nokia] The way the change is implemented in this section now allows Type 2 MAC CE to be transmitted in response to 4-step CFRA which seems to have not been agreed so far (although this might be beneficial to allow.. 😊)? | [Nokia] For now, we could add a separate condition for the Type 2 (Long??) MAC CE which can be received only when C-RNTI MAC CE was included in MAC CE. Proposed resolution: First change to allow normal TAC in MsgB for fallbackRAR: 1> when a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MsgB for an SpCell: Second change with the separate condition for Type 2: 1> when a Timing Advance Command Type 2 is received in response to a MsgA transmission including C-RNTI MAC CE as specified in clause 5.1.5a: 2> apply the Timing Advance Command for PTAG; 2> start or restart the timeAlignmentTimer associated with PTAG. | => okay |
| 34 | ### 5.4.1 UL Grant reception 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell or for transmission of MsgA payload in case of 2-step RACH: [Nokia] “for” seems not to work here and unnecessary condition ”in case of 2-step RACH” | [Nokia] Change “for” to “with a” and remove the highlighted condition. [Ericsson] We agree with Nokia. | - Agree the sentence is now a bit too long with multiple Or conditions and hard to understand. => Go with the proposal from Nokia |
| 35 | #### 5.4.2.1 HARQ Entity 2> if the uplink grant was configured for MsgA transmission for 2-step CBRA; or (…) 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant configured for 2-step RACH was selected; or: [Nokia] Seems desirable to align the terminology with the previous section. | [Nokia] Proposed wording: 2> if the uplink grant was configured for transmission of MsgA payload; or (…) 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant configured for transmission of MsgA payload was selected; or [Ericsson] We support the Nokia proposal. | => okay (also aligned with the wording of “determined” grant per the other comments) |
| 36 | #### 6.1.3.4a Timing Advance Command Type 2 MAC CE - TAG Identity (TAG ID): This field indicates the TAG Identity of the addressed TAG. The TAG containing the SpCell has the TAG Identity 0. The length of the field is 2 bits; [Nokia] Prefer to remove the TAG ID for now. This does not depend on CFRA as that would be only applied for HO in case of Rel-16 and hence only SpCell (PTAG). | [Nokia] Remove the TAG ID. | => okay (since only HO case was agreed for CFRA at plenary, we can delete this now) |
| 37 | ### 6.2.2a MAC subheader for MsgB ### 6.2.3a MAC payload for Random Access Response for 4-step CBRA [Nokia] Seems that we should have subheaders for each RAR type in MsgB as well as payload per RAR type. And this should be 2-step. | [Nokia] Either allocate multiple subclauses for different MAC subheader types or then use “MAC subheaders for MsgB” and have separate subclauses under this for successRAR, fallbackRAR, backoff, etc. Fix to “2-step Random Access” or “MsgB”. | - first need to define fallbackRAR based on the other email discussion outcome (reuse legacy format). Then we can update this part accordingly. => okay (details to be added after the email discussion on 107#68 progresses) |
| 38 | ### [Sharp] The transmission of the contents of the MSG3 buffer in PUSCH resource is described in sections 5.4.2. So in 5.1.3a, only preamble is transmitted. ### 5.1.3a MsgA transmission 1> instruct the physical layer to transmit the MsgA using the selected PRACH occasion, corresponding RA-RNTI, PREAMBLE_INDEX, PREAMBLE_RECEIVED_TARGET_POWER. NOTE: The MsgA transmission includes the transmission of the contents of the Msg3 buffer in the PUSCH resource corresponding to the selected PRACH occasion and PREAMBLE_INDEX (see TS 38.213 [6]) ### ----------------------------------------------------- ### 5.4.2 HARQ operation #### 5.4.2.1 HARQ Entity (…) 2> if the uplink grant was configured for MsgA transmission for 2-step CBRA; or 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant configured for 2-step RACH was selected; or: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 3> if a MAC PDU to transmit has been obtained: 4> deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process; 4> instruct the identified HARQ process to trigger a new transmission; #### 5.4.2.2 HARQ process (…) If the HARQ entity requests a new transmission for a TB, the HARQ process shall: 1> store the MAC PDU in the associated HARQ buffer; 1> store the uplink grant received from the HARQ entity; 1> generate a transmission as described below. To generate a transmission for a TB, the HARQ process shall: 1> if the MAC PDU was obtained from the Msg3 buffer; or 1> if there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer: 2> instruct the physical layer to generate a transmission according to the stored uplink grant. | 38.1 [Sharp] 1> instruct the physical layer to transmit the MsgA preamble using the selected PRACH occasion, corresponding RA-RNTI, PREAMBLE_INDEX, PREAMBLE_RECEIVED_TARGET_POWER. | 38.1 - This depends on how we model it. Currently, the modelling is such that msg3 transmission is only initiated upon either fallback or with 4-step RACH. In case of 2-step RACH, there is no “msg3 transmission”. Instead, the payload in msg3 buffer is transmitted in PUSCH occasion which is part of msgA. However, the physical layer needs to get the resource for both preamble and the associated PUSCH at once. => Keep the current description (can be revisited later) |
|  | ## [Sharp] Now there could be 3 types of UL grant, received, configured and determined. The treatment of a “determined” UL grant is missing ## 5.4 UL-SCH data transfer ### 5.4.1 UL Grant reception Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined as specified in subclause 5.1.2a for the transmission of the MsgA payload. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion: 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: (…) 1> else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI: (…) For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall: 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell or for transmission of MsgA payload in case of 2-step RACH: (…) | 38.2 [Sharp] The same treatment with UL grant received in RAR could be used for the determined UL grant. i.e. 2> deliver the uplink grant and the associated HARQ information to the HARQ entity. 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: or 1> if an uplink grant has been determined for transmisison of the MsgA payload 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant or a transmission of the MsgA payload: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI. 2> if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant: 3> start or restart the configuredGrantTimer for the correponding HARQ process, if configured. 2> deliver the uplink grant and the associated HARQ information to the HARQ entity. | 38.2 (same as above it seems?) - In general, I agree that we need to make the modelling of MSGA payload transmission better. Currently the text is a bit inconsistent about “determining the UL grant” at some places. For now an editor’s note is added that this needs to be fixed (in section 5.4.1) => keep the current wording, but agree that this needs to be fixed later. |
| 39 | ## [Sharp] The ul grant for MSGA payload is determined in RA as described in 5.4.1. Here the same description is preferred in the yellow part ### 5.4.2 HARQ operation #### 5.4.2.1 HARQ Entity 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or 2> if the uplink grant was received in a Random Access Response; or 2> if the uplink grant was configured for MsgA transmission for 2-step CBRA; or 2> if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant configured for 2-step RACH was selected; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery: (…) | [Sharp] e.g. 2> if the uplink grant was configured determined for MsgA transmission for 2-step CBRA; 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant configured for MsgA transmission for 2-step RACH CBRA was selected determined | -It seems the main problem is the terminology to use “configured” which may be confused with the “configured grant”. We can change the wording to determined as proposed to be clear… May need further fine tuning later. =>seems okay (general wording in this section is now aligned) |
| 40 | ### 5.1.4a MsgB reception for 2-step CBRA 2> if the MsgB contains a MAC subPDU with success RAR; and 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: 3> set the C-RNTI to the value of C-RNTI in the successRAR; 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 4> process the received Timing Advance Command (see subclause 5.2); [LG #1] We think that ‘else’ should be added to this yellow highlighted condition as UE doesn’t need to check this part if receiving fallback indication. | [LG #1] Suggest adding ‘else’ 2> else if the MsgB contains a MAC subPDU with success RAR; and 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: 3> set the C-RNTI to the value of C-RNTI in the successRAR; 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 4> process the received Timing Advance Command (see subclause 5.2); | - In general, I agree, however, some companies are saying that it is not mandatory for the UE to stop monitoring the MSGB window even after receiving fallback indication (e.g. the current agreement has a “may”). So, to me it is a bit unclear which is the correct behavior. May be, we should agree to fix the UE behavior one way or the other. - For now, we have implemented this as suggested by LG => okay |
| 41 | ### 5.1.4a MsgB reception for 2-step CBRA 2> else: 3> select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF; 3> if msgATransMax is configured, and PREAMBLE_TRANSMISSION_COUNTER = msgATransMax + 1: 4> perform the Random Access Resource selection procedure for 4-step CBRA/CFRA (see subclause 5.1.2) after the backoff time. 3> else: 4> perform the Random Access Resource selection procedure for 2-step CBRA (see subclause 5.1.2a) after the backoff time. [LG #2] For switch to 4-step CBRA, should UE apply backoff time? As this backoff is to distribute 2-step CBRA attempts, UE may not need to wait until backoff time expiry. | [LG #2] Suggest removing yellow highlight. | - the backoff time should be common to both cases (we can leave this FFS if companies think an explicit agreement is missing). => add editors note that this is FFS |
|  | ### 5.1.5 Contention Resolution 3> else if 2-step CBRA procedure was selected; and 3> if msgATransMax is configured and PREAMBLE_TRANSMISSION_COUNTER < (msgATransMax + 1): 4> perform the Random Access Resource selection for 2-step CBRA procedure (see clause 5.1.2a) after the backoff time. 3> else: (i.e. 4-step CBRA or PREAMBLE_TRANSMISSION_COUNTER ≥ msgATransMax + 1) 4> perform the Random Access Resource selection for 4-step CBRA/CFRA procedure (see clause 5.1.2) after the backoff time. [LG #3] Same comment as LG #2 | [LG #3] Suggest changing as the followings: 3> else if 2-step CBRA procedure was selected; 4> if msgATransMax is configured and PREAMBLE_TRANSMISSION_COUNTER = msgATransMax + 1: 5> perform the Random Access Resource selection procedure for 4-step CBRA/CFRA (see subclause 5.1.2). 4> else: 5> perform the Random Access Resource selection for 2-step CBRA procedure (see clause 5.1.2a) after the backoff time. 3> else: (i.e. 4-step CBRA) 4> perform the Random Access Resource selection for 4-step CBRA/CFRA procedure (see clause 5.1.2) after the backoff time. |  |
| E001 | ## 3.1 Definitions [Ericsson] We agree with the rapporteur that a definition for msgB is required. Maybe also MsgA. The definition should be aligned with stage-2.v | [Ericsson] What about: MsgA: Message consisting of a preamble transmission on PRACH and a transmission on UL-SCH (the MsgA payload) as part of a Random Access procedure. MsgB: Message transmitted on DL-SCH as a response to MsgB as part of a Random Access procedure. The more detailed explanation of the contents of MsgB can be left to subclause 6. | - In general this looks fine. - We have not added this yet because it is unclear if we need these here or both here and in stage-2 etc. May be, we can discuss this later once the stage-2 is stable. => Discuss the addition of definitions later |
| E002 | ### 5.1.3a MsgA transmission .. 2> obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer. [Ericsson] It seems inherently wrong to put MsgA in the Msg3 buffer but it might be correct depending on the underlying design and assumptions. | [Ericsson] Perhaps the rapporteur could clarify the design behind this choice of not creating a “MsgA buffer”? We also note the comment from the rapporteur in subclause 5.4.2.2. Is it so that MsgA payload and Msg3 are the same? If so, that can be used in the definition of MsgA and we would not need any “Msg3 or the MsgA payload” (as in subclause 5.14 for example). If the there is a case where the msgA MAC PDU is different from a possible msg3 MAC PDU a differentiation would be needed. | - Currently, the assumption is that the MSGA payload is simply stored in the MSG3 buffer in the UE, this then simplifies the handling of fallback. However, it is also possible to define a new MSGA payload buffer (which then needs to be copied into the MSG3 buffer in case of fallback). - No strong view on how we model this and happy to hear other options on how to do this. For now, we kept this as is. => keep as is and continue discussion |
| E003 | ### 5.1.4a MsgB reception for 2-step CBRA .. 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers; and 1> if the C-RNTI MAC CE was included in MsgA: 2> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> consider the Random Access procedure successfully completed. 2> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 3> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 4> stop MsgB-ResponseWindow; 4> consider this Random Access Response reception successful; 4> consider the Random Access procedure successfully completed. [Ericsson] It is unclear what happens if the C-RNTI was included in MsgA, and the PDCCH transmission is an uplink grant but the TA timer is not running. | [Ericsson] Clarify what would be the UE action in this circumstance. | - If TAT is not running, then contention resolution is only considered successful if the received TB includes the TAC type2 MAC CE (otherwise, the UE might receive an UL grant in some scenarios like the one you point out) and might use the UL grant – which is not the correct behavior (since the UE has no TAT running). => Seems no change needed?? |
| E004 | ### 5.1.4a MsgB reception for 2-step CBRA 2> if the MsgB contains a MAC subPDU with fallbackRAR; and 2> if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see subclause 5.1.3a): 3> stop MsgB-ResponseWindow; 3> consider this Random Access Response reception successful; 3> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 4> process the received Timing Advance Command (see clause 5.2); 4> set the TEMPORARY_C-RNTI to the value received in the fallbackRAR; 4> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 4> process the received UL grant value and indicate it to the lower layers and proceed with Msg3 transmission; 4> Once Msg3 is transmitted, proceed to Contention Resolution step (see subclause 5.1.5). [Ericsson] The text after the highlighted text is very similar to the corresponding text in subclause 5.1.4, while some parts are missing (SI request, SRS-only cell). It makes it difficult to maintain in the future. | [Ericsson] Suggest jumping to the part of subclause 5.1.4 executed if Random Access Response reception is considered successful. The highlighted condition below must be addressed somehow as there is no need to obtain a new Msg3 in the 2-step fallback case. 2> if the Random Access Response reception is considered successful: 3> if the Random Access Response includes a MAC subPDU with RAPID only: 4> consider this Random Access procedure successfully completed; 4> indicate the reception of an acknowledgement for SI request to upper layers. 3> else: 4> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 5> process the received Timing Advance Command (see clause 5.2); 5> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); 5> if the Serving Cell for the Random Access procedure is SRS-only SCell: 6> ignore the received UL grant. 5> else: 6> process the received UL grant value and indicate it to the lower layers. 4> if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s): 5> consider the Random Access procedure successfully completed. 4> else: 5> set the TEMPORARY_C-RNTI to the value received in the Random Access Response; 5> if this is the first successfully received Random Access Response within this Random Access procedure: 6> if the transmission is not being made for the CCCH logical channel: 7> indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission. 6> obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer. | - Not sure I fully understood the comment, however, in general I have some sympathy with the general proposal (especially that duplicating similar text makes it difficult to maintain). However, jumping to a specific part of another section will be confusing (assuming this is the proposal)… if the UE procedure can execute a certain subclause in its entirety, then we can jump to that subclause, but jumping to a specific part within a given subclause is not something that is clean I guess. =>keep the current text |
| E005 | ### 5.1.4a MsgB reception for 2-step CBRA 2> if the MsgB contains a MAC subPDU with success RAR; and [Ericsson] It is not clear what “success RAR” is. Doesn’t the UE need to compare the preamble in the “success RAR” to determine that the RAR is for it and not some other UE? | [Ericsson] Add an Editor’s note to state that the UE has to compare the preamble of the “success RAR” or something. We suggest first checking the preamble and on next level checking the CCCH SDU. We would like to in general have a structure not using a “success RAR” but rather the explicit content for a successful response. See E006. | - successRAR will be defined (based on the ongoing email discussion 107#68) - Note that there is no RAPID in case of successful response => keep as it is |
| E006 | ### 5.1.4a MsgB reception for 2-step CBRA 2> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: [Ericsson] If the Contention resolution fails, nothing happens. Currently, Contention resolution only fails if the contention resolution timer expires, which is not how 4-step works. | [Ericsson] This is related to E005. Suggest updating to 2> if the MsgB contains a MAC subPDU with success RAR; and [Note our comment E005] 3> if the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU transmitted in MsgA: […] 3> else: 4> consider Contention Resolution not successful; | - Seems the assumption is that RAPID is included in case of successful response. But this proposal has been rejected. => keep as it is |
| E007 | # 6 Protocol Data Units, formats and parameters [Ericsson] We don’t think there is need to have the subclauses with letters (e.g. 6.1.5a) | [Ericsson] Suggest the following: 6.1.5a  6.1.6 6.2.2a  6.2.4 6.2.3a  6.2.5 In subclause 6.2 we would (with our proposal) then keep the subheader and payload for RAR together followed by the subheader and payload for our new additions. | - No strong view, for now the current headings are kept and we can change them once they are stable. => keep current headings for now (but can be changed based on further feedback) |
| E008 | Table 7.1-2: RNTI usage. [Ericsson] Updating “N/A” to “CCCH, DCCH” breaks 4-step. | [Ericsson] Changing to “CCCH, DCCH, N/A” seems weird. Would “CCCH, DCCH, None” be better? No strong view though. | - Actually, this could be MSGB-RNTI (may need to add a new row then) - Suggest we remove this change for now (can be added after the details of MSGB-RNTI are clear). => undo the change for now. |

# Review Issue list (Rev2)

| # | Brief description of the issue | Suggested resolution | Proposed way forward by rapporteur |
| --- | --- | --- | --- |
| Z000 | Xxx | Xxx | xxx |
| H001 | 5.1.1 - rsrp-ThresholdSSB-2stepRA: an RSRP threshold for selection of 2-step random access | We think it would be clearer to say that this is “an RSRP threshold for the selection of 2-step CBRA and 4-step CBRA”, because for 2-step CFRA and legacy CFRA, anyway you don't need this threshold for the RACH type selection and it is done by network indication. [OPPO]: similar view as Huawei, for the RACH type selection, given we will specify contention-free 2-step RACH, so here it’s better to clarify the RSRP is for selection of contention-based 2-step random access. | -I agree, I guess we can keep it as rsrp-ThresholdSSB-2stepCBRA. - This just means that for CFRA there is no selection based on RSRP. So, I will change it as below (assuming no one disagrees with the above statement). We may need an explicit agreement for this though? => change it back to rsrp-ThresholdSSB-2stepCBRA |
| H002 | 5.1.3a The MAC entity shall, for each MSGA: | Not clear why here “for each msgA” is added. I think for a single MAC entity, there can at most be one RACH procedure, either it is 2-step RACH or 4-step RACH | This was just following the corresponding wording in 4-step RACH (where it says “for each Random Access Preamble”). => keep the wording |
| H003 | 5.1.3a 1> compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted; | There is still a RA-RNTI here [OPPO]: align the terminology with MSGB-RNTI | => okay |
| H004 | 5.1.4a Once the MSGA is transmitted, regardless of the possible occurrence of a measurement gap, the MAC entity shall: 5.1.2 1> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB (the MAC entity shall select a PRACH occasion randomly with equal probability among the consecutive PRACH occasions allocated for 2-step random access according to subclause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB); | This has not been discussed before, although in 4-step RACH, UE can monitor both msg2/4 without the consideration of gap. It is up to the UE implementation whether to transmit preamble at gap in R15. We need to discuss what is the situation for 2-step RACH. | - This was suggested in a few tdocs although not explicitly agreed. I thought it is obvious. Let us keep and I will add an FFS that this should be agreed. => keep the text and add a note that an explicit agreement is needed |
| H005 | 5.1.4a 3> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 4> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 5> consider this Random Access Response reception successful; 5> consider the Random Access procedure successfully completed. | Here, after the reception of the MAC CE, the UE should process the TA, as defined in Section 5.2. So here, section 5.2 should be referred | => okay (add reference) |
| H006 | 5.1.4a 3> if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see subclause 5.1.3a): 4> consider this Random Access Response reception successful; ================= 3> else if the MSGB contains a successRAR MAC subPDU; and 3> if the CCCH SDU was included in the MSGA and the UE Contention Resolution Identity in the MAC subPDU matches the CCCH SDU: 4> if this Random Access procedure was initiated for SI request: 5> indicate the reception of an acknowledgement for SI request to upper layers. 4> else: 5> set the C-RNTI to the value received in the successRAR; 5> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 6> process the received Timing Advance Command (see subclause 5.2); 6> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); Editor’s Note: It is FFS whether the MAC should provide the above power control related parameters to physical layer per above 5> if the next MAC subPDU contains a SRB SDU; 6> forward the SRB SDU to higher layer; 4> consider this Random Access Response reception successful; 4> consider this Random Access procedure successfully completed. | Here it should be msgB reception successful? Or fallbackRAR reception successful. Prefer to fallbackRAR reception successful, this is also aligned with R15 [OPPO]: similar view as Huawei, fallbackRAR MAC subPDU is included in the MSGB, maybe we can use MSGB reception successful. Similarly, at the reception of successRAR, if the contention resolution is successful, we can consider the successRAR reception as successful | - I guess the main question is whether “MSGB reception” terminology can be used also for C-RNTI case. From the comments in the past, it seems that companies believe that even in case of C-NTI, the response from gNB addressed to the C-RNTI of the UE is treated as MSGB. If this is the common understanding, I think we can change the wording of “Random Access Response reception successful” to “MSGB reception successful”. However, looking at the comments from Oppo below (see O003) => Keep the current text and discuss further (add an Editor’s Note) |
| H007 | 5.1.4a 5> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 6> process the received Timing Advance Command (see subclause 5.2); 6> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); | For 2-step RACH, preamble can only be transmitted on the SpCell. So, here it should be SpCell. | Okay, => replace “Serving Cell where the Random Access Preamble was transmitted” with SpCell. |
| H008 | #### 5.1.4a 1> if msgB -ResponseWindow expires, and the Random Access Response Reception has not been considered as successful based on descriptions above: 2> increment PREAMBLE_TRANSMISSION_COUNTER by 1; 2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> indicate a Random Access problem to upper layers; 3> consider the Random Access procedure unsuccessfully completed. | We are fine to adopt only one counter for 2-step rach and 4-step as there is only one RACH procedure at a MAC entity for a certain time. But, this has not been agreed/discussed [OPPO]: no strong view, but maybe we need an Editor’s note here to say whether to re-use legacy counters for 2-step RACH is FFS. | - Yes, this is FFS and I think this is covered by the existing Editor’s note below the section 5.1.4a. =>keep as is |
| H009 | #### 5.1.4a 3> select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF; 3> if msgATransMax is configured, and PREAMBLE_TRANSMISSION_COUNTER = msgATransMax + 1: 4> perform the Random Access Resource selection procedure as specified in subclause 5.1.2 after the backoff time. =================================================== 3> if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see subclause 5.1.3a): 4> consider this Random Access Response reception successful; 4> apply the following actions for the Serving Cell where the Random Access Preamble was transmitted: 5> process the received Timing Advance Command (see clause 5.2); 5> set the TEMPORARY_C-RNTI to the value received in the fallbackRAR; 5> indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP); | The state variable and RRC parameter here should be italic | => okay |
| H010 | 5.2 1> when a Timing Advance Command Type 2 is received in response to a MSGA transmission including C-RNTI MAC CE as specified in clause 5.1.4a: 2> apply the Timing Advance Command for PTAG; 2> start or restart the timeAlignmentTimer associated with PTAG. | The indentation here is not right | => fixed! |
| H011 | 5.2 1> when a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell: 2> if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble: 3> apply the Timing Advance Command for this TAG; 3> start or restart the timeAlignmentTimer associated with this TAG. 2> else if the timeAlignmentTimer associated with this TAG is not running: 3> apply the Timing Advance Command for this TAG; 3> start the timeAlignmentTimer associated with this TAG; 3> when the Contention Resolution is considered not successful as described in clause 5.1.5; or 3> when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE: 4> stop timeAlignmentTimer associated with this TAG. 2> else: 3> ignore the received Timing Advance Command. | We don’t think the UE should ignore the TA if the TA is obtained with 2-step RACH. This needs to be discussed. | -in 2-step RA, timing advance can either be in MSGB or included in the type 2 TAC MAC CE. So, both these cases are covered explicitly, so it unclear why the UE will ignore the TA. => no change needed |
| H012 | 5.4.1 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant or determined as specified in subclause 5.1.2a for the transmission of the MSGA payload: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI. 5.4.2 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or 2> if the uplink grant was received in a Random Access Response; or 2> if the uplink grant was determined as specified in subclause 5.1.2a for the transmission of the MSGA payload; or 2> if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant determined as specified in subclause 5.1.2a for the transmission of the MSGA payload was selected; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 4> if the uplink grant size does not match with size of the obtained MAC PDU; and 4> if the Random Access procedure was successfully completed upon receiving the uplink grant: 5> indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission; 5> obtain the MAC PDU to transmit from the Multiplexing and assembly entity. | We agree with the intention that the msgA payload should be considered as new transmission. But this has not been discussed | -Do we need an FFS for this then? => Add an FFS |
| H013 | 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell or with a transmission of MSGA payload: 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; 2> if the configuredGrantTimer for the corresponding HARQ process is not running: 3> consider the NDI bit for the corresponding HARQ process to have been toggled; 3> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. | We wonder why the network would configure overlapping configured grant and msgA payload. The UE should only take this action when there is 2-step RACH ongoing, the current text is not crystal clear on this We agree with the intention. But this has not been discussed. [OPPO]: share similar view on this. | - I guess, the logic is similar to the MSG3 grant… i.e. there is no specific reason why the network will provide a msg3 grant that overlaps with a configured grant, yet, we covered this case explicitly in the MAC spec for whatever reason. So, it will be good to be consistent in this case. => keep as is |
| H014 | 5.4.2.1 Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response or for UL transmission for MSGA payload, HARQ process identifier 0 is used. | This also needs to be discussed first on what should be the harq process identity for the magA payload | -Again, we were following the MSG3 baseline. But, we can haven an FFS here too. => Add an FFS |
| H015 | To generate a transmission for a TB, the HARQ process shall: 1> if the MAC PDU was obtained from the Msg3 buffer; or 1> if there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer: 2> instruct the physical layer to generate a transmission according to the stored uplink grant. | Similar to the above issue on measurement gap. This has not been discussed yet on the priority between measurement gap and PUSCH payload. Also, whether to transmit PRACH resource on measurement gap will influence this since RPACH and PUSCH are associated here. We can discuss about this in the next meeting | Added an editor’s note per above for measurement gap considerations. => covered by the editor’s note about measurement gap |
| H016 | 5.1.4 NOTE 3: If the MAC entity receives both a grant in a Random Access Response and an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or CS-RNTI. | This part also needs to be modified for 2-step RACH Agreements are needed and we can discuss in the next meeting | - Should we add an FFS? Perhaps we can discuss based on HW contribution then . => discuss based on contributions |
| H017 | #### 6.1.3.4a Timing Advance Command Type 2 MAC CE | Change the name to 12-bit TAC MAC CE | - Let us discuss the name online => discuss online and pick a name. |
| H018 | ## 7.1 RNTI values RNTI values are presented in Table 7.1-1. Table 7.1-1: RNTI values. Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFEF; RA-RNTI, Temporary C-RNTI, C-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, and SP-CSI-RNTI. FFF0–FFFD; Reserved. FFFE; P-RNTI. FFFF; SI-RNTI. | Add msgB-RNTI to the list of RNTIs | - Agreed, that this is needed. For now no change done pending the discusson on RNTI. => keep as is (pending the discussion on RNTI) |
| O001 | Section 5.1.3a 1> if this is the first MSGA transmission within this Random Access procedure: 2> if the transmission is not being made for the CCCH logical channel: 3> indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission. | [OPPO]: I guess 5.1.3a will anyway apply to the 2-step CFRA (if specify), we have not yet discussed whether UE needs to include C-RNTI in the msgA payload for 2-step CFRA case. Actually in last meeting during the 2-step CFRA offline discussion, some companies proposed not to include C-RNTI in the payload due to the dedicated preamble. So we prefer to have a note here for possible FFS in the future. | - Yes, entire CFRA related procedure is FFS and this running CR as of now has no CFRA related aspects included in it explicitly (apart from some forward looking generalizations that we made to remove explicit mention of CBRA at places). - Let us discuss CFRA separately (no need to add CFRA related FFS points in this yet). => keep as is |
| O002 | Section 5.1.3a 1> compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted; 1> instruct the physical layer to transmit the MSGA using the selected PRACH occasion and the associated PUSCH resource, using the corresponding MSGB-RNTI, PREAMBLE_INDEX, | [OPPO]: we’ve commented in the previous comments to change RA-RNTI to MSGB-RNTI. [OPPO]: it’s not clear for now for 2-step CFRA, whether the UE needs to monitor MSB-RNTI? It seems to me only C-RNTI monitoring is sufficient? Maybe we should add an FFS here saying that when msgA transmission is due to 2-step CFRA, whether UE needs to compute MSGB-RNTI is FFS. | => okay fixed => CFRA aspects are not yet included in this CR |
| O003 | ### 5.1.4a MSGB reception and contention resolution for 2-step random access | [OPPO]: sorry for not being comments this in the v0, just feel that the title may not sufficient to cover the contents in this section. Based on my understanding, this section also related to the C-RNTI addressed PDCCH reception for the C-RNTI MAC CE case. To me, C-RNTI addressed PDCCH reception is not MSGB reception. Maybe we don’t need to clarify, since contention resolution for 2-step random access is related to C-RNTI addressed PDCCH reception. | - It seems companies have different view on what constitutes MSGB then? In this case, this needs a bit more discussion I guess (see H006 too). => keep as it is for now but discusss later. |
| O004 | Section 5.1.4a: 1> start the msgB-ResponseWindow as configured in RACH-ConfigCommon-2stepRA at the first PDCCH occasion from the end of the MSGA transmission as specified in TS 38.213 [6]; | [OPPO]: we have not yet discussed the RRC signaling structure. Just not sure whether we will introduce a new IE, i.e., RACH-ConfigCommon-2stepRA, or is it possible to include the 2-step RACH related RRC parameters into the existing RACH-ConfigCommon without introducing the new RACH-ConfigCommon-2stepRA? | - okay, I agree eventually this needs to be aligned with RRC. - may be it is easier to just remove the “as configured in RACH-ConfigCommon-2stepRA” to avoid the details (since msgB-ResponseWindow should be unique anyway). => delete “as configured in RACH-ConfigCommon-2stepRA” |
| O005 | Section 5.1.4a: 5> process the received UL grant value and indicate it to the lower layers and proceed with Msg3 transmission; Editor’s NOTE: In this case there is no need to obtain the Msg3 content again since the Msg3 buffer has already been obtained during MSGA transmission part. So, just processing the received UL grant should automatically result in transmission of Msg3 | [OPPO]: Confused by the “and proceed with Msg3 transmission” and the Editor’s note “just processing the received UL grant should automatically result in transmission of Msg3”. I think the UL grant received will be processed in the section 5.4.2, i.e., the HARQ entity, by specifying something like “if the uplink grant was received in a fallbackRAR”. then the UE would need to obtain the MAC PDU again from the msg3 buffer (if msg3 buffer is agreed) to a normal HARQ buffer, e.g., HARQ buffer #0 like legacy? For the editor’s understanding, I guess “Msg3 buffer has already been obtained” is for the ul grant in MSGA, not for ul grant in fallbackRAR. | - I guess the point is that we are reusing the MSG3 buffer for both MSGA payload and MSG3 (in case of fallback). So, in case of fallbackRAR, the UL grant will basically result in the transmission of MSG3 payload. Is this still confusing? If so, may be we can discuss what to change… For now I am not sure what exactly is the proposed change. => keep as is and discuss |
| O006 | Section 5.1.4a: 5> if the next MAC subPDU contains a SRB SDU; 6> forward the SRB SDU to higher layer; | [OPPO]: a SRB SDU an SRB SDU; In Legacy, msg4 can include RRC message, but we didn’t mention the behavior “forward the SRB SDU to higher layer”, instead we use “consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;”. So wondering do we really need a behavior to handle the SRB? | - we can also say “and finish the disassembly and demultiplexing of the MAC PDU” instead. I can change it this way (I think someone else commented along similar lines). => change to “ and finish the disassembly and demultiplexing of the MAC PDU”to align with legacy text |
| C001 | Section 5.1.1, for the case of PDCCH ordered RACH | [CATT] Is it so that PDCCH order case does not use 2s rach? If so the rach type selection procedure in section 5.1.1 needs clarify on this part. Add clarification in the section that in this case rach type selection is not needed. | - for sure the PDCCH order with CFRA is not applicable. - it is open whether PDCCH order with CBRA is applicable (we can discuss this based on contributions). => Leave as is and discuss the CBRA PDCCH order case based on contributions |
| C002 | Section 5.4.2.1 > if there is a MAC PDU in the Msg3 buffer and the uplink grant determined as specified in subclause 5.1.2a for the transmission of the MSGA payload was selected; or: | [CATT] here the UL grant determination refers to section 5.1.2a, which does not mention UL grant. Say PUSCH resource or UL resource? | - Agree, this is still not crystal clear. I will try to fix this. => fix the wording across the sections |
| S001 | 5.1.1 Whether threshold (rsrp-ThresholdSSB-2stepRA) for RA selection is specific to carrier specific (SUL or NUL) or common is FFS. | A note can be added to clarify this. | - Yes, we can add a note. => add a note to say it is FFS whether the threshold is carrier specific (for NUL and SUL) |
| S002 | 5.1.1 Current text assumes that RA prioritisation is not applied to 2 step CBRA | A note can be added that this is FFS. | - yes, add a note => add note |
| S003 | 5.1.1 RA initiated by PDCCH order can be contention based. So if both 2 step and 4 step RA is configured in BWP, UE needs to select one. RA initiated by PDCCH order can be contention free. In this case selection is not needed and UE can perform legacy contention free RA. However current text assumes that RA selection is always performed. | The case when : the ra-PreambleIndex has been explicitly provided by PDCCH" needs to be excluded. | - PDCCH order for CBRA is FFS for now and we can discuss this based on contributions. - PDCCH order for CFRA is not supported (only HO is supported for CFRA). => discuss based on contributions whether PDCCH order for CBRA is needed. |
| S004 | 5.1.1, If handover command includes legacy CFRA resources, it seems UE may end up performing 2 step CBRA if 2 step CBRA is configured in BWP and threshold criteria is met. This is not the intended behaviour. | In this case there are two options: Option 1: UE perform legacy RA procedure: during each attempt UE select between legacy CFRA and 4 step CBRA Option 2: If 2 step RA selection criteria is met: - during each attempt UE select between legacy CFRA and 2 step CBRA. If 2 step RA selection criteria is not met: - select between legacy CFRA and 4 step CBRA For option 1: The case when legacy CFRA resources are provided by RRC needs to be excluded from RA type selection in section 5.1.1 For option 2: legacy CFRA resource selection needs to be added to section 5.1.2a | - Yes, this needs to be discussed (we can discuss all the scenarios together now that we also need to discuss the 2-step CFRA case). => discuss based on contributions. |
| S005 | 5.1.4 A 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in MSGA: 3> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Random Access Response reception successful; 4> consider the Random Access procedure successfully completed. 3> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 4> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 5> consider this Random Access Response reception successful; 5> consider the Random Access procedure successfully completed. The highlighted (in yellow) condition is checked by UE even when timeAlignmentTimer is running. This is unnecessary. This condition should be checked only if timeAlignmentTimer is not running. Note that condition highlighted in green is combination of three conditions. Even when timeAlignmentTimer is running, if any of two conditions are false UE will execute statement highlighted in yellow. | Text can be revised as follows: 1> if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers: 2> if the C-RNTI MAC CE was included in MSGA: 3> if the timeAlignmentTimer associated with the PTAG is running: 4> if the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 5> consider this Random Access Response reception successful; 5> consider the Random Access procedure successfully completed. 3> else: 4> if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 5> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 6> consider this Random Access Response reception successful; 6> consider the Random Access procedure successfully completed. | => okay |
| S006 | ### 5.4.1 UL Grant reception If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion: 1> if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or 1> if an uplink grant has been received in a Random Access Response: 2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant or determined as specified in subclause 5.1.2a for the transmission of the MSGA payload: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI. 2> if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant: 3> start or restart the configuredGrantTimer for the correponding HARQ process, if configured. 2> deliver the uplink grant and the associated HARQ information to the HARQ entity. The change highlighted in green seems incorrect. NDI toggling is not done for Msg3 transmission in UL grant in legacy RA procedure. So why it should be done for MsgA payload. | Remove change highlighted in green | - If we agree that the HARQ process ID 0 will be used for the MsgA payload transmission (as we specified in the CR), I guess the sentence is still needed. The concerned case is that the NW schedules the UL grant with harq process ID 0 after the RA procedure, and in this case the NDI for harq process ID 0 shall be considered as toggled? => keep as is |
| S007 | ### Text in 5.4.2.1 (highlighted below) seems to support rebuilding for 2 step RACH which we have not agreed For each uplink grant, the HARQ entity shall: 1> identify the HARQ process associated with this grant, and for each identified HARQ process: 2> if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or 2> if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or 2> if the uplink grant was received in a Random Access Response; or 2> if the uplink grant was determined as specified in subclause 5.1.2a for the transmission of the MSGA payload; or 2> if the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery; or 2> if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to clause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant determined as specified in subclause 5.1.2a for the transmission of the MSGA payload was selected; or: 3> if there is a MAC PDU in the Msg3 buffer and the uplink grant was received on PDCCH for the C-RNTI in ra-ResponseWindow and this PDCCH successfully completed the Random Access procedure initiated for beam failure recovery: 4> obtain the MAC PDU to transmit from the Msg3 buffer. 4> if the uplink grant size does not match with size of the obtained MAC PDU; and 4> if the Random Access procedure was successfully completed upon receiving the uplink grant: 5> indicate to the Multiplexing and assembly entity to include MAC subPDU(s) carrying MAC SDU from the obtained MAC PDU in the subsequent uplink transmission; 5> obtain the MAC PDU to transmit from the Multiplexing and assembly entity. 3> else: 4> obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any; | Either remove the text or add an editor note. | => add editor’s note |
| E009 | ## 7.1 RNTI values RNTI values are presented in Table 7.1-1. Table 7.1-1: RNTI values. Value (hexa-decimal); RNTI. 0000; N/A. 0001–FFEF; RA-RNTI, Temporary C-RNTI, C-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, and SP-CSI-RNTI. FFF0–FFFD; Reserved. FFFE; P-RNTI. FFFF; SI-RNTI. | Ericsson: Currently we have open if the msgB RNTI, is equivalent to RA-RNTI, if this is a new similarly determined RNTI or a new UE specific RNTI. Also related is the open issue is msgB HARQ and how this is feasible to support for a multiplexed case. A UE specific RNTI (e.g. based on contention resolution ID) that is monitored in addition to msgB-RNTI. Can be a complementary approach that also has some power saving benefit, since the UE will only look for the DCI addressed to it prior to decoding PDSCH. It is complementary in that the msgB here is only for one UE, and so it will cost more PDCCH overhead when an msgB-RNTI is addressed to multiple UEs, but on the other hand an RRC message can be sent to the UE in the msgB to minimize setup latency and the PDSCH overhead will be less than in the multiplexed. We propose to add an editors note w.r.t this | - I guess, we already have an Editor’s Note in section 5.1.4a that the actual RNTI and its format is FFS. We can repeat it here in this section too if it helps. => Repeat the Editor’s Note from section 5.1.4 here. |
| Q001 | Section 5.1.4a 5> if the next MAC subPDU contains a SRB SDU; 6> forward the SRB SDU to higher layer; | RAN2 hasn’t agreed on whether SRB SDU is a separate MAC subPDU or SRB message is included in the successRAR. Maybe an Editor’s note is better. | - My original intention was to update the CR based on the outcome of the email discussion on MSGB format. However, may be we won’t have time for this. So, I will add an editor’s note for now. - anyway this part has now been reworded per the other comments above. So, this should be okay now. => text revised |
| v001 | Section 5.1.4a 3> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Random Access Response reception successful; 4> consider the Random Access procedure successfully completed. 3> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 4> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 5> consider this Random Access Response reception successful; 5> consider the Random Access procedure successfully completed. …… 5> if the next MAC subPDU contains a SRB SDU; 6> forward the SRB SDU to higher layer; 4> consider this Random Access Response reception successful; 4> consider this Random Access procedure successfully completed. | We think the phrase “Random Access Response reception” should be revised as “Contention Resolution” as contention resolution is solved in this case. According to the agreement made in RAN2#106, If a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB including a TAC of 12 bit is successfully decoded, the UE should will stop the reception of msgB. In another words, the UE will stop the msgB-ResponseWindow. UE’s behavior on stopping the msgB-ResponseWindow needs to be explicitly captured in the running CR. In conclusion, it can be revised as follow, 3> if the timeAlignmentTimer associated with the PTAG is running and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission: 4> consider this Contention Resolution successful; 4> stop msgB-ResponseWindow; 4> consider the Random Access procedure successfully completed. 3> else if a downlink assignment has been received on the PDCCH for the C-RNTI and the received TB is successfully decoded: 4> if the MAC PDU contains the Timing Advance Command Type 2 MAC CE: 5> stop msgB-ResponseWindow; 5> consider this Contention Resolution successful; 5> consider the Random Access procedure successfully completed. …… 4> stop msgB-ResponseWindow; 4> consider this Contention Resolution successful; 4> consider this Random Access procedure successfully completed. | - The only thing that matters is that in this case we have to declare that RA procedure is complete. Whether contention resolution is complete or not in this case seems irrelevant. In case of fallback we have a separate section for contention resolution (which kicks-in upon transmission of MSG3). So, I guess, both options will work in this case. No strong preference from my side, but for now I have kept it as is. - For the stop condition for msgB-response window, in the previous version, it was present, however, companies commented that the agreement was that the UE “may” stop monitoring (not that the UE shall stop it). Hence, this explicit statement was removed. Like I said above, I tend to agree with you that it is better to have an explicit statement here about UE behavior but, it seems we need more discussion for this. So, let us discuss this online. => keep as is |
| v002 | Section 5.1.1 1> if the RSRP of the downlink pathloss reference is above rsrp-ThresholdSSB-2stepRA: 2> select 2-step random access procedure; 2> perform the random access resource selection procedure for 2-step random access (see clause 5.1.2a). | According to the agreement achieved in the RAN2#107, if the UE is configured with 2-step RA, the RSRP is above a configurable threshold then the UE shall use the 2-step RA procedure. Considering that RA resource for 2-step CBRA, 4-step RACH, and CFRA can be configured to a UE simultaneously, the UE should give priority to the CFRA even though the RSRP is above the threshold for 2-step RACH. The sentence “select 2-step random access procedure” should be revised as “shall select 2-step random access procedure”. Besides, the following sentence highlighted in yellow should be removed. | - Right at the beginning of the clause there is a “shall” “ the MAC entity shall:” … So, we don’t need to add another “shall” in front of the “select 2-step RA procedure” statement. - Then, I agree that some changes might be needed here once CFRA is added. But for now this CR doesn’t have any CFRA related aspects included yet. So, we can discuss this once we have some basic agreements about CFRA - I am not sure why we should delete the sentence highlighted… it is just saying that upon selecting 2-step RA, the UE should proceed to section 5.1.2a (which is the next step it should do)?? => keep as is |