3GPP TSG-RAN WG2 Meeting #107bis	R2-1913649

Chongqing, China, 14th - 18th October 2019

Agenda item:	6.2.2.2

Source:	Charter Communications

Title:	Remaining Issues on Consistent LBT Failure Detection in NRU

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Operation in unlicensed spectrum requires conformance to regulatory rules. Particularly, listen-before-talk (LBT) rule ensures that operating nodes do not transmit over each other, hence avoiding collision in a shared spectrum. While LBT is important for operation in unlicensed bands, it affects operation of some of the NR baseline procedures such that it calls for additional attention in case of repetitive LBT failures. In NR-Unlicensed WID [1], the implications that LBT brings has been captured in a few of the objectives.

In RAN2 #105bis meeting [2], the following agreements related to the impact of UL LBT success/fail were reached:

- Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection

In RAN2 #107 meeting [3], the following agreements related to consistent UL LBT failure were reached:

- L2 LBT failure mechanism take into account any LBT failure regardless UL transmission type.

- The UL LBT failure mechanism will have the same recovery mechanism for all failures regardless UL transmission type

Baseline Mechanism, further enhancements not precluded:

- A “threshold” for the maximum number of LBT failures which triggers the “consistent” LBT failure event will be used.

- Both a timer and a counter are introduced, the counter is reset when timer expires and incremented when UL LBT failure happens

- The timer is started/restarted when UL LBT failure occur.

- UL LBT failures are detected per BWP

- The UE will report the occurrence of consistent UL LBT failures on PSCell and SCells. The assumption is to reuse SCell failure reporting for BF

In this contribution, we express our views regarding remaining details for consistent LBT failure detection and the follow-up action(s) that the UE should perform.

# 2	Consistent LBT failure detection

Based on the agreements from RAN2#107 [3], consistent LBT failure (CLF) and the process for detection and handling of such event has the following elements:

- LBT failure during any UL transmission, whether is PHY- or MAC-initiated, is considered

- LBT failures are counted per BWP, for any LBT category and for any CAPC

- Upon each LBT failure occurrence the associated counter is incremented and the associated timer is started/restarted

- The associated counter is compared to a configured threshold and when reached, the UE goes through a recovery procedure

In the following, we discuss some more details about above procedure.

## 2.1 	Accurate detection of consistent LBT failure events

Any transmission by an NR-U device requires channel access based on LBT procedure, whose outcome may be LBT success or failure. LBT failure happens when another device, whether an NR-U or another RAT device, engages in transmission. The duration of transmission of the competing devices depend on the type of RAT, and it could be as short as a few NR OFDM symbols or as long as several NR slots. Thanks to the regulatory limitation on maximum channel occupancy time (MCOT), the busy channel duration may not be more than about 5-10 NR slots (in FR1).

To detect a consistent LBT failure event, each occurrence of LBT failure/success may be considered a sample of the channel status, where based on a set of such samples a UE attempts to decide whether a CLF event has occurred or not. Therefore, detection of a consistent LBT failure event depends on the following factors:

- How often the LBT failure/success occurrences are observed, in other words how often a UE attempts to perform channel access.

  - Less often sampling may cause a delayed detection of a CLF event, if the channel is truly congested.

  - More often sampling may cause detection of a CLF event during consecutive transmission by a few neighbour competing devices which may be a transitory event and not a true CLF event.

- How correlated the gathered LBT failure/success samples are, in other words whether a significant portion of samples are taken during a single or consecutive channel access by a competing device.

  - More correlated samples may cause a false early detection if most of the samples are taken during an interval where a few neighbouring devices transmit consecutively.

For a more accurate evaluation of the channel status, it is best if the LBT success/failure occurrences are less correlated and obtained as uniformly as possible during a sufficient interval, e.g. during several MCOTs.

Observation 1: For a more accurate evaluation of the channel status, it is best if the LBT success/failure occurrences are less correlated and obtained as uniformly as possible during a sufficient interval, e.g. during several MCOTs.

Given above criteria, the associated timer and counter for CLF detection procedure should be configured such that:

- The CLF timer is long enough that spans several MCOT durations.

  - The setting of the CLF timer may seem to be straightforward, as long as it is configured with a value of several MCOTs. However, depending on how congested the BWP/sub-band(s) are, the CLF timer can be adjusted. For instance, in a congested BWP/sub-band(s) a longer CLF timer may be more reasonable, to avoid the UE to declare a CLF event (and taking the subsequent actions). Note that a congested channel situation should be reflected in channel occupancy (CO) reporting upon which the gNB may decide to initiate a BWP switching for the UE.

- The CLF counter does not reach its threshold during the COT(s) of a few neighbouring devices.

  - The setting of the CLF counter is less straightforward since some of the UE channel access occurrences are managed by the gNB, some of them are initiated by the UE.

Observation 2: The threshold of the consistent LBT failure counter should be configured by the gNB based on the congestion level of the BWP/sub-band(s).

For a less correlated enumeration of LBT failure occurrences, the events that have occurred during a correlated set of transmissions should be counted as one event. While it may not be straightforward to define a correlated set of transmissions, it is fair to say that a single transmission by a device or a set of transmissions within a COT (as is defined LAA and NRU spec) or TXOP (as is defined in 802.11 spec) can be considered a correlated set of transmissions. Therefore, CLF detection becomes more accurate if all LBT failure occurrences during such interval are counted as one event. In order to achieve this, an auxiliary timer may be used to aid MAC to count the LBT failure occurrences properly. Therefore, we suggest to introduce a new timer, with value may be set to the maximum COT duration, where all LBT failure events before expiration of this timer is counted as one LBT failure event. This timer starts after an LBT failure event, and is not restarted during subsequent LBT failure events unless it has expired.

Observation 3: For a less correlated enumeration of LBT failure occurrences, the events that have occurred during a correlated set of transmissions should be counted as one event. Transmissions within a COT/TXOP can be considered a correlated set of transmissions.

Proposal 1: Introduce a new timer, whose value may be set to the maximum COT duration, where all LBT failure events before expiration of this timer is counted as one LBT failure event. This timer starts after an LBT failure event, and is not restarted during subsequent LBT failure events unless it has expired.

## 2.2 	Consistent LBT failure detection per BWP or sub-band(s)

In RAN2#107 meeting, it was agreed that the UL LBT failure events are detected per BWP [3].

Operation in the unlicensed spectrum defined for NR-U [1] is subject to channel access in units of a sub-band, i.e. non-overlapping 20MHz sub-band(s). This helps a better coexistence among competing devices/RATs when accessing the unlicensed spectrum.

If the active BWP consists of multiple sub-bands, it is quite possible that various sub-bands may experience various number of competing devices. For instance, if the BWP consists of two sub-bands, it may happen that there are more competing devices (e.g. Wi-Fi devices) in one sub-band that another.  This is more pronounced in NR-U (compared to LAA) due to wide-band operation that baseline NR allows. The disadvantage of performing CLF detection per BWP is that a CLF event may be declared due to high congestion in one sub-band while other sub-bands may be not that congested. In other words, performing CLF detection per BWP is accurate if the congestion level in all the sub-bands within the BWP is relatively uniform.

Another problem that CLF detection per BWP could bring is that various UL channels may not be configured per each sub-band within the BWP. For instance, SR or RACH resources may be configured in only one sub-band within a multi sub-band BWP. Hence, LBT failure event during SR or RACH transmission counts toward the total number of LBT failures for a CLF event detection, while per sub-band enumeration would help gNB for a more efficient configuration.

Moreover, pending ongoing discussion in RAN1 and RAN4, it may be allowed that transmission in a BWP that consists of multiple sub-bands occurs across sub-bands with successful LBT, and avoid transmission across sub-band(s) with failed LBT. If partial BWP transmission is allowed, per-BWP CLF detection brings further inaccuracy.

Observation 4:  Pending ongoing discussion in RAN1 and RAN4, if partial BWP transmission is allowed, per-BWP consistent LBT failure detection brings further inaccuracy.

To avoid above disadvantage, one solution is to revise RAN2#107 agreement and allow consistent LBT failure detection to be performed per sub-band.

Proposal 2: To avoid inaccurate consistent LBT failure procedure associated with per-BWP enumeration, suggest to revise previous agreement and allow consistent LBT failure detection to be performed per sub-band.

## 2.3 	Procedure after Consistent LBT failure detection

After a CLF event is detected, there are various ways for a UE to react. One solution may be for the UE to initiate a re-establishment procedure. While re-establishment may seem straightforward, it could cause a long delay which is not desirable, unless the current BWP is the only BWP that UE is configured.

If multiple BWPs configured, the UE has the choice of trying other BWPs before starting the re-establishment procedure. Therefore, we suggest that, after detection of a CLF event, the UE should first perform random access on other (inactive) BWPs. In the event of multiple configured BWPs, the BWPs that have no frequency-overlap with the current BWP should be prioritized. Alternatively, or after exhausting all the non-overlapping BWPs, the UE should initiate RA procedure on the initial BWP (if different than current BWP).

Proposal 3: After detection of a consistent LBT failure event, the UE should first perform random access on other configured BWPs, where the BWPs that have no frequency-overlap with the current BWP should be prioritized.

If RAN2 agrees to revise previous agreement and allow consistent LBT failure detection to be performed per sub-band, upon a CLF event detection for a sub-band, the UE may avoid BWP switching if RA resources are configured for remaining sub-bands of the BWP. The UE should report the CLF event for the sub-band to gNB.

Proposal 4: If RAN2 agrees to revise previous agreement and allow consistent LBT failure detection to be performed per sub-band, upon a consistent LBT failure event detection for a sub-band, the UE may avoid BWP switching.

After a CLF event detection and after a BWP switch the associated timer(s) and counter of consistent LBT failure detection should be reset.

Proposal 5: After a consistent LBT failure event detection and after a BWP switch the associated timer(s) and counter of consistent LBT failure detection should be reset.

## 2.1 	Consistent LBT failure detection during LBT Cat-2

While it is possible to maintain a counter/timer for each LBT category and even each CAPC of LBT Cat-4, we believe such level of granularity may add to the complexity unnecessarily. Having separate counter for each LBT category and CAPC offers additional information about LBT failure statistics, the details may not make a difference since statistically, persistent LBT failure for a higher priority LBT category/CAPC should also trigger persistent LBT failure for a lower priority LBT category/CAPC. However, an exception would be persistent LBT failures during LBT Category 2. Given the priority that LBT Category 2 has (due to a deterministic short back-off), a persistent LBT failure should be (compared to a lower threshold and) acted upon in timelier manner.

In UL transmission, there may not be occasions that a UE performs LBT Cat-2, hence above observation may not be that relevant currently to UL transmission, unless RAN1 assigns LBT Cat-2 for some specific transmissions. However, there are instances that a gNB performs LBT Cat-2 for transmission (e.g. SSB transmission). A consistent LBT failure in DL, particularly for LBT Cat-2, should be treated in a timely manner.

Proposal 6: A consistent LBT failure in DL, particularly for LBT Cat-2, should be treated in a timely manner. Associated procedure is FFS.

# 3	Conclusions

In this contribution, we highlighted our concerns regarding consequences that LBT could bring in several MAC procedures. We have proposed the following:

Observation 1: For a more accurate evaluation of the channel status, it is best if the LBT success/failure occurrences are less correlated and obtained as uniformly as possible during a sufficient interval, e.g. during several MCOTs.

Observation 2: The threshold of the consistent LBT failure counter should be configured by the gNB based on the congestion level of the BWP/sub-band(s).

Observation 3: For a less correlated enumeration of LBT failure occurrences, the events that have occurred during a correlated set of transmissions should be counted as one event. Transmissions within a COT/TXOP can be considered a correlated set of transmissions.

Proposal 1: Introduce a new timer, whose value may be set to the maximum COT duration, where all LBT failure events before expiration of this timer is counted as one LBT failure event. This timer starts after an LBT failure event, and is not restarted during subsequent LBT failure events unless it has expired.

Observation 4:  Pending ongoing discussion in RAN1 and RAN4, if partial BWP transmission is allowed, per-BWP consistent LBT failure detection brings further inaccuracy.

Proposal 2: To avoid inaccurate consistent LBT failure procedure associated with per-BWP enumeration, suggest to revise previous agreement and allow consistent LBT failure detection to be performed per sub-band.

Proposal 3: After detection of a consistent LBT failure event, the UE should first perform random access on other configured BWPs, where the BWPs that have no frequency-overlap with the current BWP should be prioritized.

Proposal 4: If RAN2 agrees to revise previous agreement and allow consistent LBT failure detection to be performed per sub-band, upon a consistent LBT failure event detection for a sub-band, the UE may avoid BWP switching.

Proposal 5: After a consistent LBT failure event detection and after a BWP switch the associated timer(s) and counter of consistent LBT failure detection should be reset.

Proposal 6: A consistent LBT failure in DL, particularly for LBT Cat-2, should be treated in a timely manner. Associated procedure is FFS.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

RAN2 #105b Chairman notes, Xi’an, China, RAN2#105b, April 2019

RAN2 #107 Chairman notes, Prague, Czech Republic, RAN2#107, August 2019