3GPP TSG-RAN WG2 Meeting #107						            R2-1913880

Chongqing, China,14th – 18th October 2019

Title:	[DRAFT] Reply LS on SCell BFR Operation

Response to:	R2-1912013/ R1-1909833

Release:	Release 16

Work Item:	NR_eMIMO-Core

Source:	Apple [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Fangli XU

E-mail Address:	fangli_xu@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN1 for providing the feedback to our questions in the previous LS and the additional information on SCell BFR.

RAN2 has discussed the RAN1 feedback and RAN2 achieved the following agreements on the design aspects of SCell BFR:

- From RAN2 perspective, BFR-SR resource may not be configured.

- At most one BFR-SR resource can be configured per CG.

- From RAN2 perspective, BFR-SR retransmission procedure should be considered, which is similar as D-SR retransmission procedure, i.e. relaying on the max transmission number and prohibit timer.

- If both BFR-SR and D-SR are triggered at the same time, UE should prioritize the BFR-SR transmission procedure.

- BFR MAC CE can be transmitted on any available uplink grant for new data transmission.

- When BFR MAC CE is triggered, UE will cancel the BFR-SR transmission if it the MAC CE is transmitted via any available uplink grant.

- From RAN2 perspective, new “ACK” indication via L1 DCI or L2 MAC CE to the BFR MAC CE transmission should be supported.

- If UE receives the new “ACK” indication, UE can regard the SCell BFR procedure successful completion.

- UE operation upon BFR MAC CE transmission failure should be further studied.

- SCell BFR MAC CE is the MAC CE with the highest priority in CONNECTED mode.

- SCell BFR MAC CE carries the BFRQ information for one cell.

- The SCell BFR MAC CE includes the three information: failed SCell index, candidate beam information, no identified beam information.

Q1: For the point 7, is it feasible for RAN1 to support the explicit “ACK” indication in L1 DCI?

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account as well as requests responses to the questions provided.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#108		18 – 22 November 2019	Reno

3GPP RAN2#109		24 – 28 February 2020   	Athens